Ideal Interval Method Based Model for Water Quality Evaluation

Juliang Jin 1，Shujuan Wang 1，Yiming Wei 2

( 1.College of Civil Engineering, Hefei University of Technology, Hefei, Anhui 230009, PRC, jinjl66@xinhuanet.com; 2. Institute of Policy and Management, Chinese Academy of Sciences, Beijing 100080, China )

Abstract：The key of water quality evaluation is to establish the nonlinear relation between water quality indexes and water quality grades. Nowadays only evaluation criterion of water quality, which is expressed by interval concept, reflects this relation information during practical evaluation process. For this reason, a new model for water quality evaluation is presented based on improved ideal interval method with using accelerating genetic algorithm (IIM-WQE). Research results show that IIM-WQE is directly driven by the example series produced from evaluation criterion of water quality, and water quality relative grades computed by using all membership degrees information are regarded as evaluation results of water quality, which can avoid distorting when using the principle of maximum membership degree and can heighten precision of water quality evaluation. IIM-WQE is visual, handy and universal, which can widely be applied to different system evaluations if only evaluation criterion or samples of evaluation indexes and evaluation result values are known. [Nature and Science 2004,2(1):24-28].

Key words：water environment systems engineering; evaluation of water quality; ideal interval method;  genetic algorithm; relative membership degree

1  Introduction

Water Quality Evaluation (WQE) is to comprehensi-

vely evaluate water quality grades according to evaluation criterion of water quality and water quality indexes. It is very important to heighten the scientific level of water planning management and prevention and control of water pollution, to promote sustainable development of regional society, economy, resource and environment (Chen, 1998; Jin, 2002). Due to the incompatibility of evaluation results among individual water quality index of real water, it will lack practice for judgment of evaluate water quality grades by using evaluation criterion table of water quality directly, for this reason, many evaluation methods have been presented now. Each method has its applicability (Chen, 1998; Jin, 2000; 2001a; 2002a; 2002b; Hu, 1998). All the essences of these methods are try to describe the complex nonlinear relation between evaluation indexes and water quality grades, while nowadays only evaluation criterion of water quality which is expressed by interval concept (Chen, 1998; Hu, 1998) reflects this relation information during practical evaluation process. For this reason, in this paper a scheme of ideal interval method applied to evaluation of water quality, named IIM-WQE for short, is presented based on Yang (2003) and accelerating genetic algorithm (AGA) (Jin, 2001b; 2002a). The evaluation results of IIM-WQE continuous are real number values, so its evaluation precise is high. In the end an application case of IIM-WQE was done.

2  Establishment of IIM-WQE

Suppose evaluation criterion of water quality is {([a*(i,j)，b*(i,j)]，i)|i=1~ni，j=1~nj}. Where a*(i,j) and b*(i,j) are lower limit and upper limit of variation interval, which is named ideal interval (Yang, 2003) of ith water quality grade jth evaluation index respectively; i is criterion grade value of ith water quality grade; ni and nj are the number of grades and evaluation indexes of water quality evaluation criterion respectively. IIM-WQE establishment procedure includes four steps as follows:

Step1: to create sample series of water quality evaluation criterion randomly and standardize them. Produce uniformly and randomly nu index sample values x*(k,j) in the variation interval of each index , corresponding criterion grade values are y(k)=i; in order to fully reflect the importance of border values of each index in evaluation criterion, select each border value of indexes once, and corresponding criterion grade value is the average of the two criterion grades values related to this border value. Thus sample series of water quality evaluation criterion {x*(k,j)，y(k)}|k=1~nk，j=1~nj} are obtained, where nk is the number of the samples. In order to eliminate dimensional effect, and to make IIM-WQE have universality, the indexes are treated as follows:

x(k,j)=x(k,j)xmax(j)  (k=1~nk，j=1,nj)    (1)

a(i,j)=a(i,j)xmax(j)   (i=1~ni，j=1,nj)    (2)

b(i,j)=b(i,j)xmax(j) (i=1~ni，j=1,nj)      (3)

where xmax(j) is the maximum of the jth evaluation index in water quality evaluation criterion, that is

xmax(j)=maxib(i,j).

Step 2: to compute the distance D(k,i)between criterion sample x(k,j) and ideal interval of criterion grade [a(i,j)，b(i,j)]:

D(k,i)=j=1njw(j)d(k,i,j)                 (4)

d(k,i,j)=a(i,j)−x(k,j),x(k,j)<a(i,j)0,x(k,j)∈[a(i,j),b(i,j)]x(k,j)−b(i,j),x(k,j)>b(i,j)     (5)

where w(j) is the weight of the jth index, it can be determined by consulting expert with subjective weight method based on the analytic hierarchy process, by objective weight method based on projection pursuit according to the sample series of water quality criterion (Jin, 2001a; 2002a), or by combining the subjective weight and objective weight; where k=1~nk, i=1~ni, j=1~nj.

Step 3: to compute relative membership degree values r(k,i) of the sample {x(k,j)} relative to criterion grade ideal interval:

r(k,i)=e−cD(k,i)i=1nie−cD(k,i)           (6)

where c is a model parameter, it is a constant more than 1 generally. The more c is, the more easily criterion sample x(k,j) tend to criterion grade which has small relative membership degree D(k,i). In order to avoid a distorting judgment by using maximum membership degree rule, to

utilize all the relative membership degree information, and to make the judgment more accord with apply the fact,

water quality relative grade (Chen, 2002) can be adopted as computed value of water quality criterion grade which corresponds to the criterion sample x(k,j):

h(k)=i=1nir(k,i)i               (7)

And then c can be optimized by solving equations (8)

minf(c)=k=1nk|h(k)−y(k)|        (8)

AGA is a universal global optimization method, which simulates best selected and bad abandoned principle and chromosome population information exchange mechanism in the creation evaluation process. It is more handy and effective to solve optimization problem like equations (8). The detail algorithm of AGA can be seen in Jin (2002a) or Jin (2001b).

Step 4: to comprehensively evaluate water quality grade. Suppose each water quality evaluation index of researched water sample is {z*(k,j)|k=1~nz，j=1~nj}, where nz is the number of water samples needed evaluation. Replace x*(k,j) with z*(k,j), put it into equations (1), and then water quality grade h(k) (k=1~nz) can be computed corresponding to evaluation example according to equations from (2) to (8).

3  Case Study

Now as is a case, rich nutrient comprehensive evaluation of lake water environment quality illustrates the process of applying IIM-WQE. Evaluation criterion of lake water environment quality (Hu, 1998) is seen in Table 1, where the left end value of ideal interval of the first water quality grade of TP, TOD, TN indexes, and the right end value of that of transparency index are determined by the authors.

Table 1  Evaluation criterion of lake water environment quality (Hu, 1998)

| evaluation index | water quality grade |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | even poor nutrition | poor nutrition | middle nutrition | rich nutrition | even rich nutrition |
|  | 1st grade | 2 nd grade | 3 rd grade | 4 th grade | 5 th grade |
| TP(μg/L) | [0, 1] | [1, 4] | [4, 23] | [23, 110] | [110, 660] |
| TOD(mg/L) | [0.00, 0.09] | [0.09, 0.36] | [0.36, 1.80] | [1.80, 7.10] | [7.10, 27.10] |
| Transparency(m) | [37, 74] | [12, 37] | [2.4, 12] | [ 0.55, 2.4] | [0.17, 0.55] |
| TN (mg/L) | [0.00, 0.02] | [0.02, 0.06] | [0.06, 0.31] | [0.31, 1.20] | [1.20, 4.60] |

According to Table 1 and step 1 of IIM-WQE modeling, evaluation criterion sample series composed of 54 sample dots from No.1 to No.54 shown in Table 2 can be gained. Transform the 54 indexes sample values {x*(k,j)|k=1~54，j=1~4} to {x(k,j)| k=1~54，j=1~4} according to equations (1), scale each interval in Table 1 by using equations (2) and equations (3), each index weight is 1/4, calculate evaluation criterion grade of water quality h(k) (k=1~54) by using equations from (4) to (7), put them to equations (8) with {y(k)|k=1~54}, solve equations (8) by using AGA, and obtain the optimum evaluation value of c which is 99.888, and the corresponding minimum of objective function f(c)=2.771. Error analysis between evaluation criterion grades of water quality and grades of IIM-WQE are shown in Table 3 when c=99.888. Table 2 and Table 3 show that the nonlinear relation between water quality indexes and water quality grades described by IIM-WQE is satisfied. So IIM-WQE can be used to evaluate rich nutrition grades of the five lake water quality, its result is shown in Table 4, where the evaluation results of other methods are also listed. Table 5 shows the distances and membership degree values between lake water quality samples and ideal intervals of criterion grades.

Table 2  Comparison results between evaluation criterion samples of water quality and values of IIM-WQE

No.         water quality index            water quality grade  No.          water quality index          water quality grade

TP      TOD  transparency    TN    criterion   IIM-WQE          TP       TOD  transparency  TN  criterion  IIM-WQE

(μg/L)   (mg/L)  (m)         (mg/L)                            (μg/L)    (mg/L)      (m)  (mg/L)

1   0.658   0.059  50.857      0.001  1.000    1.008   28    12.631   0.479    4.738   0.257  3.000  3.045

2   0.559   0.051  42.650      0.015  1.000    1.120   29     7.879   1.129    4.840   0.249  3.000  3.074

3   0.106   0.034  67.902      0.009  1.000    1.000   30    14.074   1.405    3.723   0.140  3.000  3.102

4   0.978   0.061  42.207      0.010  1.000    1.137   31    21.122   1.442    3.487   0.142  3.000  3.151

5   0.646   0.051  47.659      0.015  1.000    1.025   32     9.431   0.655   11.033   0.091  3.000  2.729

6   0.078   0.056  73.851      0.010  1.000    1.000   33    23.000   1.800    2.400   0.310  3.500  3.499

7   0.742   0.086  54.876      0.013  1.000    1.002   34    36.301   2.753    1.872   0.802  4.000  3.986

8   0.607   0.014  54.330      0.011  1.000    1.003   35    63.988   3.232    1.064   1.159  4.000  4.003

9   0.142   0.029  43.305      0.000  1.000    1.089   36    84.708   4.817    2.045   0.465  4.000  3.998

10   0.316   0.003  48.395      0.008  1.000    1.018   37    95.742   5.358    0.594   0.805  4.000  4.013

11   1.000   0.090  37.000      0.020  1.500    1.500   38    50.872   5.349    0.816   1.151  4.000  4.015

12   2.790   0.195  14.496      0.040  2.000    2.241   39    34.379   2.639    2.191   0.917  4.000  3.990

13   3.862   0.356  18.619      0.049  2.000    2.090   40    87.044   3.024    1.423   0.856  4.000  4.000

14   3.569   0.338  12.537      0.055  2.000    2.440   41    32.149   4.351    1.692   0.382  4.000  3.965

15   1.816   0.300  14.218      0.034  2.000    2.263   42    84.474   1.817    1.552   1.138  4.000  4.001

16   3.011   0.268  29.132      0.057  2.000    1.959   43    67.301   3.868    2.334   0.758  4.000  3.998

17   1.060   0.132  25.247      0.059  2.000    1.994   44   110.000   7.100    0.550   1.200  4.500  4.500

18   3.368   0.187  33.033      0.034  2.000    1.832   45   649.959  21.826    0.210   3.396  5.000  5.000

19   3.842   0.324  20.456      0.036  2.000    2.044   46   161.122  19.802    0.428   3.580  5.000  5.000

20   3.889   0.299  17.987      0.058  2.000    2.109   47   281.367  10.708    0.515   2.934  5.000  5.000

21   3.414   0.270  18.595      0.033  2.000    2.076   48   443.507  26.418    0.364   2.626  5.000  5.000

22   4.000   0.360  12.000      0.060  2.500    2.501   49   355.354   8.808    0.438   2.126  5.000  5.000

23  21.883   1.292   8.447      0.165  3.000    2.999   50   549.721   9.753    0.250   2.602  5.000  5.000

24  10.488   0.797   2.943      0.196  3.000    3.088   51   184.419  11.128    0.263   3.523  5.000  5.000

25  13.009   1.414   9.719      0.197  3.000    2.966   52   342.262  24.807    0.370   2.658  5.000  5.000

26   9.853   1.116   6.878      0.255  3.000    3.026   53   181.062  12.666    0.274   2.875  5.000  5.000

27   9.925   1.479   6.117      0.079  3.000    3.002   54   233.632  26.579    0.250   1.943  5.000  5.000

Table 3  Error analysis between evaluation criterion grades of water quality and grades of different models

Evaluation             percent of absolute error falling the following range (%)   mean absolute error  mean relative error

method               [0, 0.1]   [0, 0.2]  [0, 0.3]  [0, 0.4]  [0, 0.5]  [0, 0.6]       (grade)             (%)

pp method[5]            48.00    68.00    92.00   92.00   96.00   100.00        0.15               7.74

interpolation method[6]    89.66    93.10    93.10   94.83   98.28   100.00        0.05               1.58

IIM-WQE             81.48    92.59    98.15   98.15  100.00   100.00        0.05               2.65

Table 4  Computed grades of five lakes water quality by using different model

water quality index                computed water quality grades of different models

TP        TOD     Transparency     TN        NN[ 3]  Logistic curve[4]   interpolation[6]  IIM-WQE

Lake                (μg/L)   (mg/L)      (m)      (mg/L)

Hangzhou Xi Lake    130      10.30      0.35       2.76         5       4.92         4.860        5.000

Wuhan Dong Lake     105      10.70      0.40       2.00         4       4.80         4.546        4.999

Qinghai Lake         20       1.40      4.50       0.22         3       3.09         3.028        3.150

Chao Lake            30       6.26      0.25       1.67         4       4.59         4.058        4.241

Dian Lake            20      10.13      0.50       0.23         4       3.67         3.638        3.998

Table 5  Distance and membership degree values between lake water quality samples and ideal intervals

distance between lake water quality samples          membership degree values between lake water qualities

lake        and ideal intervals of criterion grades                 samples and ideal intervals of criterion grades

1st grade  2nd grade  3 rd grade  4th grade  5th grade  1st grade  2nd grade  3rd grade  4th grade  5th grade

Hangzhou Xi Lake   0.416   0.326     0.259      0.123      0.000        0.000     0.000      0.000    0.000     1.000

Wuhan Dong Lake   0.369   0.278     0.212      0.077      0.002        0.000     0.000      0.000    0.001     0.999

Qinghai Lake       0.140   0.050     0.000      0.017      0.153        0.000     0.006      0.838    0.156     0.000

Chao Lake         0.282   0.191     0.125      0.027      0.038        0.000     0.000      0.000    0.759     0.241

Dian Lake         0.235   0.144      0.083     0.034       0.087       0.000     0.000      0.007    0.988      0.005

Table 3, Table 4 and Table 5 show that: 1) The evaluation results of IIM-WQE are the same as other methods basically. 2) According to the evaluation criterion of environment quality in Table 1, the results of IIM-WQE are reasonable. 3) The results of IIM-WQE that the water quality grades of these five lake from big to small are Hangzhou Xi Lake, Wuhan Dong Lake, Chao Lake, Dian Lake, and Qinghai Lake in turn, where the first two lakes can be judged as 5th grade (even rich nutrition), the later lake can be judged as 3rd grade (middle nutrition), the rest can be judged as 4th grade (rich nutrition).

4  Conclusion

In order to describe the nonlinear relation between water quality indexes and water quality grades, a new method for water quality evaluation based on improved ideal interval method and accelerating genetic algorithm (IIM-WQE) is presented. IIM-WQE belongs to non-function model evaluation methods, its evaluation process is directly driven by sample series produced from water quality evaluation criterion, which overcomes the difficulty how evaluation function relation between evaluation indexes and evaluation grades is reasonably structured. The results of IIM-WQE are of real values, high precision. IIM-WQE is visual, handy and universal, which can widely be applied to system evaluation of evaluation criterion or evaluation indexes and comprehensive evaluation values are known.

Acknowledge

Foundation Item: The Excellent Young Teachers Program in Higher Education Institute of MOE, P.R.C.

[Department of Education and Person (2002)350], Anhui Provincial Science and Technology Fund of Excellence Youth, Anhui Provincial Natural Science Foundation (No. 01045102), Chinese National Natural Science Fund Project (No. 40271024).

References

[1]  Chen S. Fuzzy optimum recognition theory and  application  for

complex water resource system[M]. Changchun: Jilin University Press, 2002.

[2]  Chen S. Theory and Application for Engineering Fuzzy Sets[M]. Beijing: Chinese National Defence Industry Press, 1998.

[3]  Hu M, Guo D. Fuzzy Neural Network Method for Evaluation of

Lake Water Quality Rich Nutrition[J]. Chinese Environment Science Research, 1998,(4):40-2.

[4]  Jin J, Din J, Water Resource Systems Engineering[M]. Chendu:

Science and Technology Press of Sichuan, 2002a.

[5]   Jin J, Wei Y, Din J. Projection Pursuit Model for Comprehension Evaluation of Water Quality[J]. Chinese Journal of Environment

Science, 2001a,21(4):431-434.

[6]  Jin J, Wei Y, Fu Q, et al. An Interpolation Model for Comprehensive Evaluation of Water Quality [J]. Chinese Journal of Water Hydraulic, 2002b,(12): 91-4, 100.

[7]  Jin J, Yang X, Din J. Improvement of Standard Genetic Algorithm-Accelerating Genetic Algorithm[J]. Systems Engineering—Theory & Practice, 2001b , 21(4):8-13.

[8]  Jin J, Yang X, Jin B, et al. New Model for Comprehensive Evaluation of Water Environment Quality[J]. Chinese Environment Monitor ,2000, 16(4):42-7.

[9]  Yang X, Yang Z, Li J, et al. Comprehensive Evaluation of Atmosphere, Environment Quality for Multi-objective Decision-IIM[J]. Chinese Environmental Engineering, 2003, 21(3):70-2.