Control of the Behavior Dynamics for

Motion Planning of Mobile Robots

Xingjian Jing1,2, Yuechao Wang1, Dalong Tan1

1. Robotics Laboratory, Shenyang Institute of Automation, Chinese Academy of Sciences

Nanta Street 114, Shenyang, Liaoning 110016, China, E-mail: xjjing@sia.cn

2. Graduate School of Chinese Academy of Sciences, Beijing, China

Abstract: A new approach to the motion planning problems of mobile robots in uncertain dynamic environments based on the behavior dynamics is proposed. The fundamental behavior of a mobile robot in motion planning, which is regarded as a dynamic process of the interaction between the robot and its local environment, is modeled and controlled for the motion planning purpose. The behavior dynamics is the law in a dynamic process that the involved objects must obey, and it is controlled by the robot’s dynamics. Based on the control of the behavior dynamics, the dynamic motion-planning problem of the mobile robots is transformed into a control problem of the integrated planning-and-control system. No restrictions are assumed on the shape and trajectories of obstacles. Collision avoidance between multiple mobile robots can also be realized. The stability of the integrated planning-and-control system can be easily guaranteed. Simulations illustrate our results. [Nature and Science. 2004;2(4):57-64].

Key Words: Behavior dynamics; motion planning; mobile robots; uncertain dynamic environment

1.  Introduction

In all the applications of mobile robots, a good motion planning method is very important to accomplish tasks efficiently and stably [1-2]. However, whenever it comes to dealing with an environment that is totally or partially unknown or even dynamically changing, such a dynamic motion-planning problem is intractable [3], since the mobile robot is required to decide its motion behavior on line using only sensors’ limited information. Various methods have been proposed for this purpose, such as (1) Configuration-time space based methods [4-7,30], (2) Planning in space and time independently [8-11], (3) Artificial potential fields based methods [12-18], (4) Behavior based methods [19-20], (5) Intelligent computing based methods [20-24]. Some new methods are also proposed recently, e.g., cooperative collision avoidance and navigation [20,25,26], velocity obstacles method [27], collision cone approach [28], etc. Other sensor-based navigation frameworks can also refer to [13,29]. Significant improvements on the motion-planning problems of a robot have been obtained in the past decades. However, many of the existing methods are only kinematic planning, or rely on some knowledge of the global environment, or require some constraints on the shape or velocity of obstacles, etc. Moreover, behavior decision-making usually is to simply sum up all the impacts from different obstacles with different weights in conventional behavior-based methods [19]. This may lead to counteraction of different reactive behaviors, and consequently result in unexpected motion behavior or performance of the mobile robot.

In motion planning problems of mobile robots, motion behaviors of the mobile robot can be classified into two fundamental behaviors: Collision-Avoidance behavior, and Going-to-the-Goal behavior (in short, CA-behavior and GG-behavior, respectively). How these behaviors are realized and performed in motion planning, how the goodness of the motion planning is. In most of the existing motion-planning methods, motion behaviors are generally regarded as static and discrete behavioral reactions to the environments, instead of dynamic processes. These methods cannot effectively map the local changing environment into dynamic behaviors of the robot. The reactive behaviors, together with the robot dynamics, cannot be integrated into one uniform planning-and-control system to be designed to achieve some performances of the whole system. For these reasons, the conventional behavior-based methods may offer poor performance for the robotic system of large mass or high velocity.

In this paper, the fundamental behaviors of a mobile robot is modeled as dynamic processes of the interactions between the robot and its local environments, then the desired dynamic motion behaviors can be obtained based on the control of these dynamic processes, which are called behavior dynamics. The control input of the behavior dynamics is right the desired acceleration of the mobile robot. The behavior dynamics and the robot’s dynamics are integrated into one uniform planning-and-control system, and thus a new method using only sensors’ information for the motion planning of mobile robots in uncertain dynamic environments is proposed. The behavior dynamics is sensor-based, and no knowledge of the shape and velocity of the obstacles are needed. The dynamic constraints of the mobile robot are considered. Collision avoidance between multiple mobile robots can also be realized. The stability of the whole planning-and-control system can be guaranteed. To the best of our knowledge, this behavior dynamics-based approach is novel. Simulations are given to illustrate our method.

Notations: A black bold symbol denotes a vector, e.g. a vector V, and then V denotes its norm. x=arg(⋅) means to choose an x satisfying (⋅). For any vector A, let e(A)=A/A. A point  p in a plane is written as p=(px, py), or p=[px,py]T, where px and py are the corresponding components on each coordinate. Let eX=[1,0]T.

2.  Modeling of the fundamental behaviors in motion planning

In this section, behavior dynamics is defined and modeled. Without loss of generality, we regard a mobile robot as a point mass, and restrict our study to the 2-D planar case. Similar method can be extended to more than 2-D cases.

Figure 1. The robot meets an obstacle

At time t, without speciality, refer to Figure 1. The mobile robot R is at the original point of the global coordinates {X,Y} with velocity Vr and meets an obstacle at point O with velocity VO,. Let point G=(xd,yd) be the goal of the robot, and V=Vr-VO, be the relative velocity of the mobile robot with respect to the obstacle. The angle from vector Vr and VO to vector V are denoted by α and β, which are also written as α=∠(V,Vr) and β=∠(V,VO), respectively. This paper assumes that, all the angles in this paper belongs to [−π,π], and an angle is positive if it is formed by rotating a vector anti-clockwise, else it is negative. In Figure 1, γ,θr,θO are the angles from X-coordinate to the vectors V, Vr, and VO, respectively. Obviously, γ=θr+α=θO+β. From point R, there are two lines that are tangent to the boundary of the obstacle O and denoted by L+ and L-. The angles from vector V to L+ and L- are denoted by γ+andγ−, respectively. For any vector V 0, it cannot only be written as V=[Vx,Vy]Tusing its corresponding components on each coordinate, also be denoted by [V,γ]T, where V=V, γ=∠(V,eX). For the two forms, we have

Vx=Vcosγ,and Vy=Vsinγ.                       (1)

Using the variables given above, the collision condition can be written as

γ+⋅γ−≤0                                                           (2)

Once inequality (2) holds, the mobile robot is heading a collision. That is, if inequality (2) holds, the relative velocity vector V should be controlled to rotate a desired angle γ+orγ−in order to avoid a collision. This is the dynamic process of a collision-avoidance behavior. The model of this dynamic process is developed in the following.

From (1), it is easy to obtain the following relationship:

V=Vx or

V                                   (3)

where J(V,γ)=cosγ−VsinγsinγVcosγ.

Recalling that V=Vr-VO, and from (3) we have

V   (4)

Utilizing (3a), Equation (4) yields

V                    (5)

Recalling that γ=θr+α=θO+β, and the following relationships

sin(ω+σ)=sinω⋅cosσ+cosω⋅sinσ ∀ω,σwith some calculations, Equation (5) further yields

V           (6)

Equation (6) is the Behavior Dynamics of a Collision-Avoidance behavior, in short as CA-dynamics. It is the law that the involved robot and obstacle must obey in a collision- avoidance process. Equation (6) is developed with assumption that V 0. It is noted that V =0 can not hold for all the time in order to avoid an obstacle, thus this paper only considers the case V 0. In each planning period, VO can be assumed to be constant. Hence, we have VO=0and θO=0. Otherwise, the last term of equation (6) can be estimated and thus its effect can be cancelled using some methods. Then (6) can be rewritten as follows:

V     (7a)

As for the CA-dynamics in (7a), the control input is [Vr,θr]T, it is easy to be transformed into [Vrx,Vry]T, which is the acceleration of the mobile robot in the global coordinates. It should be noted that α, V and γcan all be detected or calculated in the local coordinates on the mobile robot, therefore, there are only sensor information needed in Equation (7a). This facilitates the motion planning for mobile robots in dynamic uncertain environments. Once Inequality (1) holds, the control problem of (7a) is: Given the desired [Vd,γd]Tfor a collision-avoidance behavior, the task is to find a control law

Vr=uV(Vd,V,γd,γ,α)                       (7b)

such that system (7) is stable, i.e.,

limt→T1V(t)=Vd, limt→T1γ(t)=γd (∃T10).

Though [Vd,γd]T is time varying, it can be regarded as constant in each planning period. For a collision-avoidance behavior, such that γ+⋅γ−<0 does not hold, should only γd be set to be γ+ or γ− for the above control problem whatever V is. That is, γ is the key variable to be controlled in system (7) to avoid an obstacle, and Vd can be set to be any positive value. For this reason, as for the CA-dynamics we need only consider a simplified system as follows:

γ=−1VsinαVrVcosα⋅Vr                   (8)

That is, only the steering angle of V is considered.

If the goal of the mobile robot is regarded as a special obstacle, then the CA-dynamics can also be regarded as the dynamics of GG-behavior. In this case, V=Vr, α=0. Then Equation (7a) is now written as

V=Vr                                                      (9)

Obviously, (9) is included in system (7a), and [Vd,γd]T in (7b) for GG-behavior is to be defined.

Another behavior frequently generated in a collision-avoidance process or navigation of the mobile robot is the Wall Following (WF) behavior, which is to follow the boundary of an obstacle till avoiding it. The dynamics of WF-behavior is the same to (9), and the desired [Vd,γd]T in (7b) for WF-behavior is also to be defined. If there exists collision risk, then CA-behavior is carried out. Once there is no collision risk but the obstacle is still near enough to the mobile and between the mobile robot and its goal, then WF-behavior is generated in this case.

From above all, it is noted that all the fundamental behaviors in motion planning problem can be described by the same dynamics model (7) with different desired [Vd,γd]T.

3. Control of the behavior dynamics and the robot dynamics

In order to illustrate our idea, only a simple case for the robot dynamics is considered in this paper. For other cases, similar results can be developed. Hence, for simplicity, the robot dynamics considered in this paper is described by

M(q)q+f(q,q)q+g(q)=τ               (10)

Using the computed torque control

τ=M(q)u+f(q, {q)q+g(q)                    (11)

then (10) can be rewritten as

q=u                                                           (12)

It is now needed to plan the desired states [qdT,qdT,qdT]T for (12). Utilizing (7b), we have

q=Vx       (13)

where, Vr=q,θr=∠(q,eX). (13) is the desired qd for (12). This implies that it needs only to control the acceleration of the mobile robot for a desired behavior. Now, the integrated planning-and-control system based on behavior dynamics can be described as:

ν=B(V,Vr,α,θr)⋅q                            (14a)(14b)

which further yields

ν=B(V,Vr,α,θr)⋅u                      (15a)

where, ν=V,

B(V,Vr,α,θr)=cosαVrsinα−1VsinαVrVcosα⋅J(Vr,θr)−1     From (14), control of the behavior dynamics is in fact a control problem in the acceleration space of the mobile robot. The control problem for (15a) is: Given a desired νd=[Vd,γd]T for a desired behavior, to find the control law u, such that limt→∞ν(t)=νd. For this problem, the following result is obvious.

Theorem 1. Given a desired νdfor (15a), choose any a simple linear feedback control law u as follows:

u=−K1eν                                               (15b)

Then (15a) is asymptotically stable, i.e., limt→∞ν(t)=νd. Where, eν=ν−νd,BK1>0.

It should be noted that (15b) is the desired qd for the mobile robot. To realize a desired behavior, is to control the robot’s dynamics to follow a desired acceleration. For this reason, we can let qd(t)=q(t0)+qd(s)ds, and

qd(t)=q(t0)+qd(s)ds.

Then for the robotic system, it is easy to design a tracking control law to guarantee the realization of the desired behavior.

Figure 2.  A structure of the whole system

From aforementioned discussions, stability of a desired behavior and the whole planning-and-control system is easy to be guaranteed. The motion-planning problem is now transformed into a control problem of the behavior dynamics. A structure of the whole system can be referred to Figure 2. By using behavior dynamics, the planning task is now to give the desired νd at each planning time. Various methods can be adopted to plan the desired νd, a simple method is provided in the following section.

4. Realization of the desired behaviors

In this section, we design the desired νd using the local knowledge of the environment in order to realize the desired behaviors for the mobile robot according to the control laws in Theorem 1.

As for the GG -behavior, let [Vd,γd]T in (7b) be:

Vd=min(0.7⋅Vmax,G−R)                  (16)

where, Vmax is a positive constant, G=(xd,yd)T and R are the goal and current position of the mobile robot, e(G-R) is the desired direction for the robot to move in the global coordinates, which is assumed to be known. And the desired [Vd,γd]T in (7b) for WF-behavior is given as

Vd=min(0.7⋅Vmax,G−R)                    (17)

where γ+,γ−corresponds to the obstacle to be followed.

Since the relative velocity is the robot’s velocity for the GG/WF-behavior, we have

qd=Vdcosγd, and qdis (17b), qd=[xd,yd]T.    (18)

where Vd=min(0.7⋅Vmax,G−R).

As for the CA-behavior, we only consider the control problem of the simplified CA-dynamics (8). If γ+⋅γ−≤0, we let

γd=argminγ∈γ+,γ−(γ)                                  (19)

The qd,qdand qd are still (18).

Considering the dynamic constraints of the mobile robot, that is, the maximum velocity and acceleration are assumed to be Vmax and amax, respectively. Let P(R, t) be the observable region of the mobile robot, in which an obstacle can be effectively detected by the sensors. For an obstacle Oi, let ∂Di be its boundary. If there exists a point P∈∂Di∩P(R,t),such that |∠(e(Vi),P−R)|=0 or π,then let Pci=arg|∠(e(Vi),P−R)|=P∈∂Di∩P(R,t)0 or π,

otherwise let Pci=∞.LetVi=Vr -VOi, and Vi=||Vi||. Let li=(Pci−R)⋅e(Vi) be the collision distance (C-distance) of the mobile robot and an obstacle Oi. If the mobile robot is heading to an obstacle Oi and the distance li is too near, only going in the contrary direction with the maximum acceleration can the robot guarantee the safety. It is easy to prove that the minimum distance for this case is l0=Vi22(amax+VOie(Vi)). If the obstacle is static or its velocity is changing slowly, i.e., VOi≈0, then l0=Vr22amax. Once the robot is approaching to this distance, the steering angle γof the relative velocity Vi should be modified to the direction opposite to the current direction. For this purpose, we have the following results.

For an obstacle Oi, we redesign the γd in (19) to be

γds=argminγ∈γ+,γ−(γ)     (20)

where, ∀x, pos(x)=max(0,x), sign(x)=1 x≥0, ks>0. Note that γd→±π whenever li→l0, and γd→±γds if li=∞.

Note that (20) provides the γd only for one obstacle case. Considering the multiple obstacles case, we need give the optimal γd for the robot to go according to all the steering angles of the relative velocity Vi  corresponding to different obstacles.

We have mentioned that there are two steering angles, i.e. γi+and γi−, corresponding to an obstacle Oi. And in order to avoid the obstacle, the relative velocity Vi should be adjusted such that γi−γiγi+not holds. Let Di=γ|γi−≤γ≤γi+, it is called dangerous region with respect to the obstacle Oi. Considering the idea used in (20), Di can be rewritten as

Di=γ|γi−+δ(γi−)≤γ≤γi++δ(γi−)      (21)

where, δ(γ)=π−|γ|1+ks⋅pos(li−l0)⋅sign(γ). Then for all the obstacles in the observable region P(R, t), the dangerous region is

D=iDi                                                      (22)

Then from (22), the decision-making space of γd for the mobile robot is

U=[−π,π]                                    (23)

The desired γd for the robot to avoid the obstacles should be chosen from U. However, a constraint for this decision-making corresponding to the dynamic constraints of the mobile robot should be satisfied. From (15b) and note that qd≤amax, we haveK1γd≤amax. Hence, we have γd≤K1−1amax.

With consideration of the GG-behavior, decision of γdcan now be described as an optimization problem as follows (Planning Problem for γ, in short PP-γ):

To find γ in U (23) such that

Min J(γ)

And satisfying |γ|≤K1−1amax.

Let

J(γ)=kaγ2+kb(γ−γdG)2+kcsat(K1−1amax)   (24)

where sat(γ,Γ)=0 |γ|≤Γ, and ka,kb,kc>0, ka + kb =1, kc should be enough large, γdG is the desired steering angle γd for GG-behavior. Under the evaluation function of (24), the decision-making of PP-γ can be formulated to be

γd=argminγ∈U(J(γ))                                     (25)

In (25), the CA-behavior, GG-behavior and the dynamic constraints of the mobile robot are all considered. Different ka and kb lead to a different tradeoff between the CA-behavior and GG-behavior. In order to try to guarantee the safety of the mobile robot and no local minima are encountered, we can let ka << kb. Note that the following equation is hold:

argminγ(J(γ))argminγkaγ2+kb(γ−γdG)2+kcsat(K1−1amax)argminγ(γ−kbγdG)2+kcsat(K1−1amax)Hence, (24) can also be substituted by

J(γ)=(γ−kbγdG)2+kcsat(K1−1amax)          (26)

5. Simulations

In this section, simulations are provided to illustrate our method. Any dynamic model of an omni-directional mobile robot can be used in the simulations. Parameters of the mobile robots used in simulations are: amax=0.5 m/s2, Vmax=0.5 m/s, ωmax=0.1 rad/s, the radius of the robot is r =0.3 m. The effective detecting radius of the sensors is 1.5 m. Velocity of the moving obstacles, if any, is 0.35m/s. T1 is chosen to be the planning period, and let T1=0.1s. Moreover, let ka=0.9, kb=0.1, kc=5 (in (24) and (26)), ks=100 (in (20)).

Specially, we assume that, only the distance between the mobile robot and a static obstacle is less than 1 m, then the CA-behavior with respect to this static obstacle is adopted. Additionally, in order to remove the oscillation on the trajectory and guarantee the safety when the mobile robot goes along the boundary of an obstacle, the following strategy is used:

(1) If |γ|≤δ, let γ=0, where δ is a small positive number.

Simulations are given under different situations in order to illustrate our method (Figure 3-4). In Figure 3, the environment is static with “SIA”-shape obstacles. The result shows that the mobile robots can coordinate to avoid collision with each other in this environment when they meet, and they can also navigate the U-shape obstacles without being trapped in local minima. Note that wall-following-like behavior is automatically generated in this case. Figure 4 shows a more complex environment, in which there are not only static obstacles but also two moving obstacles. The robots can effectively avoid collision with the unknown moving obstacles.

From the simulation, it can be seen that, (1) The trajectories planned for the mobile robot is smooth, and there are no local minima encountered Shapes of obstacles can be arbitrary, and no knowledge is required about the boundary or velocity of obstacles. (2) All the variables needed in the decision-making of PP-γ are in the local coordinates, and only local knowledge of the environment is needed. (3) Our method can be adaptable to dynamic environments, it has fast response to moving obstacles. (4) Our method can make different mobile robots coordinate to avoid collision with each other without “dead-lock”.

Figure 3. Two mobile robots are moving in an uncertain static environment with “SIA”-shape obstacles.

Figure 4. Two mobile robots are moving in a uncertain complex environment with static and moving obstacles.

6. Conclusions

The fundamental behaviors in motion planning of mobile robots are regarded as dynamic processes, and thus are modeled and controlled for motion planning purpose. Control of the behavior dynamics is shown to

be an acceleration control problem of the mobile robot. The behavior dynamics and the robot’s dynamics are formulated into an integrated planning-and-control system. The motion-planning problem is thus transformed into a sensor-based control problem. The stability of a desired behavior can be guaranteed. By controlling and modeling of the behavior dynamics, a unique insight to the motion-planning problem is provided. Our method can effectively consider the robot dynamics into the motion-planning problem, use only relative coordinates and local knowledge of the environment, and respond quickly to obstacles of arbitrary shape. It should be noted that, behavior dynamics may also be used in some other motion control problems of robots such as formation control, cooperation of multi-robots, tele-operation, etc. Further study will focus on these problems.

Correspondence to:

Xingjian Jing

Robotics Laboratory

Shenyang Institute of Automation

Chinese Academy of Sciences

Nanta Street 114, Shenyang, 110016, China

E-mail: xjjing@sia.cn

References

[1]    Salichs MA, Moreno L. Navigation of mobile robots: open questions. Robotica 2000;18:227-34.

[2]    Hwang YN, Ahuja. Gross motion planning - a survey. ACM Computing Surveys 1992;24(3):220-91.

[3]    Canny J, Reif J. New lower bound technigues for robot motion planning problems, 28th IEEE Symp on Found of Comp Sci. Los Alamitos, 1987.

[4]    Sanborn J, Hendler J. A model of reaction for planning in dynamic environments. Int J Art Intell Eng 1988;3(2):95-101.

[5]    Fujimura K, Samet H. A hierarchical strategy for path planning among moving obstacles. IEEE Trans on Robot and Automat. 1989;5(1):61-9.

[6]    Reif J, Sharir M. Motion planning in the presence of moving obstacles. J ACM. 1994;41(4):764-90.

[7]    Svestka P, Hovermars M. Coordinated path planning for multiple robots. Robotics and autonomous systems 1998;23:125-52.

[8]     Kant K, Zucker S. Towards efficient trajectory planning: the path-velocity decomposition. Int J Robot Res  1986;5(3):72-89.

[9]     Fraichard T. Dynamic trajectory planning with dynamic constraints: a state-time space approach. IEEE/RSJ Int Conf on Intell Robots and Sys 1993:1393-400.

[10]  Fujimura K. Time-minimum routes in time-dependent networks. IEEE Trans Robot Automat 1995;11(3):343-51.

[11]   Ferrari C, Pagello E, Ota J, Arai T. Multi-robot motion coordination in space and time. Robotics and autonomous systems 1998;25:219-29.

[12]  Krogh B. A generalized potential field approach to obstacle avoidance control. Robotics Research: The next five years and beyond, Bethlehem, PA, USA, Aug, 1984, pp 950-955.

[13]  Samer A. Masoud, Ahmad A. Masoud. Constrained motion control using vector potential fields. IEEE trans Systems, Man and Cybernetics—part A: Systems and Humans. 2000;30(3).

[14]  Guldner J, Utkin VI. Sliding mode control for gradient tracking and robot navigation using artificial potential fields. IEEE Trans Robot and Automat. 1995;11(2):247-53.

[15] Tsourveloudis NC, Valavanis KP, Hebert T. Autonomous vehicle navigation utilizing electrostatic potential fields and fuzzy logic. IEEE trans On Robotics and Automation 2001;17(4):490-7.

[16]   Jing XJ, Wang Y. Artificial Coordinating Field (ACF) Control of Mobile Robot for Collision Avoidance and Navigation. The 2002 international conference on Control and Automation, June 15-18, Xiamen, China, pp 2478-2482.

[17]   Jing XJ, Wang Y. Artificial Coordinating Field based Coordinating Collision Avoidance. IEEE International Conference on Robotics, Intelligent Systems and Signal processing, Changsha, Hunan, China, 2003, pp 126-30.

[18]  Oustaloup A, Orsoni B, Melchior P, Linares H. Path planning by fractional differentiation. Robotica 2003;2159-69.

[19]  Balch T, Arkin RC. Behavior-based formation control for multirobot teams. IEEE trans Robotics and automation 1998;14(6):926-39.

[20]  Jing XJ, Wang Y, Tan D. Cooperative motion behaviors using biology-modeling behavior decision-making rules. Control Theory and Application (in Chinese). 2003;20(3):407-10.

[21]   Jing XJ, Wang Y, Tan D. Rational genetic algorithm and its application to motion cooperation of multiple mobile robots. Acta Automatica Sinica (in Chinese). 2002;28(6):869-875.

[22]  Pham DT, Parhi DR. Navigation of multiple mobile robots using a neural network and a petri net model. Robotica 2003;21:79-93.

[23]  Zalama E, Gaudiano P, Lopez-Coronado J. A real-time unsupervised neural network for the low level control of a mobile robot in a nonstationary environment. Neural Networks 1995;8:103-23.

[24]  Nefti S, Oussalah M, Djouani K, Pontnau J, Intelligent adaptive mobile robot navigation. Journal of Intelligent and Robotic Systems 2001;30:311-29.

[25] Fujimori A, Teramoto M. Cooperative collision avoidance between multiple mobile robots. Journal of robotic system 2000;17(7):347-63.

[26]   Sgorbissa A, Arkin RC. Local navigation strategies for a team of robots. Robotica 2003;21:461-73.

[27]  Fiorini P, Shiller Z. Motion Planning in dynamic environments using the relative paradigm. IEEE Int Conf of Automat and Robotics 1993;1:550-66.

[28]  Chakravarthy N, Ghose D. Obstacle avoidance in a dynamic environment: a collision cone approach. IEEE Trans Syst Man and Cyber – part A: Syst and Humans. 1998;28(5):562-74.

[29]  Victorino AC, Rivers P, Borrelly JJ. Safe navigation for indoor mobile robots, part I: a sensor-based navigation framework. The Int J Robotics Research 2003;22(12):1005-18.

[30]  Hsu D, Kindel R, Latombe JC, Rock S. Randomized kinodynamic motion planning with moving obstacles. Int J Robotics Research 2002;21(3):233-55.

.