Timed Colored Petri Nets Based

Modelling and Scheduling of Aero-engine maintenance

Xinmin Tang， Shisheng Zhong

School of Mechatronics Engineering, Harbin Institute of Technology, Harbin, Heilongjiang 150001, China

txmofhit@hit.edu.cn; zss@hit.edu.cn

Abstract: Aero-engines maintenance procedure can be treated as a typical discrete event dynamic system. A theory that synthesized timed colored Petri net is proposed. The theory adopts a bottom-up approach by merging all sub modules, and each one characterizes the critical resource contention among processes. Nevertheless Petri net seems inapplicable to dynamic scheduling modelling because of its structural rigidity. Shared composition method of Petri net is introduced, which makes its structure adjustable dynamically. Given a set of preemptive dispatching rules, the makespan can be analyzed using linear state equation in the sense of max-plus algebra. To minimize makespan of each aero-engine, genetic algorithm is adopted to arrange the firing sequence of transitions. Dynamic scheduling strategy uses rolling horizon composed by colors is presented. According to the simulation result, the module is verified to be effective and robust. [Nature and Science. 2006;4(2):46-51].

Keywords: timed colored Petri nets; shared composition; static scheduling; dynamic scheduling

1 Introduction

Aero-engine maintenance procedure can be treated as a typical discrete dynamic system (DES). It involves planning, scheduling and controlling of resources, aerial materials, activities, etc. Woxvold proposed a solution to planning and scheduling of aircraft maintenance by combining of MRP (Materials Requirements Planning), PAC (Production Activity Control) and CPM (Critical Path Method) into a unitary structuring technique, which was applied to maintenance of Boeing 737 commercial aircraft[1]. Goldratt used project management technology based on TOC (theory of constraints) to solve maintenance scheduling problem in Israeli air force, which shorten aircraft maintenance duration dramatically by some case studies[2].

In the maintenance work shop, maintenance tasks with different work level may be carried out concurrently, as a result machines resource conflicts are unavoidable. Additionally, maintenance procedures are always interrupted by stochastic events unexpected. Scheduling methods mentioned above do not provide strict mathematical model and are usually applied to static other than dynamic environment. Petri nets as a tool to characterize a concurrent, asynchronous, conflicting, stochastic system is widely used in discrete dynamic system modelling, simulating and scheduling[3]. Timed colored Petri nets is one of high level model which has advantages over basic Petri nets in making the model of  a DES much more compact and concise.  In this paper, this is gained by introducing “colors” to distinguish among tokens presenting different aero-engines.

The organization of this paper is described as follows. In Sections 2, sub modules based on timed colored Petri nets describe interactions between components of aero-engine and machine resources are built, and then shared composition method of Petri nets is adopted to realize bottom-up modelling . In Section 3, the makespan of each aero-engine is analyzed using max-plus algebra, then Genetic algorithm is adopted to optimize the firing sequence of transitions during static scheduling. In Section 4, to deal with discrete events emerge in workshop, a dynamic scheduling strategy uses rolling horizon is presented, which made the model more adaptive and robust. In Section 5, an example is demonstrated and the performance is analyzed. Finally, conclusions are provided.

2 Bottom-up Modelling of Aero-engine Maintenance Procedure

- 2.1  Construction of Sub Modules

Suppose that the directional chart G represents the maintenance workflow of an aero-engine component. Each node of the chart is multivocal, firstly it specifies the maintenance process wi∈W, where Wrepresents all processes of the component, secondly it specifies the machine mj∈Mto process wi, where M represents all machines. It is assumed that one process only uses one machine and the mapping between WandMis f:W↦M. For w1(m1),w2(m2)∈W, if there exists a directional edge, then the component must be transferred to machine m2 after accomplished processing in machine m1.

Definition 1 (Work Processing Module). WPM is a state machine WPM=(PW,TW,IW,OW,MW0)and satisfies the following conditions: (1) There exists but only one place pidle∈PW is called idle place and MW0(pidle)>0. (2) ∀p∈ PW−pidle is called operation place and MW0(p)=0.

The rules to construct WPMfrom G are described as follows: (1) Build up one by one mapping from nodes W to transitions TW, g:W↦TW. (2) For wu,wv∈W, if there exists a directional edge, then add place pv to PW, let I(pv,tv) =O(pv,tu)=1 and MW0(pv)=0. (3) If in-degree deg+(wi) of wi∈Wis zero, then add idle place pidle to PW, let I(pidle,ti)=1 and MW0(pidle)=1.

Definition 2 (Resource Sharing Module). RSM is extended from WPMand satisfies the following conditions: (1) ∀p∈PM is called resource place and |MM0(p)|≥1 represents the quantity of the resource. (2) ∀t∈TW, ∃p∈PM, such that p∈t∩t, where IM(p,t)= OM(p,t)=1, which means resources are not exhaustive and should be released after the process is accomplished.

The rules to extendRSMfrom WPMare described as follows: (1) ∀t∈TW, Build up one by one mapping from M to PM, h: M↦PM. (2) ∀p∈PM, ∀t∈TW, if f(g−1(t))=h−1(p), then let IM(p,t)=OM(p,t)=1, else let IM(p,t)=OM(p,t)=0.

According to definition 2, the resource place looks like “resource dispatching and recycling center”. Its initial marking specifies that the resource is available and processes related must compete for the resource[4].

- 2.2  Extension to Sub Modules

It is obvious that RSM reveals the interactions between processes of a single component instance. In order to characterize the resources contention among multiple instances of the component, the original Petri nets should be expanded for more meaningful. An effective resolution is to set up relationship between color set and nodes of Petri nets including places and transitions. Additionally, the deterministic time delay should be related with the transitions of RSM.

Definitions 3 (Timed colored resource sharing module). TCRSM extended by RSMis defined as follows: TCRSM=(PW∪PM,T,C,I,O,M0,D) is timed colored Petri nets, where PW are maintenance operation state places, PM are machine resource places; T=TW are maintenance processes; C is color function defined from places and transitions into color set, ∀pi∈PW, C(pi)=ai,1,⋯,ai,ui, ui=|C(pi)|, ∀pj∈PM, C(pj)=mj, mj∈M; ∀ti∈TW, C(ti)=C(pi)∪mi;  I:C(p)× C(t)↦N is input function that specifies input place of transition t; O:C(t)×C(p)↦N  is output function that specifies output place of transition t; initial marking M0 is defined from PW∪PM into vector and M(p)(c)represents the token number with a binding color c in the place p;D:C(T)↦R+ is time delay function specify the firing time duration of transitions.

Transition ti∈T with a binding color ai,h is said to be firing enable at marking a M if ∀p∈ti M(p)(ai,h)≥I(ai,h,ai,h) and M(p)(mi)≥I(mi,mi), where mi∈M. When enabled transition ti begins to fire, the new marking is calculated from:

∀pi∈ti:M'(pi)(c)=M(pi)(c)−I(c,c),c=ai,h,mi (1)

Once transition ti begins firing, it will finish after duration D(ti). The new marking is calculated from:

∀pj∈ti:M'(pj)(c)=M(pj)(c)+O(c,c),c=ai,h,mi(2)

The flowing of colored token from place to place represents the components of aero-engine transferring from one machine to another, which reveals’ the busy and idle state of machine resources in the discrete time duration. TCRSM based on timed colored Petri nets has properties as follows:

Property 1 (Conservation). Note that the WPM is a state machine, then ∀p∈PW, such that |p|≤1 and |p|≤1, so ∀M∈R(M0), such that ΓP→PW(M)= ΓP→PW(M0), where ΓX→Y(Z) represents the projection of Z from X to subset Y. For RSM, ∀p∈PM: I(p,t)=O(p,t), in other words, resource tokens are not exhaustive and can be reusable after released, so ΓP→PM(M)=ΓP→PM(Mi). Synthetically, there exists vector  ϖT=[1,1,⋯1]T, such that ϖTM=ϖTM0, where M(pi)=ni,hai,h, ni,h=M(pi)(ai,h) is the quantity of token with a binding color ai,h. So TCRSM is conservative.

Property 2 (Boundedness). Note that RSM is conservative, then there exist ϖTM=ϖTM0, where ϖT= [1,1,⋯1]T, that is ∀pi∈PW∪PM, satisfies M(pi)=ni,hai,h, then ∀pi∈PW∪PM: M(pi)≤ ni,hai,h, so TCRSM is bounded.

Property 3 (Deadlock-free). For RSM, ∀p∈PM, ∃t∈ TW, such that IM(p,t)=OM(p,t), that is p=p, which means RSM contains siphons and traps, call them Σ. If for every resource place of Σ, there exists at least one token, that is ∀p∈Σ:M0(p)>0, each siphon contains a marked trap (MST), so TCRSM is deadlock-free[4].

- 2.3  Shared Composition of Sub Modules

To characterize the resources contention among inhomogeneous components have different processing routines, shared composition of sub modules is introduced. Jiang Changjun discussed the state constancy and behavior constancy on shared composition Petri nets[5].

Definition 4 (Shared composition). For timed colored Petri nets TCRSMi=(PWi∪PMi,Ti,Ci,Ii,Oi,Mi), where PW1∩PW2=Φ, PM1∩PM2≠Φ, i=1,2. If TCRSM=(P,T,C,I,O,M) satisfies that: (1) P=P1∪P2, P1∩P2≠Φ; (2) T=T1∪T2, T1∩T2=Φ; (3) I=I1∪I2, O=O1∩O2; (4) if  p∈PM1∩PM2, let C(p)= C1(p)∪C2(p), else if p∈PW1∪PW2, let C(p)=Ci(p), i=1,2; (5) if   p∈PM1∩PM2, let M(p)=ni⋅mi, where  ni=maxM1(p)(mi),M21(p)(mi), else if p∈PW1∪PW2, let M(p)=Mi(p),i=1,2, TCRSM is the shared composition Petri nets of TCRSM1 and TCRSM2, for short, TCRSM= TCRSM1ΟpTCRSM2.

Property 4 (State constancy).  Suppose that TCRSMi= (PWi∪PMi,Ti,Ci,Ii,Oi,Mi) is timed colored Petri nets, i=1,2, and TCRSM=TCRSM1ΟpTCRSM2. If ∀M∈R(M0), ∃Mi∈R(M0i), such that   ΓP→(Pi−(P1∩P1))(M)=ΓP→(Pi−(P1∩P1))(Mi), then TCRSM is state constant, where Pi=PWi∪PMi, i=1,2.

Proof: Let TCRSMi=(Pi,Ti,Fi,M0i), i=1,2 be sub nets of TCRSMi and satisfy the following conditions: (1) Pi=PWi; (2) Ti=Ti; (3) Fi=Ii∩(PWi×Ti)∪ Oi∩(Ti×PWi); (4) if p∈PWi, then Mi0(p)=M0i(p). Obviously, ∀Mi∈R(Mi0), such that ΓP→PW(M)= ΓP→Pi(M)∈R(M0i), then ∃Mi∈R(M0i), ΓP→PW(M)= ΓP→PW(Mi) is satisfied. Since Pi−P1∩P2=PWi, according to definition, TCRSM satisfies state constancy.

State constancy reveals that sub modules keep mark of all places except shared ones unaltered after merged, which means in the merged module, new arrival component does not change the state of processing components. This property provides theoretic support for dynamic scheduling driven by events.

3 GA Based Multiple Aero-engine Static Scheduling

Static scheduling can be described as follows: all components are ready for processing at the beginning and there are no new arrivals, after scheduling the processing orders of all components are assume to be unaltered and machines never break down.

- 3.1  Static Scheduling Makespan Calculating

There are three ways to dealing with scheduling problem in Petri nets. One is by using reachability graph to determine a firing sequence of transitions to get the optimal schedule. The second way is by heuristic search of reachability graph. The last is using decision rules to solve conflicts[6]. In this paper, genetic algorithm is adopted to determine the firing sequence of transitions Petri nets.

Suppose that the processing order is given, which means the firing sequence of transitions with binding color instances is σ=, then the deterministic state transmission equation can be describe as: M(n+1)=M(n)+W⋅S(n), where W is weight function defined from colored arcs, S(n) is the firing vector from nth to (n+1)th step. According to the firing rules of timed Petri nets, if transition ti with a binding color ai,h is fire enable, the firing procedure is summarized as follows:

1) Before (n+1)th step, suppose that arrival timestamp of the token with color ai,h in input operation place of transition ti is fti(ai,h,n), arrival timestamp of the token with color mi in input resource place of ti is fti(mi,n), then the firing timestamp eti(ai,h,n+1) of transitions F(L)=Tmax−T(L)Tmax−Tmin+ε with a binding color ai,h is calculated from:

eti(ai,h,n+1)=fti(ai,h,n)⊕fti(mi,n)           (3)

2) After (n+1)th step, transition tiaccomplishes firing after a duration di, the timestamp is fti(ai,h,n+1), which is calculated from:

fti(ai,h,n+1)=eti(ai,h,n+1)⊗difti(mi,n+1)=fti(ai,h,n+1)            (4)

In formulas (4) above, max-plus algebra symbols “⊕” and “⊗” are introduced. The two symbols are defined respectively as follows: set2 and a⊗b=a+b. Suppose that initial timestamp is fti(ai,h,0)=fti(mi,0)=0 and the iterations ends after N steps when there are no enable transitions, then the makespan is calculated from T=ft1(c,N)⊗ ft2(c,N)⋯⊗ftu(c,N), where u=|T|.

Obviously, given the firing sequence of transitions, makespan can be calculated within linear time complexity. In the next section, genetic algorithm is adopted to determine the firing sequence of transitions Petri nets.

- 3.2  Genetic Algorithm Based Static Scheduling

Genetic algorithm behaves excellently when dealing with configuration optimization problems. In this paper, Genetic operator encodes the transition firing sequence of Petri nets into chromosome and the scheduling algorithm decodes the chromosome into schedule. In the later genetic algorithm search procedure, population evolves generation by generation to converge to an optimal or near optimal solution.

1) Encoding Cheng analyzed the advantages and disadvantages of nine encoding schemas for classical job shop scheduling[7]. In this paper, each gene represents a transition and chromosome represents the transitions firing sequence. Encoding of transitions firing sequence can be carried out by using natural numbers. To be specific, all transitions in the sub module are encoded by the same number which is different from other sub modules, and every transition can be distinguished from each other by order of number in chromosome.

- Table 1. Encoding Schema of Chromosme

| Process | 1-1 | 2-1 | 1-2 | 2-2 | 2-3 | … |
| --- | --- | --- | --- | --- | --- | --- |
| Transition | t1(h1) | t1(h2) | CPNH=(PH,TH,CH,IH,OH,MH0) | t2(h2) | t3(h2) | … |
| Chromosome | 1 | 2 | 1 | 2 | 2 | … |

Suppose that two instances of aero-engine module h1 and h2 are operated in work shop concurrently, then genes must be either “1” or “2”. For example, as shown in table 1. if transitions in sub module of h2 are described as t1(h2), t2(h2), …, then the first “2” must represents t1(h2), and the later must represents t2(h2), the rest may be deduced by analogy.

2) Selection Single aero-engine maintenance scheduling aims for minimal makespan, therefore those chromosomes with minor scheduling makespan owned more fitness. Suppose that there are N individuals in the generation G, for each chromosome L∈G, its fitness is calculated from:

F(L)=(Tmax−T(L))/(Tmax−Tmin)+ε          (5)

where ε is a compensate value to prevent the fitness from being zero, Tmax and Tmin are the maximal and minimal makspan values of individuals from G respectively. If the individuals of next generation are chosen by roulette, then those fitter ones will survive at higher possibility.

If multiple aero-engines are operated in work shop concurrently, the fitness is multi-objective function. According to weighted sum approach proposed by Tasahiko, the weighted fitness function FΣ(L) is defined as follows:

FΣ(L)=wi⋅Fi(L)                    (6)

where n is the number of aero-engines, Fi(L) is the fitness function of ith chromosome, wi is the weighted factor of ith chromosome.

3) Crossover Illegal chromosomes maybe results from ordinary crossover operation, to avoid which a new crossover operation strategy is proposed. To be specific, first dividing the components’ numbers into two subsets named set1 and set2,  and making sure that the intersection of the two subsets is empty, then scan the two father chromosomes and let son chromosome be:

son1=Γparent1→set1(parent1)∪Γparent2→set2(parent2))   (7)

where Γparent1→set1(parent1) represents the projection of parent1 from parent1 to subset set1, finally exchangingparent2 withparent1, a new son chromosome is produced after repeating the operation above.

4) Mutation Mutation is fairly straightforward. The procedure is to exchange two genes in chosen chromosome randomly.

4 Rolling Horizon Based Dynamic Schduling

Aero-engine maintenance is a dynamic procedure and may be interrupted by stochastic events unexpected, therefore seeking for dynamic scheduling strategies adaptive to the alteration seems to be necessary considerably. Obviously, it is robust and sensitive to unexpected events, which differs from static scheduling.

- 4.1  Rolling Horizon and Scheduling Sub Module

Inspired by the rolling horizon optimization for the predictive control technology, Fang Jian proposed a period and event-driven rolling horizon scheduling strategy, which means scheduling objects are those components included in rolling horizon[8]. In this paper, corresponding definitions about rolling horizon in timed colored Petri nets are defined as follows:

Definition  5 (Rolling horizon). Rolling horizon CH is defined as the subset of colors of the merged timed colored resource sharing module TCRSM, that is CH⊆C. The color set number of rolling horizon |CH| is called capacity.

Definition 6 (Dynamic scheduling sub module). For sub nets CPNH=(PH,TH,CH,IH,OH,MH0) of TCRSM,  which satisfies: (1) PH=p∈P|CH(p)≠Φ∪PM; (2) TH=t∈T|CH(t)≠Φ; (3) if p∈PH, and t∈TH, letIH(p,t)=I(p,t); (4) if p∈PH, and t∈TH, let OH(p,t)=O(p,t), CPNH is defined as the dynamic scheduling sub module of TCRSM corresponding to Rolling horizon  CH.

There are two ways to construct scheduling sub module: one is to reconstruct the module by “bottom-up modelling” and keep the marking of operation places and resource places unaltered in TCRSM, the other is to decomposed the merged TCRSM into two sub ones and one of whose color set is rolling horizon. The scheduling sub module is the reduction of merged system and used to dynamic scheduling.

- 4.2  Rescheduling Driven by Events and Period

There are two strategies for rescheduling: continuous scheduling and periodical scheduling. The former executes rescheduling once events emerges, which can cope with unexpected events, while the latter executes rescheduling periodically. In this paper, a hybrid strategy is adopted. Some key events including new project arriving, due date changing and machine breaking down are defined as following. If any key event takes place, then executes rescheduling, else executes rescheduling periodically.

1) New componentsCN Arriving New instances should be added to module, which means new colors should be added to color set of  merged TCRSM, that is C'W(PW)=CW(PW)∪CN, C'W(TW)=CW(TW)∪CN, and keep the sate of processed and processing components unchanged, that is ∀p∈PW, ∀c∈CW: M'(p)(c)= M(p)(c).

2) Changing of due date Tci Suppose that Fi(L)is the makespan of ith chromosome, wi is the weighted factor of ith chromosome, if the variation value of Tci is ΔTci, then wi should be adjusted, let w'i= wi⋅(1+ΔTci/Fi(L)) and the weighted fitness function FΣ(L) should be adjusted accordingly.

3) Machine m∈M breaking down   If a machine breaks down while machining, then remove the corresponding colored token kept by transition and keep the colored tokens’ number M(p) in resource places unchanged, else if the machine is idle and becomes unusable, then let  M'(p)=M(p)−1. If the broken down machine is recovered and becomes usable, the let M'(p)=M(p)+1, where p=h(m), h is the mapping function from machines to resource places.

5 Case Studying

Assembly workflow of aero-engine is shown in Figure 1, eight modules of aero-engine at the most left denote branches in the assembly procedure where each branch means one part of engine. Each part of an aero-engine is machined, balanced and assembled. In addition, blocks with the same name denote the same type of machine. The number above a block means the operation time of the operation on this machine. Arrows means sequence of operations.

- Figure 1. Aero-engine assembly workflow

Aero-engine assembly module represents by Petri nets is shown Figure 2. The mapping procedures from workflow can be described as follows: firstly the Resource Shared Modules of HPC, T2N, HPTR, HPCR, LPT, CRF, AGB, FAN are constructed respectively according to the workflow of each module, then merge all eight Resource Shared Modules by shared composition.

Figure 2. Petri nets module of aero-engine assembly

As shown in Figure 3. we choose 3 aero-engines situation with starting simultaneously. The rationality of Petri nets is verified by three aspects: (1) All processes follow the sequence of workflow strictly. (2) All processes are executed one by one without overlapping. (3) All processes are executed as closely as possible, which reduce the running time and increase the efficiency of utilization of machines.

- Figure 3. Static scheduling result for 3 aero-engines

From static scheduling result, for 1PstP aero-engine it takes 174.7 hours to accomplish the assembly procedure, while it takes only 366.4 hours for the 3 aero-engines. This is because the total time duration of aero-engines is only regarding the path which require the longest operation time. We maximize the utility of critical machines to minimize the time duration of processes operated on them. Thus, the makespan is not proportional to the number of aero-engines.

6 Cconclusion

To cope with the problem of concurrency and asynchrony of multiple aero-engines maintenance, shared composition method of Petri net is adopted to realize bottom-up modelling. The constructed model characterizes interactions between components with different processing routines and machine resources. Genetic algorithm is adopted to arrange firing sequence of transitions for static scheduling. To deal with discrete events emerge in workshop, a dynamic scheduling strategy uses rolling horizon composed by colors is presented, which made the model more adaptive and robust.

# Acknowledgments

The authors also wish to acknowledge the financial support of the National Natural Science Foundation of China (60373102, 60572174).

Correspondence to:

Xinmin Tang

School of Mechatronics Engineering

Harbin Institute of Technology

Harbin, Heilongjiang 150001, China

Telephone: 01186-451-8641-3847

E-mail: txmofhit@hit.edu.cn

References

- P. Samaranayake, Development of engineering structures for scheduling and control of aircraft maintenance, International Journal of Operations & Production Management 2002;22(8): 843-867.

- Junyu Chen, The Study of Maintenance Scheduling in Aviation Engines, National Chiao Tung University, Taiwan, 2003: 9-12.

- Mu Der Jeng, Frank DiCesare, Synthesis Using Resource Control Nets for Modelling  Shared-Resource Systems, IEEE Transactions on Robotics and Automation 1995; 11( 3):317-327.

- Claude Girault, Rüdiger Valk, Petri Nets for Systems Engineering A Guide to Modelling , Verification, and Application, Springer-Verlag Berlin Heidelberg, 2003:214-215.

- Jiang Changjun, Dynamic constancy of Petri nets, Science in China (Series E) 1997; 27(6):567-573.

- Zhonghua Huang, Zhiming Wu, Deadlock-Free Scheduling Method for Automated Manufacturing Systems Using Genetic Algorithm and Petri Nets, Proceedings of the 2004 IEEE International Conference on Robotics & Automation 2004;566-571.

- Pan Quanke, Multi-Objective Scheduling Optimization of Job-shop in Intelligent Manufacturing System, Nanjing University of Aeronautics and Astronautics, Nanjing, 2003:20-25.

- Fang Jian, Xi Yugeng, “The Genetic Algorithms-Based Rolling Horizon Scheduling Strategy,” Control Theory and Appliciations 1997;14(4):589-594.