Evaluation of Water Quality: Physico – Chemical Characteristics of Ganga River at Kanpur by using Correlation Study

Priyanka Trivedi1, Amita Bajpai2 and Sukarma Thareja1,*

1Department of Chemistry, Christ Church College, CSJM Kanpur University, UP, India

2CWA Kanpur Jalsansthan Benajhawar, Kanpur

*E-mail address: sukarmathareja@yahoo.com

Abstract:  We present an extensive investigation of physico-chemical parameters of water samples of Ganga river at Kanpur. Water samples under investigations were collected from Jalsansthan Benajhawar Kanpur sampling station during pre monsoon (April - May), monsoon (July - August) and post monsoon (October - November) seasons in the year 2008. Correlation coefficients were calculated between different pairs of parameters to identify the highly correlated and interrelated water quality parameters and t-test was applied for checking significance. The observed values  of different  physico-chemical parameters like  pH, temperature,  turbidity , total hardness (TH), Iron, Chloride, total dissolved solids(TDS), Ca2+, Mg+2 , SO4-2, NO3-, F-1, total alkalinity (TA), Oxygen consumption (OC), Suspended solids (SS) of samples were compared with standard values recommended by world health organization (WHO). It is found that significant positive correlation holds for TA with Cl-, Mg+2, Ca+2, TH, TDS, fluoride and OC. A significant negative correlation was found between SS with chloride, Mg+2, TDS, fluoride and OC. All the physico - chemical parameters for pre monsoon, monsoon and post monsoon seasons are within the highest desirable or  maximum permissible limit set by WHO except turbidity  which was high while NO3- , Cl-1 and F- are less than the values prescribed by WHO. [Nature and Science. 2009;1(6):91-94]. (ISSN: 1545-0740).

Keywords: Physico-chemical parameters, correlation, t-test.

1. Introduction

It is a fact that good water quality produces healthier humans than one with poor water quality. Ganga River is life line of Kanpur and its water is used for domestic and agriculture purposes therefore, effective maintenance of water quality is required through appropriate measurements. Physico-chemical and micro-biological characteristics may describe the quality of water (Sinha, 1986), therefore, an analysis on physico-chemical parameters of Ganga water was made by many workers (Mehrotra, 1990;. Sinha et.al. 2000) Regular monitoring of all the parameters is very difficult and laborious task even if adequate manpower and laboratory facilities are available. Therefore, statistical correlation technique has been used for comparison of physico-chemical parameters. The present work deals with the study of 15 physico-chemical parameters like pH, temperature,  turbidity , TH ,  Fe , Cl- , TDS ,  Ca2+, SO4-2 , NO3- , F-1, TA,  Mg+2 ,  OC,   SS of Ganga river water in Kanpur. The observed values of various physico-chemical parameters of water samples were compared with standard values recommended by World Health Organization (WHO) and are given in Table-1. The objective is to minimize the complexity and dimensionality of large set of data. Systematic calculation of correlation coefficient between physico-chemical parameters has been carried out and significant correlation has been further verified by using t- test. (Bhandari and Nayal, 2008; Garg et.al, 1990; Sarkar et. al., 2006).

2. Experimental

Water samples were collected from Jalsansthan Benajhawar Kanpur sampling station during pre monsoon(April - May), monsoon (July - August) and post monsoon (October - November) phase in year 2008. During sampling pH, temperature, and turbidity  were determined using digital pH meter, thermometer and turbidimeter respectively. F- and nitrate was estimated using colorimetric method. The laboratory analysis of samples was done using standard methods (APHA,1998), titrimetric method was used for the determination of total alkalinity and gravimetric method for total dissolved solid and total suspended solids Mohr’s argentometric titration method was used for chloride (Vogel, 1978). Sulphate was estimated using turbidometric method

(Vogel,1978) Whereas Ca+2  ,  Mg+2  and   TH   was determined      by   EDTA     titrimetric         method (Vogel, 1978). Atomic absorption spectrophotometer was used for determination of Fe and Cr contents. All the chemical used were of AR grade .In order to calculate correlation coefficients, correlation matrix was constructed by calculating the coefficients of different pairs of parameters and correlation for significance was further tested by applying t-test (http://www.spss.com) decreases or increases with increasing or decreasing n values of chloride ion.

A significant positive correlation was found between Ca2+ and TDS (r=0.816, t = 8.711). The magnesium content of Ganga water increases or decreases with increase or decrease in the value of TDS (r = 0.889, t= 9.179) and fluoride(r = 0.977, t = 6.239) as it shows significant positive correlation with these parameters respectively

Total dissolved solid shows significant positive correlation with OC(r = 0.827, t = 9.07)  and fluoride (r = 0.89, t = 9.051), TDS shows significant negative correlation with SS ( r = - 0.971, t = 3.049).SS shows significant negative correlation with OC(r = - 0.894, t = 3.817) and fluoride(r = - 0.928, t = 4.016).

Table 1. The average values of physico- chemical parameters of Ganga River water at Kanpur.

HDL: Highest Desirable Limit; : MPL; Maximum Permissible Limit

| No. | Parameters | Units | Drinking Water WHO Standard |  | Experimental Values (Range) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | HDL | MPL |  |
| 1 | Temperature | 0C | --- | ----- | 22-30 |
| 2 | Turbidity | NTU | 5 | 10 | 18-470 |
| 3 | pH value | - | 6.5 to 8.5 | No relaxation | 8.4- 8.9 |
| 4 | Total hardness (as CaCO3) | mg/l | 300 | 600 | 122-212 |
| 5 | Iron | mg/l | 0.3 | 1.0 | 0.2-0.8 |
| 6 | Chlorides | mg/l | 250 | 1000 | 7-26 |
| 7 | Dissolved Solids | mg/l | 500 | 2000 | 256-500 |
| 8 | Calcium | mg/l | 75 | 200 | 28-48 |
| 9 | Sulphate | mg/l | 200 | 400 | 50-91 |
| 10 | Nitrate | mg/l | 50 | No relaxation | 0-1.772 |
| 11 | Fluoride | mg/l | 1.0 | 1.5 | 0-0.4 |
| 12 | Total Alkalinity | mg/l | 200 | 600 | 13-246 |
| 13 | Magnesium | mg/l | 30 | 150 | 9.23-26.24 |
| 14 | Oxygen Observed from KMnO4 at 370C in 3 hrs. | mg/l | 3.0 | No relaxation | 2.4-7.8 |
| 15 | Suspended Solids | mg/l | 20 | 150 | 70-280 |

| Paired Parameters |  | r | I t I |
| --- | --- | --- | --- |
| Pair 1 | pH & OC | 0.821 | 4.762 |
| Pair 2 | TA & TH | 0.97 | 4.694 |
| Pair 3 | TA & Ca+2 | 0.821 | 9.145 |
| Pair 4 | TA & Mg+2 | 0.851 | 10.179 |
| Pair 5 | TA & Cl- | 0.91 | 10.511 |
| Pair 6 | TA & TDS | 0.988 | 8.233 |
| Pair 7 | TA & OC | 0.88 | 9.942 |
| Pair 8 | TA & Fluoride | 0.836 | 9.85 |
| Pair 9 | TH & Mg+2 | 0.916 | 9.973 |
| Pair 10 | TH & Cl - | 0.87 | 10.091 |
| Pair 11 | TH & TDS | 0.993 | 8.653 |
| Pair 12 | TH & Fluoride | 0.912 | 9.516 |
| Pair 13 | Ca+2 & TDS | 0.816 | 8.711 |
| Pair 14 | Mg+2 & Cl - | 0.872 | 2.175 |
| Pair 15 | Mg+2 & TDS | 0.889 | 9.179 |
| Pair 16 | Mg+2 & SS | - 0.935 | 3.261 |
| Pair 17 | Mg+2 & Fluoride | 0.977 | 6.239 |
| Pair 18 | Cl - & TDS | 0.877 | 9.256 |
| Pair 19 | Cl - & OC | 0.985 | 4.639 |
| Pair 20 | Cl - & SS | - 0.923 | 3.348 |
| Pair 21 | TDS & OC | 0.827 | 9.07 |
| Pair 22 | TDS & SS | - 0.971 | 3.049 |
| Pair 23 | TDS & Fluoride | 0.89 | 9.051 |
| Pair 24 | OC & SS | - 0.894 | 3.817 |
| Pair 25 | S.S & Fluoride | - 0.928 | 4.016 |

Table 2. Different Paired Samples Correlations9

Note: Significant if t 0.05 >2.14

3. Results and Discussion:

The observed pH value ranging from 8.4 to 8.9 show that the present water samples are slightly alkaline in pre-monsoon season. These values are within maximum permissible limit prescribed by WHO.(www.lenntech.com/drinking-water-standards.htm.) Other parameters like turbidity (18 - 470 NTU), TH (122 - 212 mg/l), Fe contents ( 0.2 - 0.8 mg/l), Chloride ( 7- 26 mg/l), TDS ( 256 - 500 mg/l),The  Ca2+( 28 - 48 mg/l, SO4-2 ( 50 – 91 mg/l ), NO3-(0-1.772 mg/l), F-1 ( 0-0.4 mg/l),  TA (13 -246 mg/l) , Mg+2 (9.23-26.24 mg/), OC ( 2.4-7.8 mg/l), SS ( 70 -280 mg/l) are found within the highest desirable or maximum permissible limit set by WHO.(Trivedi and Goel., 1986) However, turbidity and Fe contents are observed to be on higher side in all seasons  and  pre-monsoon and monsoon seasons respectively.

In the present study for the year 2008, pH shows significant positive correlation with OC (r = 0.821, t = 4.762). A significant positive correlation was found between total alkalinity and total hardness ( r = 0.97, t = 4.694), Ca2+( r= 0.821, t = 9.145), Magnesium( r = 0.851, t = 10.179), Cl- ( r = 0.91, t =10.511), TDS( r=

0.988, t =  8.233), OC( r = 0.88, t = 9.942) and fluoride( r= 0.836, t = 9.85) This shows that with increase or decrease in the values of TA ;  TH , Ca2+, Mg2+, Cl -, total dissolved solids, OC and F-  also exhibit decrease or increase in their values.

Total hardness bears positive correlation with Mg2+ (r = 0.916, t = 9.973), Cl- (r = 0.87, t =10.091), TDS (r = 0.993, t = 8.653), and fluoride (r = 0.912, t = 9.516). It is suggested that total hardness of water samples is mainly due to the presence of the MgCl2  and Magnesium fluoride (Bhoi,2005) Sulphate and Nitrate  ions bear negative correlation with  Ca +2 indicating that Calcium sulphate  and Calcium nitrate may be absent in water samples. NO3– shows negative correlation with Mg +2 which indicates that Mg(NO3)2 may be absent in water samples. Similarly Fe  also shows negative correlation with Cl -1 and SO4 -2  implying iron chloride and  iron sulphate may be absent in water samples.

Chloride ion bears significant positive correlation with TDS  (r = 0.877, t = 9.256), Mg2+, (r = 0.872, t = 2.175), OC (r = 0.985, t = 4.639). It reveals that Mg2+ mainly remains present as MgCl2. Chloride ion showed negative significant correlation with SS (r = - 0.923, t = 3.348) so with increase or decrease in the values of chloride ion, the values of TDS, Mg2+ and OC increases or decreases, while suspended solids

Acknowledgment:

ST is grateful  to Dr. Pervez E. Deen, Principal Christ Church College, Kanpur for his encouragement throughout the work.

Correspondence to:

Sukarma Thareja

Department of Chemistry, Christ Church College CSJM Kanpur University

Kanpur 208001, UP, India

Telephone: +91- 0512- 2598306

Emails: sukarmathareja@yahoo.com;  sukarma@iitk.ac.in

References

[1]  Sinha U.K. ,Ganga pollution and health      hazard, Inter – India Publication, New Delhi,1986

[2]  Mehrotra M.N.  J. of  the Ind. Association of Sedimentologists,1990, 9 ,1-14

[3]  Sinha A.K. , Singh V. P. And Srivastava K., Physico –chemical studies on river Ganga and its tributaries in Uttar Pradesh –the present status. In pollution and Biomonitoring of Indian Rivers.(ed.) Dr. R.K. trevedi. ABD publishers, Jaipur. 2000,1-29

[4]  Bhandari N. S and Nayal K E-Journal of Chemistry, 2008, 5, No.2,  342-346

[5]  Garg D. K, Goyal R. N and Agrawal V. P, Ind. J. Envir.Prot.1990, 10(5), 355-359.

[6] Sarkar M, Banerjee A , Pratim.P and Chakraborty S, J. Indian Chem. Soc., 2006 , 83, 1023-1027.

[7]  Standard Methods for the Examination of Water and Waste Water, 20th Ed., APHA, AWWA, WEF.   Washington DC, 1998

[8]  Vogel A.I, A text book of Quantitative Inorganic Analysis Including Elementary Instrumental Analysis 4th Edition. The English Language Book Society and Langman.Co  1978 (a)837 (b) 328-329 (c)504-506

[9]  SPSS Advanced Models™ 12.0 Web site at http://www.spss.com

[10]  World Health Organization, Guidelines for drinking water quality-I, Recommendations, 2nd Ed. Geneva WHO, 1993.www.lenntech.com/drinking-water-standards.htm.

[11]  Trivedi R.K and Goel P.K, Chemical and  Biological Methods for Water Pollution Studies, Environmental Publication, India 1986

[12] Bhoi D. K, Raj D. S, Mehta Y M, Chauhan M .B and Machhar M .T, Asian J. Chem, , 2005, 17(1), 404 - 408.

.

## Date of Submission:  8/30/2009