PHYTOREMEDIATION OF CRUDE OIL CONTAMINATED SOIL: THE EFFECT OF GROWTH OF Glycine max ON THE PHYSICO-CHEMISTRY AND CRUDE OIL CONTENTS OF SOIL

NJOKU, K.L., AKINOLA, M.O. AND OBOH, B.O.

Department of Cell Biology and Genetics, University of Lagos, Akoka Lagos, Nigeria.

njokukelechi@yahoo.com

Abstract

The remediation of oil contaminated soils has been a major problem in oil producing countries and recently use of plants to clean such soils has been on investigation. In order to identify plants that can enhance the remediation of crude oil contaminated soil, the effect of the growth of G. max on the physico-chemistry and crude oil content of soil contaminated with different concentrations of crude oil was investigated in this study. The results revealed that the pH, moisture and organic matter contents of soils contaminated with crude oil were significantly affected by the growth of G. max at differently levels of significance (P<0.001, P<0.01 and P< 0.05). Crude oil loss was enhanced in soil with 25g crude oil in the presence of G. max. Although the growth of the G. max did not significantly affect the crude oil level in the 50g and 75 g treatments, the soils became more favourable for plant growth as weeds sprouted from the contaminated soil vegetated with G. max. The implication of the findings of this study is that within 110 days, growth of G. max can lead to cleanup of crude oil contaminated soil and the reduction in toxicity of crude oil in soil. The ability of G. max to reduce the level of crude oil in oil polluted soil can help to restore polluted soils back for agricultural use. The high acceptability of G. max due to its high nutritional value, high adaptability and ease of propagation will make it an easy tool for remediation of soil contaminated with crude oil. [Nature and Science. 2009;7(10):79-87]. (ISSN: 1545-0740)

Keywords: Contamination, Crude oil, Glycine max, Cleanup, pH, Moisture, Organic matter

Introduction

Since commercial exploration of petroleum started in Nigeria in 1958 (Okoh, 2003), petroleum has continuously grown to be mainstay of the Nigerian economy. However, the exploration of petroleum has led to the pollution of land and water ways. The agricultural lands have become less productive (Dabbs, 1996) and the creeks and the fishing waters have become more or less dead (Okpokwasili and Odokuma; 1990; Odokuma and Ibo, 2002). Several civil unrests due to environmental degradation due oil exploration have also been witnessed in the Niger Delta region of Nigeria (Inoni et al., 2006).

The physical, chemical and thermal processes are the common techniques that have been involved in the cleaning up of oil contaminated sites (Frick et al., 1999). These techniques however have some adverse effects on the environment and are also expensive (Frick et al., 1999; Lundstedt, 2003). Recently, biological techniques like phytoremediation are being evaluated for the remediation of sites contaminated with petroleum. Phytoremediation is the use of plants and/or associated microorganisms to remove, contain or render harmful material harmless (Cunningham et al., 1996; Schwab and Banks, 1999; Merkl, 2005). It has been shown to be effective for different kinds of pollutants (contaminants) like heavy metals, radionuclides and broad range of organic pollutants (Schroder et al., 2002; Schnoor, 2002)

According to Pivetz (2001), plants for phytoremediation should be appropriate for the climatic and soil conditions of the contaminated sites. Such plants should also have the ability to tolerate conditions of stress (Siciliano and Germida, 1998a). Njoku, et al., (2008a) demonstrated that G. max germinates and grows in crude oil polluted soil. Also Frick et al. (1999) included G. max in the list of plants that can grow and remediate petroleum hydrocarbon contaminated sites. However, no record has shown that G. max can remediate crude oil polluted soil. It is therefore important to study the ability of G. max to affect the physico-chemistry (pH, moisture and organic matter contents) and the crude oil content of soil polluted with crude oil. The overall goal of this investigation was to evaluate the suitability of G. max for use in remediation of crude oil polluted soil.

The study is significant for some reasons. Firstly, phytoremediation has mostly involved the use of weeds (Aprill and Sims, 1990; Lee and Banks, 1993; Schwab and Banks, 1994; Qui et al., 1997; Banks et al., 2000). The use of food crops will improve the economic value of the technique (Van de Lelie et al., 2001). Secondly, although the conditions in the tropics favour phytoremediation, few researches have been carried on this technique in the tropics (Gallegos Martinez et al., 2000; Merkl et al, 2005a). There is the need therefore to evaluate the potentials of phytoremdiation in the tropics especially in Nigeria where pollution due to oil activities is high. In addition, the high nutritional value of G. max makes it acceptable by many and Njoku et al., (2008b) reported that G. max has the potential of growing in sandy loam soil, a soil type found in Niger Delta region of Nigeria.

Materials and methods

This study was carried out in the Biological garden of the University of Lagos, Akoka Lagos, Nigeria. The crude oil (Wellhead medium) was obtained from the SPDC Port Harcourt while the G. max was obtained from the Gene Bank Section of IITA Ibadan, Nigeria. The soil used is sandy loam soil and the treatments included 25g, 50g, and 75g crude oil mixed with 4000g of the soil filled in plastic containers. For each treatment, the control had no G. max grown on it. Both the treatments and the control were replicated thrice. Seven seeds of G. max were sown into each of the containers at 2cm depth and the containers were moderately watered regularly to keep the soils moist.

Soil samples were collected at the surface and 15cm depth from each container every 21 days (3 weeks) for 105 days (15 weeks). The collected soil samples were used to investigate the effect of G. max on the pH, moisture and organic matter contents of crude oil polluted soil. The soils from the surface and 15cm depths were usually mixed together and the mixture used for the study of the above physico-chemical features. The soil samples used in the study of the effect of G. max on the crude oil content of the soil were collected on the 110th day of sowing of the seeds of G. max in the soils.

The pH of the homogenized soils was determined following the protocols outlined by Eckerts and Sims (1995). The soils were air-dried and sieved to remove large particles and

debris. 5g of the sieved soils were mixed with 5mls of distilled water and stirred very well after which mixture was allowed to stand for 30 minutes. The electrode of a pH meter was put into slurry of the soil-water mixture and the pH of the soil was read off. The moisture content of the soil samples was determined according to the method of Schneekloth et al. (2002). The procedure of Schulte (1995) was used to determine the organic matter content of the soil samples.

The amount of crude oil the soil samples was determined using air-dried soils that were sieved through 1mm mesh. The crude oil in the soil was first extracted with n-hexane by shaking with a mechanical shaker for 30 minutes as was described by Okolo et al. (2005). The soil-crude oil-n-hexane mixture was filtered into a beaker of known weight through a Whatmann No.1 filter paper. The crude oil content of the filtrate was determined after heating the beaker at 40oC to a constant weight (Merkl et al., 2005b). The amount of crude oil lost from the soil was determined as the amount of crude oil added to the soil minus that in the soil at the time of analysis.

The effect of G. max on the pH, moisture, organic matter and crude oil contents of the soils was determined by comparing each parameter in soil with G. max with that in soil with G. max. Statistical analyses of the data obtained were done using Graphpad Prism 5.0 package using a 2 way ANOVA followed by Bonferroni posttests at 5%, 1% and 0.1% significance levels. Correlation analyses were also carried out.

Result and Discussion

The growth of G. max generally reduced the acidity of the crude oil polluted soil. However on days 21 and 42, the growth of G. max led to increase in the acidity of crude oil polluted soil (Table 1). On days 21 and 63, the pH of the control differed significantly from those of soil with 50g crude oil and G. max (t = 2.701 for day 21 and t = 3.696 for day 63) and those of the soil with 75g crude oil and G. max (t = 2.985 for day 21 and t = 3.838 for day 63). Negative correlations exist between the pH of soils with G. max and soils without G. max for each concentration of crude oil (p = 0.350, 0.083 and 0.683 for 25g, 50g and 75g crude oil concentrations respectively). A perfect positive correlation exists between the pH of the soil with

50g crude oil and the soil with 75g crude oil (p = 0.017) while no correlation exists the soils with 25g crude oil and G. max and 75g crude oil and G. max.

The positive correlation between the pH of the soils and the amount of crude oil added to the soil may be an implication that crude oil pollution leads to increase in soil pH. This is similar to the findings of Andrade et al. (2004) and Ayotamuno et al. (2004) who observed increase in the pH of soils polluted with crude oil. In the opinion of Dibble and Bartha (1979), the higher pH of soils with G. max than in soils without G. max means that higher degradation of crude oil took place in soil with G. max than in soils without G. max. The trend of the pH over the period of studies was against the expectation going by the reports of Ayotamuno et al. (2004) and Merkl et al. (2005c). These researchers reported that the pH of soils decreased as a result of degradation of crude oil. This decrease in the pH of soil with degradation of crude oil could be due to accumulation of organic acids produced during degradation in the soil (Merkl et al., 2005a) or the production of acid radicals through nitrification (Tisdale and Nelson, 1975). However, since soil bacteria thrive better in neutral than in acidic soils (Song et al., 1986; Phung, 1988), the increase of the soil pH towards neutral condition means more favourable conditions for soil bacteria. Many researchers have reported that bacteria play good role in the degradation of crude oil (Atlas and Bartha, 1977; Amund and Igiri, 1990; Frick et al., 1999; Van Hamme et al., 2003). This means that as observed in this study, growth of G. max can enhance the bacteria population in crude oil polluted soil and thereby lead to higher degradation of crude oil in the soil. The continual increase in the soil pH as the period of the study increased means that there was continual increase in favourable conditions soil bacteria and for biodegradation (Dibble and Bartha, 1979).

Table 1: The effect of G. max on the pH of crude oil polluted soil. Values are means ± standard error of three replicates

| Days of sampling | Control | 25g | 25g and G. max | 50g | 50g and G. max | 75g | 75g and G. max |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 21 | 4.73±0.233 | 5.30± 0.115 | 5.37±0.067 | 5.87±0.186c | 5.37±0.067a | 5.50±0.153b | 5.60±0.153a |
| 42 | 5.03±0.176 | 5.30±0.173 | 5.23±0.067 | 5.57±0.145 | 5.37± 0.033 | 5.07±0.120 | 6.23±0.433ǂ |
| 63 | 5.07±0.088 | 5.17±0.067 | 5.33±0.333 | 5.37±0.233 | 5.77±0.176a | 4.97±0.088 | 5.97±0.240aǂ |
| 84 | 5.03±0.067 | 5.13±0.088 | 5.37± 0.067 | 5.37±0.203 | 5.80±0.231 | 4.97±0.133 | 5.97±0.186ǂ |
| 105 | 5.00±0.115 | 5.13±0.033 | 5.37±0.120 | 5.37±0.233 | 5.87±0.233 | 4.97±0.088 | 5.97± 0.203ǂ |

Note: a = significant difference between treatment and control at p<0.05 significant difference between treatment and control, b= significant difference between treatment and control at p<0.01, c = significant difference between treatment control at p<0.001,  * = significant difference between soil with G. max and soil without G. max at p<0.05 ,+ = significant difference between soil G. max and soil without G. max at p<0.01, ǂ= significant difference between soil G. max and soil without G. max at p<0.001

The growth of G. max in soils polluted with 25g crude oil led reduction of the moisture content of the soil. The reverse was the case for the soils with 75 g crude oil. In the case of the soils with 50 g crude oil, the growth of G. max led to reduction of the moisture in the first 42 days and afterwards the growth of G. max enhanced the moisture content of the soil (Table 2). The control has positive correlation with the treatments and there is a positive correlation among the treatments.

Crude oil pollution causes among other things low permeability and low infiltration of water into the soil (Hutchinson et al., 2001; Andrade et al., 2004). These conditions can lead accumulation of water on the soil surface and an artificial drought in the subsurface layer of soil. This can lead to difficulty for the roots to absorb water and nutrients which in the water as the roots usually grow deeper into the soil subsurface layers. The growth of plant root into soil help to create pores in the soil and thereby enhance water penetration and infiltration in soil polluted with crude oil. This increased water penetration and infiltration could be the cause of low moisture contents of soil contaminated with 25g crude oil and that had G. max grown on it as observed in this study. This can help to eliminate water logging of crude oil polluted soil and can lead to increased aeration of the soil. The increased aeration can lead to increase in the activities aerobic microbes in the soil and this can lead to increase in the degradation of oil.

Since the phytotoxic effect of crude oil increases with the concentration of the crude (Cullie and Blanchet, 1958), the higher moisture content of the soil with 75g crude oil and G. max than in the soil with 75g crude oil and no G. max could be due to inhibition of root growth by such amount of crude oil. The inhibition of root growth can lead to low penetration of water and higher accumulation of water on the soil surface. Reduction of transpiration is one of the phytotoxic effects of crude oil (Baker, 1970). The reduction in transpiration also affects the rate at which water absorption and uptake as these are controlled by transpiration pull (Taylor et al., 1997; Kent, 2000). Therefore higher moisture content in the soil with 75g crude oil and G. max than in soil with 75g crude oil and no G. max could be attributed to reduced loss of water due transpiration and subsequent reduction in the rate of water absorption in such soil. A possible cause of the difference between the trend of moisture content in the soil with 25g crude oil and soils with 50g and 75g crude oil is that because better growth of G. max in soil with 25g crude oil led to more absorption of water from the soil than from soils with 50g and 75g crude oil.

Table 2: The effect of G. max on the percentage moisture content of crude oil polluted soil. Values are means ± standard error of three replicates.

| Days of sampling | Control | 25g | 25g and G. max | 50g | 50g and G. max | 75g | 75g and G. max |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 21 | 13.04±0.211 | 13.50±1.381 | 12.24± 0.701 | 9.97± 0.573 | 10.31± 0.693 | 5.20±1.743 | 12.59± 0.763ǂ |
| 42 | 11.06±0.647 | 13.59±0.935 | 12.42± 0.739 | 10.05±0.427 | 11.12±1.832 | 3.34±0.975 | 13.32± 0.978ǂ |
| 63 | 13.16±0.230 | 14.03±0.420 | 13.06± 0.502 | 12.41±0.290 | 15.41±0.188 | 8.19±1.236 | 15.85± 0.593ǂ |
| 84 | 13.43±0.578 | 14.05±0.677 | 13.12± 0.430 | 12.74±0.133 | 15.52±0.133 | 8.16±1.196 | 15.97±0.477ǂ |
| 105 | 15.06±0.920 | 14.07±0.580 | 14.40± 0.534 | 13.66±1.420 | 16.45±0.423 | 8.86±0.700 | 14.84±0.629ǂ |

Note: a = significant difference between treatment and control at p<0.05, b= significant difference between treatment and control at p<0.01, c = significant difference between treatment control at p<0.001, * = significant difference between soil with G. max and soil without G. max at p<0.05,+ = significant difference between soil G. max and soil without G. max at p<0.01, ǂ= significant difference between soil G. max and soil without G. max at p<0.001

The organic matter content of the soil was reduced by the growth of G. max in the first 42 days (Table 3). This might be due to the use of  growth of G. max in the first 42 days might be as a result of the use of the organic matter by the G. max as it grew. Since the plants were in their early growth stages, they could possibly be absorbing nutrients from the soil and returning little or none to the soil. Such could have caused lesser accumulation of organic matter in the vegetated soil than in non-vegetated soil. Ayotamuno et al. (2004) reported similar observation of lower organic matter contents in vegetated soil.

From day 63, the growth of G. max enhanced the accumulation of organic matter in the soils. The observed higher organic matter accumulation in vegetated soil as from day 63 has some interpretations. Firstly, it is possible that G. max started shedding its leaves from after the first 42 days and the decomposition of such leaves increased the organic matter content of the vegetated soil more than that of the non-vegetated soil. The release of organic carbon to the soil due to degradation of crude oil possibly led to accumulation of more organic matter in the vegetated soil than in the non-vegetated soil. This is because organic carbon is a major component of organic matter (Okolo et al., 2005). Also the fixation activities in the root nodules of the plant also had a possible impact on the amount of organic matter accumulated in the vegetated soil.

The organic matter content of the soils has negative correlation (p = -0.237) with the days of sampling and positive correlation (p = 0.767) with the amounts of crude oil added to the soil. This means that while the organic matter contents of soil polluted with crude oil decreases with time, it increases with the quantity of crude oil added to soil. Apart from the 25g treatment, the soil organic matter was significantly affected by the addition of crude oil and growth of G. max at different levels of significance (P<0.001, P<0.01, P<0.05) for the different days of study. The growth of G. max however did not produce any significant effect on the organic matter content within each concentration of crude oil. There was negative correlation between the organic matter content of the control and the treatments. The 25g and 50g treatments have a perfect correlation (± 1) and same applies to the 25g and G. max and 50g and G. max treatments.

Table 3: The effect of G. max on the percentage organic matter content of crude oil polluted soil. Values are means ± standard error of three replicates.

| Days of sampling | Control | 25g | 25g and G. max | 50g | 50g and G. max | 75g | 75g and G. max |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 21 | 0.89±0.118 | 1.95± 0.529 | 1.55±0.041 | 2.47±0.176 | 2.14 ±0.284 | 3.16±0.180 | 2.54± 0.170ǂ |
| 42 | 1.29±0.651 | 1.53± 0.073 | 1.15±0.367 | 2.22±0.115 | 1.65± 0.103 | 2.70±0.306c | 2.21± 0.111ǂ |
| 63 | 0.90±0.096 | 0.96± 0.060 | 1.14±0.042 | 1.67±0.111 | 1.63±0 .071 | 2.05±0.140c | 1.97± 0.119ǂ |
| 84 | 0.91± 0.096 | 0.99± 0.018 | 1.29±0.168 | 1.70±0.124 | 1.66 ±0.100 | 1.87±0.204c | 1.98± 0.127ǂ |
| 105 | 0.91± 0.142 | 1.35±0.066 | 1.33±0.123 | 1.88±0.140 | 1.78 ±0.061 | 1.91±0.228c | 1.95± 0.030ǂ |

Note: a = significant difference between treatment and control at p<0.05, b= significant difference between treatment and control at p<0.01, c = significant difference between treatment control at p<0.001,  * = significant difference between soil with G. max and soil without G. max at p<0.05 ,+ = significant difference between soil G. max and soil without G. max at p<0.01, ǂ= significant difference between soil G. max and soil without G. max at p<0.001.

The effect of crude oil the pH, moisture and organic matter content of soil observed in this study conforms with the reports of Njoku et al., (2008c) that these change with addition of crude oil to soil. Soil pH, soil moisture and soil organic matter contents have influence on the soil properties. The organic matter content of soil improves the binding processes in the soil. Such binding reduces water drainage and improves water retention ability of soil. Therefore the low organic matter in soil with 25g crude oil could be a cause of the low water accumulation in that soil. Excess binding of the soil particles together reduces root penetration and inhibit the absorption of materials. This can lead to malnourishment of plants even in the presence of abundant nutrients.

The amount of crude oil lost from the soil contaminated with 25g crude oil was enhanced by the growth of G. max. However in soils with 50g and 75g crude oil, more crude oil was lost from soils without G. max than in soils with G. max (figure 1). It is however worthy to note that in this study weeds were observed to have sprouted out from the contaminated soils with G. max and none of such was observed in the non-vegetated soil. This shows that even though the growth of G. max did not produce any significant effect on the percentage of crude oil lost from the soils the plant can reduce the quantity and toxicity of crude oil in soils. This is shown by the lesser amount of crude oil left in soil with 25g crude and G. max than in soil with same amount of crude oil and no G. max and the sprouting of weeds from the soils with 50g and 75g crude oil and G. max. The sprouting of the weeds indicates that the toxicity of crude oil in the vegetated soils reduced to the extent of allowing for the growth of weeds in such soils. This confirms the findings of Siciliano and Germida (1998a) that plants may not reduce the concentration of contaminants and yet can reduce the toxicity of such contaminants. For example, Siciliano and Gemida (1998a), observed a reduced toxicity of 2,3-dichlorobenzoic acid and 3-chlorobenzoic acid without reduction in the contaminant concentration in vegetated soil. The reduction is also a mechanism of phyoteremediation going by the definition of phytoremediation as a technique of rendering harmful materials harmless using plants and their associated microbes (Cunningham et al. 1996; Pivetz, 2001). Conversely, the absence of such weeds from soils without G. max indicates that the soil has not reach the level that will enable plants to grow.

The effect of G. max on the removal of crude oil from the soil polluted with 25g crude oil is similar to the findings of Aprill and Sims (1990), Lee and Bank (1993), Schwab and Banks, (1994) and Merkl et al. (2005b) who reported higher degradation of petroleum hydrocarbon in vegetated soils than in non-vegetated soil. The higher removal of crude oil observed in this study conforms with the reports of Frick et al. (1999) who listed G. max as one of the plants that can remediate petroleum hydrocarbon (anthracene) polluted soil. It also conforms with the suggestions of Njoku et al. (2008b) who suggested that G. max can be tried for its efficacy to remediate crude oil polluted soil. The removal of crude oil by G. max possibly occurred through one of the several mechanisms of phytoremediation. Such mechanisms include polymerization of the contaminants (Adler et al., 2004), interaction of the plant with fungi and bacteria (Siciliano and Germida, 1998) and production of root exudates and plant materials which serve as source of carbon, nitrogen and phosphorus for petroleum degrading microbes (Horvath, 1972; Rajaram and Sethunathan, 1975; Alexander, 1977; Smith, 1990; Burken and Schnoor, 1996). Nitrogen fixed in the soil by legumes reduces plant/microbes competition for nitrogen and thereby increase plant growth exudates production. This increases the ability of plants to increase the degradation of pollutants.

Figure 1: The effect of G. max on the removal of crude oil from polluted soil. Values are means ± standard error of three replicates.

Conclusion

The findings of this study indicate that growth of G. max in crude oil contaminated soils affects the physico-chemistry of the soil enhancing the degradation of crude oil. For instance, the significant effect that the growth of G. max produced on the pH and moisture content of the soil with 75g crude oil indicates that G. max affects the physico-chemistry of crude oil contaminated soil. It can also be inferred from the findings of this study that the growth of G. max in crude oil contaminated soils reduces the toxicity of crude oil in the soil. This is going by the sprouting of weeds in the soils with G. max and none of such soils without G. max. We suggest that to soil augments like cow dung should be added to crude oil contaminated soil to enhance the increase the efficacy of using G. max in remediating crude oil contaminated soils as Njoku et al. (2008a) have reported that addition of cow dung to crude oil contaminated soils enhances the growth of G. max in such soil.

Correspondence to:

Kelechi Njoku

Department of Cell Biology and Genetics

University of Lagos, Akoka Lagos Nigeria

Tel. phone: +2348033842956

Email: njokukelelechi@yahoo.com

References

Adler, P.R.,  Arora, R., El Ghaouth, Glenn, D.M. and Sola, J.M. 1994. Bioremediation of Phenolic Compounds from water with plant root surface peroxidases. J. Environ, Qual. 23, 1113-1117

Alexander, M. 1977 Introduction to Soil Microbiology, New York, John Wiley and Sons

Amund, O.O. and Igiri, C.O. 1990 Biodegradation of petroleum hydrocarbons under tropical estuarine conditions. World J. Microbiol. Biotechnol. 6, 255-262

Andrade, M.L., Covelo, E.F., Vega, F.A. and Marcet, P. 2004. Effect of the Prestige Oil spill on salt marsh soils on the coast of Galicia (Northwestern Spain). Journ. Environ. Qual. 33, 2103 – 2110

Aprill, W. and Sims, R.C. 1990. Evaluation of the Use of Prairie Grasses for Stimulating Polycyclic Aromatic Hydrocarbon Treatment in Soil. Chemosphere 20, 253-265

Atlas, R. M. and Bartha, R. 1977. Stimulated Petroleum biodegradation. Crit. Rev. Microbiol. 5, 371-386

Ayotamuno, M.J., Kogbara, R. B., Ogaji, S.O.T. and Probert, S.D. 2004. Bioremediation of a crude oil polluted agricultural soil at Port Harcourt, Nigeria. http://dspace.lib.cranfield.ac.uk/bitstream/1826/1189/1/Bio-crud± oil± Nigeria-Applied± Ecology.pdf

Baker, J. M. 1970. The effects of oils on plants. Environ. Pollut. 1, 27-44

Banks, M.K., Govindaraju, R.S., Schwab, A.P. and Kulakow, P. 2000. Part 1: Field Demonstration. In: Phytoremediation Of Hydrocarbon- Contaminated Soil, pp 3-88. (Fiorenza, S., Oubre, C.L.   and Ward,  C.H. Eds).  Baton Rouge, LA, Lewis Publishers,

Burken, J.G and Schnoor, J.L. 1996. Phytoremediation: Plant Uptake of Atrazine and Role of Root Exudates. Journ. Environ. Eng. 122, 958-963

Cullie, J. and Blanchet, B. 1958. Low-volume spraying of topical fruit: Oil base spray products with special reference to their phyotoxicity. Fruits. 13, 53-65

Cunningham, S. D., Anderson, T.A., Schwab, A.P. and Hsu, F.C. 1996. Phytoremediation of oil contaminated with organic pollutants. Adv. Agron.. 56,  55-114

Dabbs, W.C., 1996. Oil Production and Environmental Damage.

http://www.american.edu.TED/hpl.htm

Dibble, J. T. and Bartha, R. 1979. Rehabilitation of oil-Inundated Agricultural Land: A Case History. Soil Sci. 128 (1), 56-60

Eckert, D. and Sims, J. T. 1995. Recommended soil pH and Lime Requirement tests.

http://ag.udel.edu/extension/information/prod_agric/chap3-95.htm

Frick, C. M.,  Farrell, R. E. and Germida, J. J. 1999. Assessment of Phytoremediation as an in situ technique for cleaning oil-contaminated sites. Petroleum Technology Alliance Canada, Calgary. http://www.rtdf.org/pub/phyto/phylinks.htm

Gallego Martinez, M. G., Gomez Santos, A.G., Gonzalez Cruz, L. G., Motes de Oca Garcia, M. A., Yanez Trujillo, L. Y., Zermeno Eguia Liz, J. A., and Gutierrez-Rojas, M. 2000. Diagnostics and resulting approaches to restore petroleum-contaminated soil in a Mexican tropical swamp. Water Sci. Technol. 42, 377-384

Horvath, R.S. 1972. Microbial Cometabolism and the Degradation of Organic Compounds in Nature. Bacteriol. Rev. 36,146-155

Hutchinson, S.L., Banks, M.K. and Schwab, A.P 2001. Phytoremediation of aged petroleum sludge: Effect of irrigation techniques and scheduling. Journ. Environ. Qual. 30, 1516-1522

Inoni, O.E., Omotor, D.G. and Adun, F.N. (2006) The effect of oil spillage on crop yield and farm income in Delta State, Nigeria. Journ. Central Eur. Agri., 7(1), 41-49

Kent, M. 2000. Advanced Biology, Oxford University Press, Oxford, New York, pp 276-277

Lee, E. And Banks, M.K. 1993. Bioremediation of Petroleum Contaminated Soil Using Vegetation: A Microbial Study. Journ. Environ. Sci. Hlt A28, 2187-2198

Lundstedt, S. 2003. Analysis of PAHs and their transformation products in contaminated soil and remedial processes. Solfjodern Offset AB, Umea, Sweden, 55pp

Merkl, N. 2005 Phytoremediation of petroleum-contaminated soil  Margraf Publisher

Weikershim, 125pp

Merkl, N., Schutze-Kraft,  R. and Infante, C.   2005a. Assessment of tropical grasses and legumes for phytoremediation of petroleum contaminated soils. Water, Air and Soil Pollut. 165 (1-4), 195-209

Merkl, N., Schutze-Kraft,  R. and Infante, C  2005b. Phytoremediation in the tropics – influence of heavy crude oil on root morphology characteristics of graminoids. Environ. Pollut. 138 (1),  86-91

Merkl, N., Schutze-Kraft, R., and Arias, M. 2005c. Influence of fertilizer level on phytoremediation of crude oil-contaminated soils with the tropical grass Brachiaria brizantha (Hochst. ex A. Rich.) Stapf. In: Phytoremediation of petroleum-contaminated soil, pp 71-83.  (Merkl, N., Ed). Weikershim,  Margraf Publisher

Njoku, K.L., Akinola, M.O. and Oboh, B.O. 2008a. Growth and performance of Glycine max L. (Merrill) in crude oil contaminated soil augmented with cow dung. Nat. Sci. 6(1), 48-58.

Njoku, K.L., Akinola, M.O. and Oboh, B.O. 2008b. Germination, survival and growth of accessions of Glycine max  L. (Merrill) (Soybean) and  Lycopersicon esculentum L. (Tomato) in crude oil polluted soil. R. Journ. Environ. Toxicol. 2 (2): 77-84.

Njoku, K.L., Akinola, M.O. and Oboh, B.O. 2008c. Does crude oil affect pH, moisture and organic matter content of soils? Ecol. Environ. Conser, 14 (4): 731-736

Odokuma, L. O. and Ibor, M. N. 2002. Nitrogen fixing bacteria enhanced bioremediation of crude oil polluted soil. Global Journ. Pure Appl. Sci., 8 (4), 455-468

Okoh, A.I. 2003. Biodegradation of Bonny light crude oil in soil microcosm by some bacteria strains isolated from crude oil flow stations saver pits in Nigeria. Afr. Journ. Biotechn. 2 (5), 104-108

Okolo, J.C., Amadi, E.N. and Odu, C.T.I 2005. Effects of soil treatments containing poultry manure on crude oil degradation in sandy loam soil. Appl. Ecol. Environ. Res., 3 (1), 47-53

Okpokwasili, G.C. and Odokuma L.O. 1990. Effect of salinity on Biodegradation of oil spills dispersants. Waste Management, 10: 141 – 146

Pivetz,.B.E. 2001. Phytoremediation of Contaminated Soil and Ground water at Hazardous Waste Sites. Man Tech Environmental Resources Services Corporation, Ada, Ok, 36pp

Phung, T. 1988. Land treatment of hazardous wastes. In: Standard Handbook of Harzadous Waste Treatment and Disposal pp 941-951. (Freeman, H.M. Ed). New York,  McGraw-Hill.

Rajaram, K.P. and Sethunathan, N. (1975) Effect of organic sources on the degradation of parathion organic  sources in flooded alluvial soil. Soil Sci. 119, 296-300

Saupe,  S. G.(2004) Plant Physiology Biology 327.

http://employees.esbsja.edu/ssaupe/biol327/Lab/gilson-lab.htm

Schneekloth , J., Bauder, T., Broner and Wakson, R. 2002. Measurement of soil moisture.

http://www.etx.colostate.edu/drought/soilmoisture.htm

Schnoor, J.L. 2002. Phytoremediation of Soil and Groundwater: Technology Evaluation Report TE-02-01. Groundwater Remediation Technologies Analysis Centre (GWRTAC). www.gwrtac.org

Schroder, P., Harvey, P.J. and Schwitzguebel, J.P. 2002. Prospects for the phytoremediation of organic pollutants in Europe. Environ. Sci. Pollut. Res. 9(1), 1-3

Schwab, A.P. and Banks, M.K. 1994. Biologically Mediated Dissipation of Polyaromatic Hydrocarbons in the Root Zone. In: Bioremediation Through Rhizosphere Technology, pp 132 -141. American Chemical Society, Washington DC,

Schwab, A.P. and Bank, M.K. 1999. Phytoremediation of petreoleum contaminated soils. In: Bioremediation of contaminated soils, pp 783-795. (Andriano, D.C., Bollag, J.M., Frankenberger, W.T. Jr, and Sims, R.C. Eds). American Society of Agronomy, Crop Science Society of America, Soil Science Society of America, Madison

Siciliano, S. D. and Germida, J.J. 1998a. Biolog analysis and fatty acid methyl ester profiles indicate that Pseudomonas inoculants that promote phytoremediation alter te root-associated microbial community of Bromus bieberstenii. Soil Biol. Biochem. 30, 1717-1723

Siciliano, S.D. and Germida, J.J. 1998b. Mechanism of Phytoemediation: Biochemical and Ecological Interaction Between Plants and Bacteria. Environ. Rev. 6, 65-79

Smith, M.R. 1990. The biodegradation of aromatic hydrocarbon by bacteria. Biodegrad. 1, 161-188

Song, H.-G., Pedersen, T.A. and Bartha, R. 1986. Hydrocarbon Mineralization in Soil: Relative Bacterial and Fungal Contribution. Soil Biol. Biochem. 18, 109-111

Taylor, D. J., Green, N.P.O. and Stout G.W. 1997. Biological Science, 3rd Edn. UK, Cambridge University Press, pp 447-458

Tisdale, S. and Nelson, W. 1975. Soil fertility and fertilizer, 3rd ed., Macmillian Pub. Co. Inc., New York, USA

Van der Lelie, D, Schwitzguebel, J., Glass, D.J., Vangronsveld, J., and Baker,  A. 2001. Assessing phytoremediation’s progress. Environ. Sci. Techn. 35 (21), 446A-452A

Van Hamme, J.D., Singh, A. and Ward, O.P. 2003. Recent advances in Petroleum Microbiology. Microbiol. Molec. Biol. Rev., 63 (4), 503-549

Date of submission: 29/09/2009