evaluation of Phytotoxicity of compost DURING COMPOSTING PROCESS

1 Sh. M. Selim.; 2 Mona S. Zayed and *3 Houssam M. Atta

1. Unit of Biofertilizers, Dept. of Microbiology, Fac. Agric., Ain Shams Univ. The present address: Biotechnology Department, Faculty of Science and Education- Al-Khurmah, Taif University; KSA. 2. Unit of Biofertilizers, Dept. of. Microbiology, Fac. Agric., Ain Shams Univ. *3. Botany and Microbiology Department, Faculty of Science (Boys), Al-Azhar University, Cairo, Egypt. The present address: Biotechnology Department, Faculty of Science and Education- Al-Khurmah, Taif University; KSA.

houssamatta@yahoo.com and houssamatta@hotmail.com; Tel: 00966506917966

Abstract: The phytotoxicity of compost as germination index (GI) was evaluated during composting period. This study suggested that NH4+-N and heavy metals in the organic wastes were major compounds inhibiting seed germination and root elongation. The increase in GI of cress plant during composting coincided with the decrease in NH4+-N, and water extractable Zn and Cu content to very low levels, and with the increase in NO3--N and water extractable P and K content. Negative correlation was observed between GI of cress with NH4+-N, Cu, and Zn content, while positive correlation was observed with NO3-_N, P and K content. NH4+/NO3- ratio has been used as an index of compost maturity where values were decreased as composting progresses.

[Sh. M. Selim.; Mona S. Zayed and Houssam M. Atta]. Evaluation of Phytotoxicity of Compost During Composting Process. Nature and Science 2012;10(2):69-77]. (ISSN: 1545-0740). http://www.sciencepub.net. 12

Keywords: phytotoxicity, Composting process, germination index

1. Introduction

Phytotoxicity is one of the most important criteria for evaluating the suitability of compost for agricultural purposes and to avoid environmental risks before these composts can be recycled back to agricultural land. [Tiquia et al., 1996; Brewer & Sullivan, 2003 and Cooperband et al., 2003]. Pervious research work has demonstrated that application of immature compost onto the soil causes negative effects on seed germination, plant growth and development. These effects occur because an immature compost induces high microbial activity (which reduce oxygen concentration in the soil), blocks the existing soil available nitrogen [Zucconi et al., 1981a]. Immature compost also introduce phytotoxic compounds such as heavy metals [Tam and Tiquia, 1994]. Phenolic compounds [Wong, 1985], ethylene and ammonia [Tam and Tiquia, 1994], excess accumulation of salts [Tam and Tiquia, 1994], and organic acids [Manios et al., 1989] which could retard seed germination and plant growth.

Acetic acid is probably the most damaging organic acid released from immature compost, but there are also other compounds that contribute to the phytotoxic effect [Ozores-Hampton, 1998].

In fact, application on soil of no stabilized organic materials could affect both crops and the environment because of the presence of phytotoxic compounds [Butler et al., 2001]. High concentrations of salt and the release of organic acids into the composts are also correlated to inhibition of germination and growth. Phytotoxicity is often best evaluated by conducting germination or growth tests [Gariglio et al., 2002 and Brewer and Sullivan, 2003], but the test plants have to be chosen with care [Emino and Warman, 2004].

Germination Index (GI) is the best way to test the phytotoxicity of compost to plant growth because the results of it are quite straightforward and reliable. Germination bioassays are widely used to test for salinity, soil pathogens, toxic substances and some other physical and chemical properties of compost [Zucconi et al., 1985 and Gajdos 1997], which could be the major potential reasons of phytotoxicity. Several investigators reported that phytotoxic compounds are gradually eliminated during the composting process, which could explain the GI increases with composting time.

The Germination index (GI), which combines measures of relative seed germination (G%) and relative root elongation (L%), has been used to evaluate the toxicity of compost. [Tam and Tiquia, 1994, Tiquia et al., 1996 and Wong et al., 2001]. It has been noted that a GI value of 80% indicated the disappearance of phytotoxins in composts [Zucconi et al., 1981b]. [Tiquia et al., 1996] used this value not only as indication of the disappearance of phytotoxicity but also as an indication of the maturity of compost.

The germination index is a maturity test based on seed germination and initial plant growth using a liquid extract from the compost [Zucconi, et al., 1981b]. It reflects the phytotoxicity of the compost extracts at different stages of composting. The compost is considered mature when the germination index is higher than 60 %, compared to the control with distilled water [Zucconi and De Bertoldi, 1987].

The most popular germination test used by researchers is from cress test [Zucconi et al., 1981a and Erhart and Burian, 1997]. In their opinion, the compost is non-toxic when the germination rate is more than 85% or the plant seedling weights are more than 90%. Beside, composting scientists found that GI at each composting time did not show any significant changes with the dilutions of the extract, or even when the extract was diluted up to 75% with distilled water. They conducted that ammonia and low molecular weight organic acids were two phytotoxic substances proposed and plant growth increased as these disappeared. [Wei et al., 2000] reported that when the GI is more than 80 % Compost is considered mature and practically free of phytotoxic substances.

An increased GI is indicative of decreased phytotoxicity and thus of a more mature product [Tiquia et al., 1996; Bernal et al., 1998; Tiquia and Tam, 1998 and Wong et al., 2001]. However, results obtained using the GI index should be interpreted with care because they are affected by the type of seed used and the source of compost [Bernal et al., 1998; Tiquia and Tam, 1998; and Brewer and Sullivan, 2003]. Application of undecomposed wastes or non-stabilized compost to land may lead to immobilization of plant nutrients and cause phytotoxicity [Butler et al., 2001 and Cambardella et al., 2003].

In fact, germination index is the most sensitive parameter used to evaluate toxicity of compost to seedlings, and to test if compost is mature [Zucconi et al., 1981a; Wong et al., 2001 and Meunchang et al., 2005].

Tang et al. (2006) reported that extraction rate is a very important factor that influences GI. They conducted that a 10:1 extraction rate is suitable for evaluating the GI change during the maturing of cattle manure compost. They also declared that different extraction rates gave different changing patterns of GI during the maturing process.

2. MATERIAL AND METHODS

2.1. Agricultural wastes:

Agricultural residues (Corn stalks, cotton stalks and bagasse) were collected from, Qalubia Governorate, to be used for compost production. The used materials along with their chemical analyses are presented in Table (1).

2.2. Organic accelerators:

Cattle manure and poultry manure were collected from Qalubia Governorate. Mature compost was provided from the Egyptian Company for Agricultural Residues Utilization (ECARU). Cairo, and used as an additional source of microorganisms.

2.3. Cultivar:

Cress seeds (lepidum sativum) were provided from the Crops Research Institute, Agricultural Research Center, Giza, were used in this investigation.

2.4. Compost preparation:

Six heaps, 100 Kg dried weight each, were prepared from air dried and shredded wastes (corn stalks, cotton stalks and bagasse) using either of six different treatments including: cattle manure (Heap1) and treated with 1% mature compost (Heap 2), poultry manure (Heap3) and treated with 1% mature compost (Heap4), sewage sludge (Heap5) and treated with 1 % mature compost (Heap6). The weights of the raw materials used for particular compost were calculated according to [Christian et al. 1997] that give a C/N ratio 30:1, the moisture content of the composting mass was kept at 60%. The heaps were covered with black plastic sheets to avoid heat and water loss. The heaps were turned and mixed every 5 days to provide aeration. Moisture content was adjusted at each turning time and maintained by addition of water.

2.5. Compost water extracts:

A water extract of each compost was prepared by shaking the samples with distilled water at 1:10 w/v ratio for 1 hour, and then filtered [Zucconi et al., 1981b].

2.6. Phytotoxicity evaluation:

The phytotoxicity of compost extracts was evaluted by the seed germination technique [Zucconi et al. 1981b, Tam and Tiquia, 1994 and Tiquia et al., 1996].  Cress seeds (lepidum sativum) were surface sterilized by immersion in 75% alcohol for three minutes followed by transferring in 0.001 HgCl2 solution for two minutes with periodical agitation and finally thoroughly washed with sterilized distilled water to get rid of toxic chemicals [Rovira, 1956]. 10 ml of water compost extract was applied to filter paper in a Petri dish and 20 seeds were then placed on the filter paper. All experiments were run in triplicate. The Petri dishes were sealed with tape to minimize water loss while allowing air penetration and then were incubated in the dark for 72 hours at room temperature, the seed germination percentage and root length of the plants in the extracts were determined. The seed germination in distilled water was used as control. The percentage of seed germination, root elongation and germination index (GI) was calculated according to [Zucconi et al., 1981b] as follows:

Seed germination (%)

Root elongation (%)

Germination Index

The germination index was able to account for both low toxicity, which affects root growth and high toxicity, which affects germination. Coefficient of correlation (R2) was also made to determine the possible relationship between bioassay results and the chemical properties of compost as follows:

2.7. Chemical determinations:

Nitrate-N and ammonical-N were determined according to the method described by [Page et al., 1982]. Available phosphorus was extracted with 0.5M NaHCO3 and determined calorimetrically in sulphomolybdic acid system as described by [Jackson, 1973]. Available potassium was extracted in boiling HNO3 acid solution and flame-photometrically determined according to [Page et al., 1982]. Water-extractable 1:10 (W/V) Cu and Zn were determined by atomic absorption spectrometry method [page et al., 1982]. Statistical analysis of the obtained data was carried out according to [Snedecor and Cochran, 1967].

## 3. RESULTS AND DISCUSSION

The responses of cress plant (lepidum sativum) to the toxicity of the compost water extracts during the composting period in term of the relative seed germination and relative root elongation percentages were illustrated in Figures ( 1 and  2).

At the beginning of composting, the GI of cress plants was very low with initial values of 18.76 – 20.53 % compared to the control using distilled water, probably due to the phytotoxic effects of ammonia and low molecular weight organic acids [Zucconi et al., 1981b and Wong et al., 2001], but as composting proceeded, the GI value increased. At day 80 in this study the GI values were higher for inoculated heaps than uninoculated heaps. However, after composting proceeded, the GI value increased to greater than 80% (81.88 – 97.46) revealing that the phytotoxicity in these heaps was eliminated after composting. The elimination of phytotoxicity has also been widely used as a measure of compost maturity [Wei et al., 2000; Wu and Ma, 2001; Butler et al., 2001 Cambaradella et al., 2003 and Menunchang et al. 2005].

Data in Table (2) clearly show that ammonium nitrogen NH4+-N levels increased rapidly in the first few days due to the microbial degradation of nitrogen containing compounds (ammonification process) which converted a fraction of the organic N to NH3 and NH4 + ions. Then the NH4+-N content of all heaps decreased gradually. The inoculated heaps had lower NH4+-N concentrations than uninoculated heaps by the end of composting process, indicating a much more dynamic N transformation during composting. It has been noted that the absence or decrease in NH4+-N is an indication both for good composting and maturation process [Riffaldi et al., 1986 and Tiquia et al. 1996]. Decreases in NH4+-N content in all composts were observed which was in parallel with the increases in the content of NO3--N during the composting process. The NO3--N contents of all heaps increased dramatically during composting time. The decreasing in NH4+-N and increasing in NO3--N contents were attributed due to the rapid conversion of NH4+ to NO3- through nitrification process by the microorganisms during composting [Tiquia et al., 1996 and Tiquia, 2003].

NH4+ / NO3- ratio has been used as an index of compost maturity where values decreasing as composting progresses due to nitrification which converts NH4+ to NO3- [Forster et al., 1993 and  Bernal et al., 1998].When the NH4+ concentration decreases and NO3- appeared in the compost material it is considered ready to be as mature  compost [Finstein and Miller, 1985]. [Bernal et al., 1998] showed that a high NH4+ / NO3- ratio founded out that there is unstabilized materials during the process.

They reported NH4+ / NO3- ratio values <1 at the end of active composting. It is clear from data presented in Table  (3), that  the initial  ratio of NH4+ / NO3-  ranged from  6.10 to 29.78, and decreased to 0.05 and 0.43 at the end of composting period. It was conducted by [Bernal et al., 1998 and Meunchang et al., 2005] that mature compost is expected to have reduced ratio of NH4+ / NO3- .

During the initial stage of composting, the water extractable P and K of compost heaps ranged from 3.96 to 6.28 mg/g and 5.59 to 9.30 mg/g, respectively. At mature stage, these levels increased to 10.62 and 17.75 mg/g for extractable P and 15.29 and 29.36 mg/g for extractable K (Table, 4). [Georgacakis et al., 1996] also founded an increase in extractable phosphorus in the final compost product, which support the results found in this study.

As reported in Table (5) the initial concentrations of water-extractable Cu and Zn of all heaps ranged from 6.20 to 28.68 ppm and 21.47 to 69.22 ppm respectively. At mature stage these levels both decreased to between 1.64 to 5.90 ppm for water extractable Cu and 6.01 to 12.22 ppm for water extractable Zn. As composting progressed, the water-extractable Cu and Zn decreased. However, in the literature these parameters have never been considered as indices of compost maturity. Rather,  they are often used to assess the quality of compost in terms of potential environmental contamination [Tiquia and Tam, 1998].

The content of the water extractable form of the heavy metals is more important than total metal concentration [Tiquia et al., 1996]. The water-extractable Cu and Zn concentrations decreased during composting because they bind with the humic substances [Ciavatta et al., 1993].  The decline in water extractable Cu and Zn was due to the formation of complexes of these metals with chelating organic compound formed (a metal-humus complex), thus making them not water-extractable and biologically unavailable [Leita et al., 1999].

As shown in Figure (3) the germination index of cress showed a significant negative correlation with NH4+-N content (R2= 0.95). A significant positive correlation was observed between GI of cress with NO3--N content (R2= 0.85). Negative correlations were also observed between GI of cress with water extractable Zn and Cu as shown in Figure (4), R2= 0.70 and 0.58 for Zn and Cu respectively. Significant positive correlation were observed between GI of cress with water extractable P (R2=0.75) and K (R2=0.86) as shown in Figure (5). This study suggested that NH4+-N and heavy metals in the organic wastes were major compounds inhibiting seed germination and root elongation. The increase in GI of cress plant during composting coincided with the decrease in NH4+-N, and water extractable Zn and Cu content to very low levels, and with the increase in NO3--N and water extractable P and K content. The decrease in NH4+-N content to low levels was associated with the accumulation of NO3--N via the nitrification process.

This result is in agreement with that obtained by [Tiquia et al., 1996], which reported that NH4+-N was the most important factor inhibiting seed germination and root elongation of all species examined. [Meggie at al., 1967 and Wong et al., 1983] reported that ammonia generated from decomposing organic compounds and animal manure is detrimental to seed germination and seedling growth.

4. CONCLUSION

The sum of the above results showed that, un-mature compost (high levels of NH4+ to NO3- and heavy metals) inhibiting seed germination and root elongation..

Table 1. Chemical characters of organic wastes used for composting.

| Character | Corn | Cotton | Bagasse | Cattle | Poultry | Mature |
| --- | --- | --- | --- | --- | --- | --- |
|  | stalks | stalks |  | manure | manure | compost |
| Total Carbon % | 50.59 | 54.00 | 56.47 | 41.96 | 29.00 | 40.02 |
| Total Nitrogen % | 0.61 | 0.70 | 0.42 | 1.88 | 3.90 | 2.03 |
| C:N Ratio | 83.63 | 77.15 | 134.45 | 22.32 | 7.44 | 19.68 |

Table 2 . Changes in  N-NH4+ and N-NO3- concentration during the composting process of different heaps

| Time (days) |  | Heap 1 | Heap 2 | Heap 3 | Heap 4 | Heap 5 | Heap 6 | LSD 0.05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N-NH4+ (mg/Kg) |  |  |  |  |  |  |  |  |
| 0 | 382 |  | 377 | 581 | 573 | 167 | 165 | 149 |
| 5 | 535 |  | 565 | 814 | 860 | 225 | 247 | 221 |
| 10 | 508 |  | 537 | 773 | 817 | 214 | 235 | 210 |
| 20 | 325 |  | 344 | 495 | 523 | 137 | 150 | 134 |
| 30 | 260 |  | 275 | 396 | 418 | 109 | 120 | 107 |
| 40 | 208 |  | 192 | 317 | 293 | 88 | 84 | 80 |
| 60 | 133 |  | 123 | 203 | 187 | 56 | 54 | 51 |
| 80 | 85 |  | 60 | 130 | 92 | 31 | 26 | 30 |
| 100 | 55 |  | 39 | 83 | 59 | 20 | 17 | 19 |
| 120 | 44 |  | 31 | 66 | 47 | 16 | 14 | 15 |
| LSD 0.05 | 170 |  | 188 | 259 | 286 | 73 | 82 |  |
| N-NO3- (mg/Kg) |  |  |  |  |  |  |  |  |
| 0 | 41 |  | 43 | 20 | 22 | 24 | 27 | 7 |
| 5 | 82 |  | 86 | 39 | 44 | 49 | 54 | 15 |
| 10 | 73 |  | 78 | 35 | 40 | 44 | 49 | 13 |
| 20 | 88 |  | 93 | 42 | 48 | 53 | 58 | 16 |
| 30 | 106 |  | 112 | 51 | 57 | 63 | 70 | 19 |
| 40 | 127 |  | 134 | 61 | 74 | 76 | 91 | 23 |
| 60 | 183 |  | 193 | 87 | 125 | 110 | 142 | 33 |
| 80 | 241 |  | 255 | 115 | 165 | 145 | 204 | 46 |
| 100 | 292 |  | 308 | 140 | 218 | 175 | 247 | 55 |
| 120 | 322 |  | 340 | 154 | 240 | 193 | 273 | 61 |
| LSD 0.05 | 96 |  | 101 | 46 | 78 | 57 | 87 |  |

Table 3. Changes in NH4+ / NO3- ratio during the composting process of different heaps.

| Time (days) | Heap 1 | Heap 2 | Heap3 | Heap4 | Heap 5 |  | Heap 6 | LSD 0.05 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NH4+ / NO3- ratio |  |  |  |  |  |  |  |  |  |
| 0.00 | 9.36 | 8.74 | 29.78 | 25.93 |  | 6.82 | 6.10 |  | 8.12 |
| 5.00 | 6.55 | 6.55 | 20.85 | 19.45 |  | 4.60 | 4.57 |  | 5.87 |
| 10.00 | 6.92 | 6.92 | 22.00 | 20.53 |  | 4.86 | 4.83 |  | 6.19 |
| 20.00 | 3.69 | 3.69 | 11.74 | 10.95 |  | 2.59 | 2.58 |  | 3.28 |
| 30.00 | 2.46 | 2.46 | 7.82 | 7.30 |  | 1.73 | 1.72 |  | 2.18 |
| 40.00 | 1.64 | 1.43 | 5.22 | 3.93 |  | 1.15 | 0.92 |  | 1.37 |
| 60.00 | 0.73 | 0.64 | 2.32 | 1.50 |  | 0.51 | 0.38 |  | 0.59 |
| 80.00 | 0.35 | 0.24 | 1.12 | 0.56 |  | 0.22 | 0.13 |  | 0.28 |
| 100.00 | 0.19 | 0.13 | 0.59 | 0.27 |  | 0.11 | 0.07 |  | 0.15 |
| 120.00 | 0.14 | 0.09 | 0.43 | 0.20 |  | 0.08 | 0.05 |  | 0.11 |
| LSD 0.05 | 3.04 | 2.98 | 9.66 | 8.93 |  | 2.18 | 2.10 |  |  |

Table 4. Changes in water extractable P and K concentrations during the composting process in different heaps.

| Time (days) | Heap1 | Heap 2 | Heap 3 | Heap 4 | Heap 5 | Heap 6 | LSD 0.05 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extractaple P (mg/g) |  |  |  |  |  |  |  |  |
| 0 | 4.80 | 4.77 | 3.97 | 3.96 | 6.28 | 6.23 | 0.84 |  |
| 5 | 4.99 | 5.20 | 4.09 | 4.31 | 6.53 | 6.79 | 0.90 |  |
| 10 | 5.67 | 6.13 | 4.60 | 5.13 | 7.35 | 7.93 | 1.04 |  |
| 20 | 7.31 | 8.50 | 5.94 | 7.14 | 9.31 | 10.85 | 1.46 |  |
| 30 | 8.33 | 9.77 | 6.77 | 8.21 | 10.66 | 12.48 | 1.70 |  |
| 40 | 9.33 | 10.65 | 7.59 | 8.94 | 11.99 | 13.66 | 1.84 |  |
| 60 | 10.69 | 11.74 | 8.69 | 10.45 | 13.79 | 15.11 | 1.94 |  |
| 80 | 11.93 | 12.72 | 9.13 | 10.89 | 14.63 | 15.58 | 2.05 |  |
| 100 | 12.88 | 13.59 | 9.91 | 11.66 | 16.02 | 16.70 | 2.11 |  |
| 120 | 14.46 | 15.17 | 10.62 | 13.00 | 17.16 | 17.75 | 2.19 |  |
| LSD 0.05 | 3.19 | 3.34 | 2.30 | 2.89 | 3.75 | 3.87 |  |  |
| Extractaple K (mg/g) |  |  |  |  |  |  |  |  |
| 0 | 9.30 | 9.24 | 7.75 | 7.71 | 5.59 | 5.59 | 1.27 |  |
| 5 | 9.67 | 10.07 | 7.98 | 8.40 | 5.82 | 6.09 | 1.37 |  |
| 10 | 10.99 | 11.86 | 8.98 | 9.99 | 6.55 | 7.11 | 1.66 |  |
| 20 | 14.17 | 16.45 | 11.58 | 13.91 | 8.29 | 9.73 | 2.57 |  |
| 30 | 16.16 | 18.91 | 13.20 | 15.99 | 9.49 | 11.19 | 2.99 |  |
| 40 | 18.10 | 20.62 | 14.80 | 17.42 | 10.68 | 12.25 | 3.20 |  |
| 60 | 20.72 | 22.73 | 16.96 | 20.34 | 12.29 | 13.54 | 3.47 |  |
| 80 | 23.13 | 24.63 | 17.82 | 21.20 | 13.03 | 13.96 | 4.01 |  |
| 100 | 24.98 | 26.32 | 19.33 | 22.70 | 14.27 | 14.97 | 4.18 |  |
| 120 | 28.04 | 29.36 | 20.72 | 25.32 | 15.29 | 15.92 | 4.97 |  |
| LSD 0.05 | 6.19 | 6.46 | 4.48 | 5.62 | 3.34 | 3.47 |  |  |

Table 5. Changes in water extractable Zn and Cu concentrations during the composting process in different heaps.

| Time (days) | Heap 1 | Heap 2 | Heap 3 | Heap 4 | Heap 5 | Heap 6 |  |  | LSD 0.05 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extractaple Zn (mg/g) |  |  |  |  |  |  |  |  |  |  |
| 0 | 30.79 | 30.88 | 21.47 | 21.69 | 69.22 | 68.71 |  |  | 18.04 |  |
| 5 | 40.56 | 42.63 | 28.01 | 29.95 | 91.18 | 94.87 |  |  | 24.37 |  |
| 10 | 33.94 | 37.02 | 23.22 | 26.25 | 75.58 | 70.75 |  |  | 18.45 |  |
| 20 | 27.37 | 32.08 | 18.72 | 22.84 | 41.89 | 48.86 |  |  | 9.25 |  |
| 30 | 18.72 | 22.13 | 12.81 | 15.76 | 35.63 | 41.74 |  |  | 9.27 |  |
| 40 | 17.47 | 20.10 | 11.96 | 14.30 | 31.61 | 36.04 |  |  | 7.75 |  |
| 60 | 16.00 | 17.73 | 10.97 | 12.58 | 26.60 | 29.15 |  |  | 5.90 |  |
| 80 | 14.71 | 15.83 | 10.08 | 11.47 | 23.52 | 25.05 |  |  | 4.91 |  |
| 100 | 12.86 | 13.69 | 8.82 | 9.94 | 14.54 | 15.17 |  |  | 2.34 |  |
| 120 | 8.66 | 9.16 | 5.95 | 6.65 | 14.71 | 15.23 |  |  | 3.15 |  |
| LSD 0.05 | 9.65 | 10.41 | 6.66 | 7.27 | 24.72 | 25.06 |  |  |  |  |
| Extractaple Cu (mg/g) |  |  |  |  |  |  |  |  |  |  |
| 0 | 13.96 | 13.82 | 6.23 | 6.20 | 28.68 | 28.31 |  |  | 8.19 |  |
| 5 | 18.15 | 18.83 | 8.02 | 8.45 | 37.28 | 38.57 |  |  | 10.91 |  |
| 10 | 14.43 | 15.54 | 6.73 | 7.51 | 29.36 | 29.43 |  |  | 8.18 |  |
| 20 | 11.64 | 13.46 | 5.09 | 6.12 | 17.20 | 21.57 |  |  | 5.09 |  |
| 30 | 8.22 | 9.60 | 3.60 | 4.37 | 15.44 | 16.12 |  |  | 4.36 |  |
| 40 | 7.43 | 8.44 | 3.25 | 3.84 | 12.28 | 14.26 |  |  | 3.60 |  |
| 60 | 6.80 | 7.44 | 2.98 | 3.37 | 11.02 | 12.01 |  |  | 3.03 |  |
| 80 | 6.61 | 7.02 | 2.90 | 3.25 | 7.67 | 9.29 |  |  | 2.04 |  |
| 100 | 5.47 | 5.74 | 2.40 | 2.67 | 6.02 | 5.47 |  |  | 1.33 |  |
| 120 | 3.87 | 4.04 | 1.70 | 1.87 | 5.33 | 5.88 |  |  | 1.38 |  |
| LSD 0.05 | 4.14 | 4.35 | 1.87 | 1.99 | 10.06 | 10.07 |  |  |  |  |

A

B

Fig. 1. Seed germination (A) and root elongation (B) % of cress grown on compost extracts of different heaps during composting period.

Fig. 2. Germination index (GI) of cress seeds grown on compost extracts of different heaps during composting period.

Fig. 3. Relationship between germination index (GI) and each of N-NO3-and N-NH4+ concentrations in the compost extracts during the composting period of different heaps.

Fig. 4. Relationship between germination index (GI) and each of water ext. P and water ext. K concentrations in the compost extracts during the composting period of different heaps.

Fig. 5. Relationship between germination index (GI) and each of water ext. Zn and water ext. Cu concentrations in the compost extracts during the composting period in different heaps.

5. CORRESPONDING AUTHOR:

Prof. Dr. Houssam M. Atta

Botany and Microbiology Department, Faculty of Science (Boys), Al-Azhar University, Cairo, Egypt.

The present address:

Biotechnology Department. Faculty of Science and Education. Al-Khurmah, Taif University; KSA.

E-mail: houssamatta@yahoo.com

houssamatta@hotmail.com

6- REFERENCES

- Bernal, M. P.; M. A. Paredes; M. A. Sanchez-Monedero and J. Cegarra (1998). Maturity and stability parameters of composts prepared with a wide range of organic wastes. Bioresource Technology, 63:91-99.

- Brewer, L. J. and D. M. Sullivan (2003). Maturity and stability evaluation of composted yard trimmings. Compost Science & Utilization, 11(2):96–112.

- Butler, T. A., L. J. Sikora; P. M. Steinhilber and L. W. Douglass (2001). Compost age and sample storage effects on maturity indicators of biosolids compost. Journal Environmental Quality, 30: 2141-2148.

- Cambardella, C. A.; T. L. Richard and A. Russell (2003). Compost mineralization in soil as a function of composting process conditions. European Journal of Soil Biology, 39:117-127.

- Cheung, Y. H.; M. H. Wong and N. F. Y. Tam (1989). Root and shoot elongation as an assessment of heavy metal toxicity and 'Zn equivalent value of edible crops. Hydrobiologia, 188: 377-383.

- Christian, A. H.; G. K. Evanylo and J. W. Pease (1997). On farm composting: A guide to principles, planning and operations, Virginia Tech., USA, pp. 7-8.

- Ciavatta, C.; M. Govi; A. Simoni and P. Sequi (1993). Evaluation of heavy metals during stabilization of organic matter in compost produced with municipal solid waste, Bioresource Technology, 43:147-153.

- Cooperband,  L. R.; A. G. Stone; M. R. Fryda and  J. L. Ravet (2003). Relating compost measures of stability and maturity to plant growth. Compost Science and Utilization, 11: 113–124.

- Emino, E. R. and P. R. Warman (2004). Biological assay for compost quality. Compost Science & Utilization, 12 (4):342–348.

- Erhart, E. and K. Burian, (1997). Evaluating quality and suppressiveness of Austrian biowaste composts. Compost Science & Utilization, 5(3):15-24.

- Fang, M. and J. W. Wong (1999). Effects of lime amendment on availability of heavy metals and maturation in sewage sludge composting. Environmental Pollution, 106: 83-89.

- Finstein M. S. and F. C. Miller (1985). Composting of Agricultural and other Wastes. Elsevier Applied Science, Barking UK, pp. 13-26.

- Forster J. C.; W. Zech and E. Würdinger (1993). Comparison of chemical and microbiological methods for the characterization of the maturity of composts from contrasting sources. Biology and Fertility of Soils. 16(2): 93-99.

- Gajdos, R. (1997). Effects of two composts and seven commercial cultivation media on germination and yield. Compost Science & Utilization, 5: 16–37.

- Gariglio, N. F.; M. A. Buyatti; R. A. Pilatti; D. E. Gonzalez Rossia and M. R. Acosta (2002). Use of a germination bioassay to test compost maturity of willow (Salix sp.) sawdust. New Zealand Journal of Crop and Horticultural Science, 30:135–139.

- Georgacakis, D.; A. Tsadaris; J. Bakouli and S. Symeonidis (1996). Composting of solid swine manure and lignite mixture with selected plant residues. Bioresource Technology, 56: 195-200.

- Jackson, M. L. (1973). Soil chemical analysis. Prentice-Hall of India private New Delhi.

- Leita L; M. De Nobili; C. Mondini; G. Muhlbachova; L. Marchiol; G Bragato and M. Contin (1999). Influence of inorganic and organic fertilization on soil microbial biomass, metabolic quotient and heavy metal bioavailability. Biology and Fertility of Soils, 28:371-376.

- Manios, V. I.; P. E. Tsikalas and H. I. Siminis (1989). Phytotoxicity of olive tree in relation to organic acid concentration. Biological Waste, 27: 307-317.

- Meggie, C. A.; R. W. Pearson and A. E. Hilthold (1967). Toxicity of decomposing crop residues to cotton germination and seedling growth. Agronomy Journal, 59: 197-199.

- Meunchang S.; S. Panichsakptana and R. W. Weaver (2005). Co-composting of filter cake and baggasse; by products from a sugar mill. Bioresource Technology, 96: 437-442.

- Ozores, Hampton, M. (1998). Compost as an alternative weeds control method. Horticultural Science, 33 (6): 938–940.

- Page A. L.; R. H. Miller and D. R. Keeney (1982). Methods of soil analysis. Part 2. American Society of Agronomy, Inc, Soil Science Society of America, Madison, Wisconsin USA.

- Riffaldi, R.; R. Levi-Minzi, R.; A. Pera and  M. deBertoldi (1986). Evaluation of compost maturity by means of chemical and microbial analyses. Waste Management & Research. 4: 387-396.

- Rovira, A. D. (1956) Plant root excretions in relation to the rhizosphere effect. 1. The nature of root exudates from oats and peas. Plant and Soil, 7: 178-194.

- Snedecor, G. W. and W. G. Cochran (1967). Statistical Methods. 6th Ed. Iowa State Univ. Press. Ames, Iowa, U.S.A., PP. 223-398.

- Tang, J. C.; N. Maie; Y. Tada and A. Katayama (2006). Characterization of the maturing process of cattle manure compost. Process Biochemistry 41: 380–389.

- Tam, N. F. Y. and S. M. Tiquia (1994). Assessing toxicity of spent sawdust pig-litter using seed germination technique. Resources, Conservation and Recycling ,11: 261-274.

- Tiquia S. M. (2003). Evaluation of organic matter and nutrient composition of partiallyed composted and composted spent pig litter. Environmental Technology, 24: 97-107.

- Tiquia S. M. and N. F. Y. Tam (1998). Elimination of phytotoxicity during co-composting of spent pig-manure sawdust litter and pig sludge. Bioresource Technology, 65: 43–9.

- Tiquia S. M.; N. F. Y. Tam and I. J. Hodgkiss (1996). Effects of composting on phytotoxicity of spent pig-manure sawdust litter. Environmental Pollution, 93: 249–56.

- Warman, P. R. (1999). Evaluation of seed germination and growth test for assessing compost maturity. Compost Science and Utilization, 7: 33-37.

- Wei, Y. S.; Y. B Fan; M. G. Wang and J. S. Wang (2000). Composting and compost application in China. Resources Conservation and Recycling, 30: 277-300.

- Wong, M. H. (1985). Phytotoxicity of refuse compost during the process of maturation. Environmental Pollution, 37: 159-174.

- Wong, M. H.; Y. H. Cheung and C. L. Cheung (1983). The effect of ammonia and ethylene oxides in animal manure and sewage sludge on seed germination and root elongation of Brassica parachinensis. Environmental Pollution 30: 109-123.

- Wong. J. W. C.; K. F. Mak; N. W. Chan; A. Lam; M. Fang and L. X. Zhou (2001). Co-composting of soybean residues and leaves in Hong Kong. Bioresource Technology, 76: 99-106.

- Wu, L. and L. Q. Ma (2001). Effects of sample storage on biosolids compost stability and maturity evaluation. Journal of Environmental Quality, 30: 222-228.

- Zucconi, F. and M. de Bertoldi (1987). Compost specification for the production and characterization of compost from municipal solid waste. In: de Bertoldi, M.; M. P. Ferranti; P. L. Hermite; F. Zucconi (Eds), Compost, production, Quality and Use. Elsevier Applied Science. pp, 30-50.

- Zucconi, F.; M. Forte; A. Monaco and M. de Bertoldi (1981a). Biological evaluation of compost maturity. BioCycle, 22(2): 27–29.

- Zucconi, F.; A. Pera; M. Forte and M. de Bertoldi (1981b). Evaluating toxicity of immature compost. BioCycle, 22 (4): 54-57.

- Zucconi, F.; A. Monaco; and M. Forte (1985). Phytotoxins during the stabilization of organic matter. In: Gasser J. K. R. (Editor), Composting of Agricultural and other Wastes, Elsevier Applied Science Publication, New York, NY, U.S.A. pp. 73-86.

12/14/2011