Comparison Of Glycine Max and Sida Acuta in the Phytoremediation Of Waste Lubricating Oil Polluted Soil

*Stephen, E1. and Ijah, U.J.J2

- Department of Microbiology, Kogi State University, Anyingba,Kogi State, Nigeria

- Department of Microbiology, Fed. University of Technology, Minna, Niger State, Nigeria

psychsea07@gmail.com,+234-8036347036

Abstract:  Glycine max and Sida acuta were compared to evaluate their potential in phytoremediation of waste lubricating oil contaminated soil in a 56 day field experiment. The experimental methodology involved simulation of 15kg of soil with 2 litres of waste oil to achieve 13.3% (w/w) pollution level. The result of the analysis revealed that bacteria counts were higher in G.max treatment than S. acuta and the control (OPS). pH, Moisture, Electrical conductivity and Phosphorous levels were lower in the S. acuta treatment than the G. max treatment. Statistical analysis revealed that there were significant differences in pH, Electrical conductivity, Nitrogen, Iron, Nickel and Copper (p<0.05) whereas bacteria counts, Moisture, Organic matter content, Phosphorous, Lead and Zinc had no significant difference at 0.05% confidence level. The results indicate that G.max   had a better potential than S. acuta in the remediation of waste lubricating oil contaminated soil.

[Stephen, E. and Ijah, U.J.J. Comparison Of Glycine Max and Sida Acuta in the Phytoremediation Of Waste Lubricating Oil Polluted Soil. Nature and Science 2011;9(8):190-193]. (ISSN: 1545-0740). http://www.sciencepub.net.

Key words: phytoremediation, Glycine max, Sida acuta, waste lubricating oil, pollution

1. Introduction

Contamination of soils with petroleum hydrocarbons is a wide spread environmental problem and a growing concern in many countries, especially in Asia and African continents (Agamuthu et al., 2010).  The indiscriminate disposal of spent lubricating oil by motor mechanics is a common source of spent lubricating oil contamination of soil in countries like Nigeria that do not enforce strict compliance to environmental laws ( Ogbo et al., 2009). Contaminants present in soils can enter the food chain and seriously affect animal and human health (Khan, 2005). Waste lubricating oil creates unsatisfactory conditions for plant growth ranging from heavy metal toxicity to insufficiency in aeration of the soil (Stephen et al., 2010). Ogbo et al. (2009) reported that oil contamination causes slow rate of germination in plants. Adam and Duncan (2002) also reported that this effect could be due to the oil which acts as a physical barrier preventing or reducing access of the seeds to water and oxygen. Soil contamination is a problem of global scope, and yet no universal solution has been discovered to deal with it (Jankaite, 2009). Different methods have been employed in remediating contaminated soil with each having one form of disadvantage or the other. Phytoremediation, which entails the in situ use of plants and their associated microorganisms to degrade, contain or render harmless contaminants in the soil (Joner et al., 2004), is an innovative, economical and environmental compatible solution for remediating some contaminated sites (Oyelola et al., 2009).

Different types of plants have been found useful for phytoremediation of soil contaminated by hydrocarbons. Karthikeyin et al. (1999) reported that Alfalfa and Horse radish reduce concentration of kerosene-based jet fuel by 57-90% in five months. Ayotamuno et al. (2006) reported the use of Zea mays (corn) and Pennisetum purpureum  (Elephant grass) in the remediation of petroleum- hydrocarbon contaminated agricultural soil in port-Harcourt, Nigeria. Njoku et al. (2009) reported the effect of growth of Glycine max on the physicochemical properties of crude oil contaminated soil.

The choice of Sida acuta and Glycine max for this study was because (1) S. acuta is a non edible plant and thrives in disturbed habitat (2) S. acuta is a weed found virtually in all localities in Nigeria (3). G. max is successful in climates with optimum temperatures of 20oC to 30oC, performs nitrogen fixation and is grown widely in Nigeria.   The objective of this study was to determine the suitability of S. acuta and G. max in the mineralization of waste lubricating oil from contaminated soil.

2. Materials and Methods

Study area description-The experiment was conducted at the botanical garden of the Kogi State Polytechnic main Campus Lokoja, Nigeria. Lokoja is a confluence town (where Rivers Niger and Benue meets) and lies between latitudes 70 481 North and longitude 60 441 East and falls within the rich agricultural zone of the Guinea Savannah vegetation belt of the country and receives an annual rainfall of about 900-1700mm. The average temperature of the experimental area is 300C.

Field sites and experimental procedure- The experimental design consisted  of perforated plastic pots containing 15kg of soil each simulated with 2 litres of waste lubricating oil obtained from Felele mechanic Village to achieve 13.3% (w/w) pollution level. The pots were perforated to increase aeration and to avoid water logging. Pots A1 and A2 served as the control (No plant treatment), B1 and B2 received S. acuta treatments and pots C1 and C2 received G. max treatment.  The waste oil was thoroughly mixed with the soil in the pots and left undisturbed for one week (7 days). Young growing S. acuta were transplanted from the Botanical garden into bowls B1 and B2 and viable seeds of G. Max were planted in bowls C1 and C2   on the 8th day. The plants were monitored for 56 days in the rainy months of July and August.

Laboratory / Statistical methods: pH and Electrical conductivity of the soil was determined at ambient temperature using glass electrode pH and conductivity meter (Hannia, Italy). Nitrogen was determined by the micro Kjedahl method (Ibitoye, 2006). Phosphorous was determined by the Murphy and Riley (1962) method. The ignition method of Akinsanmi (1975) was used to determine the organic matter content while the dry weight method (Tropical development Institute, 1984) was used to determine the moisture content. Trace metal was determined using the Atomic absorption Flame spectrophotometer (Buck, Model VGP210, U.S.A). Microbiological analysis was carried out following the procedure described by Harrigan and McCane (1990). Analysis of variance (ANOVA) was performed using procedure of MINITAB software Release 13 (2000). Experimental precision achieved was reported at p<0.05 level.

3. Results

Figure 1 shows the total heterotrophic bacterial counts. It ranged from 6.0x105 to 1.6x106Cfu/ml for control (OPS), 6.0x105 to 1.9x106 for Glycine max treated pot (GCM) and 6.0x105 to 1.5x106 for Sida acuta treated soil (SAT). The results showed that the counts were higher at days 28 for OPS and 56 for GCM and SAT. Microbiological analysis of the soil samples showed that the bacteria associated with the plants include Pseudomonas, Bacillus, Micrococcus, Flavobacterium, and Alcaligenes. There was no statistical difference (p<0.05) in the counts between the treatments.

Table 1 shows the physicochemical properties of the soil samples. pH of the three treatments : Control (OPS), G. max (GCM) and S. acuta (SAT) ranged from 7.63 to 8.27, 7.58 to 8.17 and 6.97 to 8.17. The least pH was observed in SAT at day 56. Statistical analysis indicated that There was significant difference (P<0.05) in the pH between the treatments and the control (OPS). Treatment GCM had the highest electrical conductivity compared to the control (OPS) and SAT. There was a significant difference in the electrical conductivity between the treatments at 0.05% confidence level. Table 1 also revealed the Nitrogen and available Phosphorous in the soil. The highest concentration of Nitrogen was observed in OPS while the least concentration was observed in GCM. There was a significant difference in the treatments (P<0.05). There was an observed increase in phosphorous from day 0 to day 56 in the OPS. However, there was a sudden dip in the concentration at day 28 while there was a general decline in GCM and SAT at day 7 and gradual increase from day 28 to day 56. There was no significant difference in the phosphorous concentration between the treatments (p<0.05). The Organic matter content ranged from 5.37% to 5.80% in the OPS, 4.00% to 5.47% in GCM and 4.82% to 5.85% in SAT. The least concentration was observed in GCM at day 28 while the highest concentration was observed in SAT at day 7. No significant difference exists in the organic matter content of the soil between the treatments (p< 0.05). Moisture content of OPS ranged from 0.73-9.33%, GCM from 1.81-9.19% and SAT from 0.50- 6.96%. The lowest moisture was observed at day 56 in SAT while the highest moisture level was observed at day 7 in OPS. There was no significant difference in the moisture content between the treatments and OPS statistically.

Fig 1. Total aerobic heterotrophic bacteria count

OPS:Control; GCM: G. max treatment;

SAT:   S. acuta treatment

Table 1. Physicochemical characteristics of soil sample analysed

pH                             Ec (us/cm)   	     Moisture (%)	                                 O.M C (%)         Nitrogen (%)                         Phosphorous (ppm)

Treatments    0     7    28      56                       0     7    28      56                   0     7     28      56	  0      7      28      56                      0     7    28      56	           0        7         28      56

OPS            8.17  8.01  8.27  7.63               191    126   163    102           1.81   9.33   5.97       0.73              5.47     5.37    5.60     5.80              0.16   0.17  0.16   0.17            42.00  46.20     10.00    53.30

GCM          8.17   8.13   7.79   7.58              191     128    146    138        1.81    9.19   1.70     2.02             5.47     5.00     4.00    5.23               0.16    0.14  0.12    0.15          42.00  27.30     25.90   79.10

SAT           8.17  7.93    7.73   6.97               191     124    125    119       1.81   6.96    2.07     0.50               5.47     5.85    4.82    5.12              0.16  0.17    0.14    0.15         42.00  10.50       12.60   18.40

OPS: control; GCM: Glycine max; SAT: Sida acuta, Ec: Electrical conductivity; O.M.C: Organic matter content

Table 2.Trace metal analysed in soil sample

Fe (ppm)                                     Pb (ppm)   	                            Zn (ppm)	                                   Ni (ppm)                                    Cu (ppm)

Treatments        0          7    28      56                           0      7     28      56                  0        7     28      56	     0         7        28      56                   0         7      28         56

OPS                 1.73   1.49  1.79    1.58                 0.32    0.50   0.27    0.52            3.63   3.50   1.22    1.45             1.85      1.11      1.09    1.03              0.04   0.03   0.05      0.06

GCM                1.73   1.49   2.34     1.25               0.32     0.43    0.57   0.43         3.63    2.90   1.81     1.85            1.85       1.36      1.12   1.28               0.04   0.01   0.03     0.04

SAT                  1.73    2.69   2.10    1.98               0.32    0.48     0.62   0.49          3.63   2.11   2.33    1.35             1.85      0.57       0.84    1.01              0.04   0.02   0.01     ND

OPS: control; GCM Glycine max; SAT: Sida acuta; ND: not detected

Table 2 shows the level of trace metals in the soil treatments analysed. The result showed that Zinc had a higher concentration than Iron, Lead, Nickel, and Copper in the treatments and OPS. A general decline was observed in the level of the trace metal from Day 0 to Day 56 in both treatments and OPS. There were significant differences in the levels of Iron, Copper, and Nickel (P<0.05) between the treatments and OPS. However, no significant different exist in the Lead and Zinc concentration (P>0.05) between the treatments and OPS.

4. Discussion

Soil pH in GCM and SAT were lower than the control-OPS. This may be due to increase in metabolic activities in the soil as a result of plant growth and associated microbial activities. This finding agrees with the findings of Ijah and Abioye (2003) that observed decrease pH in kerosene polluted soil, 30 months after spill and attributed the decrease to microbial activities. Both Nitrogen and Phosphorous were lower in GCM and SAT than OPS. This may be due to the fact that Nitrogen and Phosphorous are essential for plant growth and must have been utilized by the plant. The higher concentration of Phosphorous in OPS agrees with the findings of Ijah and Abioye (2003) and Stephen et al. (2010) who reported increase in Nitrogen and Phosphorous contents of the soil polluted by hydrocarbon products. Increase in Phosphorous content in OPS could be caused by the existence of reduced condition in the soil that made Iron Phosphorous soluble and brought some Phosphorous into solution (Ayotamuno et al., 2006). The decrease in trace metal concentration in the treatments relative to OPS could be attributed to containment or bioaccumulation of the metals by the plant, natural degradation by soil micro flora enhanced by favourable pH, volatilization and other abiotic factors such as Temperature (Oyelola et al., 2009, Stephen et al., 2010).

The increase in microbial counts of GCM and SAT relative to OPS substantiated the hypothesis that an increase in microbial activity was the mechanism responsible for phytoremediation. This highlights the view that the enormous increase in microbial counts in the treatment bowls (GCM and SAT) was linked with the treatment applied. The trend was different for OPS. Similarly, the view that phytovolatilization is a likely mechanism of phytoremediation remains attractive. An evidence of this was leaf chlorosis observed in the plants during the first few weeks of remediation. Such a situation suggests that volatile organic compounds were taken up by the roots of the plants, translocated within the plants and transpired through stems and leaves. The leaf chlorosis gradually disappeared before the 35th day indicating that many of the organic compounds in the waste lubricating oil have been transferred to the atmosphere. This observation is in line with that of Ayotamuno et al. (2006) who observed similar disappearance of the chlorosis with time.

5. Conclusion

The result of this study demonstrates the potential of G.max and S. acuta in the remediation of waste lubricating oil contaminated soil. Given the short period of this study (56) days, it can be concluded that G.max and S. acuta are viable plants for phytoremediation and under field condition, G.max is a better alternative to S. acuta, and should be encouraged in the remediation of waste oil contaminated soil.

Correspondence to:

Stephen Emmanuel

Department of Microbiology, Kogi State University Anyingba, Kogi State, Nigeria

Email : psychsea07@gmail.com

Telephone :+234-8036347036

References

Adam, G.I. and Duncan, H. (2002). Influence of diesel on seed germination. Environ. Pollut. 120:363-370.

Akinsanmi, O. (1975). Certificate Agricultural Science. Longman, Nigeria. pp 104-112.

Agamuthu, P., Abioye,O.P. and Abdul Aziz, A.(2010). Phytoremediation of soil contaminated with used lubricating oil using Jatropha curcas. Journal of Hazardous Materials, 179:891-894.

Ayotamuno, M.J., Kogbara, R.B. and Egwuenum, P.N. (2006). Comparism of corn and elephant grass in the phytoremediation of a petroleum hydrocarbon- contaminated agricultural soil in Port-Harcourt, Nigeria. Journal of Agricultuture and Environment, 4 (2&4): 218-222.

Harrigan, W.F. and McCane, M.E. (1990). Laboratory Methods in Food and Dairy Microbiology, 8th ed., Academic Press, London

Ibitoye, A.A.(2006). Laboratory Manual on Basic Soil Analysis (2nd ed). Foladave Nigeria Limited, Akure, pp30-37.

Ijah, U.J.J and Abioye, O.P. (2003). Assessment of Physicochemical and Microbiological properties of soil 30 months after kerosene spill. Journal of Research in Science and Management 1(1):24-30.

Jankaite, A. (2009). Soil remediation from heavy metals using Mathematical modeling. Journal of Environmental Engineering and Landscape Management 17 (2):121-129.

Karthikeyan, R., Davis, L.C., Mankin,K.R., Erickson, L.E., Kulakow,P.A. (1999). Biodegradation of jet fuel (JP-8) in the presence of vegetation. In: proceedings of the 1999 conference on Hazardous Waste Research, St. Louis , Missouri, May 22-27, pp 243-256.

Khan, A.G. (2005). Role of soil microbes in the rhizospheres of plants growing on trace metal contaminated soils. Journal of Trace Elem. Med.Biol. 18:355-364.

MINITAB Statistical Software Release 13.1. (2000). Minitab Inc, U.S.A.

Murphy, J. and Riley, J.P. (1962). A. modified single Sowton method for the determination of phosphorous in natural water. Analytical Chemistry 27:31-36.

Njoku, K.L., Akinola, M.O. and Oboh, B.O. (2009). Phytoremediation of crude oil contaminated soil : The effect of growth of Glycine max on the physicochemistry and crude oil contents of soil . Nature and Science 7 (10):79-87.

Ogbo, E.M., Avwerosovwe, U. and Odogu, G. (2009).Screening of four common weeds for use in phytoremediation of soil contaminated with spent lubricating oil. African Journal of Plant Science 3(5):102-106.

Oyelola,O.T., Babatunde,A.I. and Odunlade,A.K. (2009). Phytoremediation of metals from contaminated soil using Lycopercium ecsululentum (Tomato) plant . International Journal of Pure and Applied Sciences, 3(2):44-48.

Stephen,E., Onuche, H.A. and Ijah, U.J.J. (2010). Physicochemical properties of waste lubricating oil simulated soil in Lokoja, Kogi State, Nigeria . International Journal of Applied Biology Research, 2(2):44-48.

Tropical Development Institute, TDI. (1984). Outlined method for the determination of organic matter content

7/15/2011