A review of the evolution of current-mode circuits and techniques and various modern analog circuit building blocks

Kasim K. Abdalla1, D. R. Bhaskar1, Raj Senani2

1 Department of Electronics and Communication Engineering, Faculty of Engineering and Technology, Jamia Millia Islamia, New Delhi-110025, India
2 Division of Electronics and Communication Engineering, Netaji Subhas Institute of Technology, Azad Hind Fauj Marg, Sector-3, Dwarka, New Delhi- 110078, India. Email: senani@nsit.ac.in

Abstract: This paper presents a brief introduction and review of some prominent current mode building blocks, which have been finding prominent attention in the area of analog signal processing. Some exemplary hardware implementations of a selected number of building blocks such as op-amp, OTA, CCI, CCII, FTFN, DDA, CFOA, CCIII, DDCC, DOCC, MOCC, DVCC, ICCII, FDCCII, OTRA, CDBA and CDTA have been reviewed.

[Kasim K. Abdalla, D. R. Bhaskar, Raj Senani. A review of the evolution of current-mode circuits and techniques and various modern analog circuit building blocks. Nat Sci 2012;10(10):1-13]. (ISSN: 1545-0740). http://www.sciencepub.net/nature. 1

Keywords: evolution; circuit; technique; analog; building; block

  - Introduction:

The development of very large scale integration (VLSI) technology, together with the ever increasing demand for fully integrated systems containing a larger number of digital as well as analog circuits on a single chip, has ensured continued interest on analog circuit design which is required to be compatible with CMOS technology. In fact, analog circuits such as continuous-time filters, sinusoidal oscillators, digital to analog (D/A) and analog to digital (A/D) converters, voltage comparators, current and voltage amplifiers, rectifiers, etc. are unavoidable analog circuits which can not be realized by digital techniques. Moreover, new applications continue to appear where new analog topologies have to be designed to ensure the trade-off between speed and power requirements.

Analog circuit design has historically been viewed as a voltage dominated form of signal processing. Recently, current-mode analog circuits have emerged in the implementation of analog functions. In current- mode circuit description, the input and the output are both taken in the current form rather than in voltage form. Current- mode signal processor can be defined as a circuit in which current is used as the active variable in preference to voltage, either throughout the whole circuit or only in certain critical areas.19 In contrast to the conventional voltage mode circuits, the current mode circuits can exhibit, under certain conditions, among other things higher bandwidth and better signal linearity. Since they are designed for lower voltage swings, smaller supply voltages can be used.95

In addition to the advancement in current-mode analog signal processing, another particular development is the emergence of new current-mode analog building blocks among which the most prominent and popular has been the current conveyor (CC)1, 3, 13. The concept of the current conveyor was first introduced by Smith and Sedra in 1968. It was called the first-generation current conveyor (CCI)1 which is a 3-terminal building block having three terminals X, Y, Z and the port variables characterized by iY = iX, vX = vY  and iZ = ±iX. The second-generation current conveyor (CCII) was developed in 1970 3 and is characterized by iY = 0, vX = vY and iZ = ± iX and further developed to the third-generation current conveyor in 1995 (CCIII) 23 which was characterized by iY = -iX, vX = vY and iZ = ± iX. Since then, the area of current mode circuits has progressed very fast and several thousand research papers have been published in this exciting area so far.

With different modifications in the basic conveyor structure, a number of newer elements have been introduced a brief account of which now follows. A second generation current controlled conveyor (CCCII) is characterized by iY = 0, vX = vY + iX rX and iZ = ± iX and was implemented in BJT technology by Fabre in 1995 24 and in 1996 26 and in BiCMOS technology in 1997. 28 Dual-output current conveyor (DO-CCII) 58 is characterized by iY = 0, vX = vY, iZ+ = iX, iZ- = - iX, can provide two opposite direction outputs when the output terminals are in the same direction. Another variant is the so-called current follower current conveyor (CFCCII).17 Inverting current conveyor (ICC) was introduced by Awad and Soliman in 1999 33 is characterized by iY = 0, vX = - vY and iZ = ± iX. Differential current conveyor (DCC) was presented by Elwan and Soliman in 1996 25 which has two X terminals and the current of the Z terminal is given by the difference between the X terminals currents i.e., iZ = iX1 - iX2. Differential voltage current conveyor (DVCC) was introduced by Elwan and Soliman in 1997 29 which is a five terminal building block with two Y terminals such that iY1 = 0 = iY2,  vX = vY1 - vY2  and iZ = iX. A DVCC with two complementary current output terminals is known as DVCCC (Differential Voltage Complementary CC) 29 and provides iZ1 = iX, iZ2 = - iX. Differential difference Current Conveyor (DDCC) was presented by Chiu, Liu, Tsao, and Chen in 1996 27, but when combining the effect of both DDCC+ and DDCC- in a single active building block (ABB) where two complementary current outputs are presented, it is known as differential difference complementary current conveyor (DDCCC) 48 and is characterized by iY1 = iY2,= iY3 = 0,  vX = vY1 -  vY2 + vY1, iZ1 = iX, iZ2 = -iX. Fully differential second generation Current Conveyor (FDCCII) was introduced by Alzaher, Elwan and Ismail in 2000 37 where each of the terminals has been doubled with respect to the original CCII, but dual-X second generation current conveyor (DXCCII) introduced by Zeki and Toker in 2002 50 has two X-terminals (Xp and Xn) as well as two related output Z terminals (Zp and Zn). Modified CCII (MCCII) was introduced by Dukic in 2007 87 where the current through the output Z terminal (always positive) does not depend on the direction of X terminal current. An operational floating conveyor (OFC) was introduced by Toumazou and Payne in 1991.16 Universal current conveyor (UCC) presented by Becvar, Vrba, Zeman and Musil in 2000 40 is able to replace any type of existing current conveyor. Modified third generation current conveyor (MCCIII) has been presented by Kuntman, Cicekoglu and Ozoguz in 2002.49 Current gain current conveyor (CGCCII) 58 is designed with an output terminal (Z) such that Z-terminal current is n times higher current with respect to X terminal current. Also there are a number of new variations of slightly different nature which incorporate such as current division network (CDN) 99 and digitally controlled current follower (DCCF) 88 in an appropriate manner to enhance the controllability of the relation between output and input currents.

The application of filters and oscillators in communication circuits needs extended high frequency performance in fully integrated circuit form. Circuit designers began to search for a more suitable active element in order to provide the unavoidable gain without imposing severe frequency limitations. At the same time, it was endeavored to keep the circuitry simple so that it can be realized in IC technologies enabling easy synthesis procedures for active circuits, hence, there are many active building blocks introduced for this purpose. The most widespread active element for on chip implementation is undoubtedly the operational transconductance amplifier (OTA) which was introduced by Wheatley and Wittlinger in 1969.2 Bialko and Newcomb in 1971 4 demonstrated how all basic linear functions can be realized using the integrated differential voltage-controlled-current-source (DVCCS). CMOS low-voltage OTA was introduced by Elwan, Gao, Sadkowski and Ismail in 2000.39 The Multiple Output OTA (MO-OTA) has appeared as a generalization of Bipolar OTA (BOTA) and its applications were demonstrated in the design of economical biquadratic filters.41, 61

The need for the floating output in some applications led to the design of monolithic floating nullor.6 The Four-terminal floating nullor (with the acronym ‘FTFN’ coined for the first time in 10-11 by Senani) has been shown to be a very flexible and versatile building block in active network synthesis. A number of papers have dealt with its implementation 6, 8, 11 and a general implementation has been described by Higashimura in 1991.15 An FTFN is equivalent to an ideal nullor and it is also called operational floating amplifier (OFA). A technique for realizing a versatile OFA using an op-amp is presented by Nordholt in 1982.7 An implementation of the operational floating amplifier (OFA) was introduced by Huijsing in 1990.14 Differential difference amplifier (DDA) has four high-impedance inputs compared to conventional operational amplifier (Op-amp) which has two high impedance inputs, and it was introduced for the first time by Säckinger and Guggenbuhl in 1987.9 Differential difference operational floating amplifier (DDOFA) is combining the advantages of the fully balanced input of DDA and the symmetrical output of OFA.32 The concept of the fully balanced four-terminal floating nullor (FBFTFN) was introduced by Alzaher and Ismail in 2002 52 who also gave its CMOS implementation and applications.

The conventional transimpedance operational amplifier (TOA) is a combination of the CCII and the voltage buffered amplifier.12 Current feedback amplifier (CFA) 38 and TOA have an identical internal structure. The absence of electronic control of the voltage gain or any other parameter in the conventional Op-amp enforced designing the current-controlled CFA (CC-CFA).89 Some interesting variants of CFAs have also been proposed such as differential voltage CFA (DVCFA) 53 and its further generalized form, namely, the differential difference complementary current feedback amplifier (DDCCFA) introduced by Gupta and Senani in 65 which was shown to be capable of realizing a number of prevalent building blocks.

A new active element termed as operational transresistance amplifier (OTRA) (commercially available as Norton amplifier 5, 44) along with its CMOS implementation was introduced by Chen, Tsao and Chen in 1992.21 On the other hand, another new element known as current differencing buffered amplifier (CDBA) was introduced by Acar and Ozoguz in 1999.34 There are numerous papers published employing CDBAs in various applications.54-56, 62, 64, 68-69 CDBA is also known as differential current voltage conveyor (DCVC).47 CDBA is basically a generalization of OTRA as an universal element. Current differencing unit (CDU) and the voltage unity-gain buffer are contained into the internal structure of the CDBA. CDU is a current conveyor of the MDCC type. A Current controlled CDBA (CC-CDBA) was presented by Maheshwari and Khan in 2004 63, while a current differencing transconductance amplifier (CDTA) was introduced by Biolek in 2003 for the first time.57 CDTA applications may not require the use of external resistors, which are substituted by internal transconductors, therefore many research papers were published employing CDTA.66, 70-73, 75, 78, 80-84, 90-91 Current controlled CDTA (CCCDTA) was modified from CDTA by Siripruchyanun and Jaikla in.85, 92, 96

  - Hardware Implementation of Some Selected Analog building Blocks:

This section presents symbolic notation and hardware implementation circuits of some selected analog building blocks directly or indirectly related to the work presented in this thesis.

Operational amplifiers (op-amps) are one of the most widely used building blocks for analog circuits and systems. Op-amps are employed from dc applications to high speed amplifiers and filters. General purpose op-amps can be used as buffers, summers, integrators, differentiators, comparators, negative impedance converters, and in many other applications. The op-amp is a differential voltage controlled voltage source. The symbolic notation of the op-amp is shown in Fig. 1.

Fig. 1 Symbolic notation of the op-amp

The characterising equations of the op-amp are:

Vo=Av(V+−V−), where Av = open-loop voltage gain.

A typical CMOS implementation of the op-amp is shown in Fig. 2.

Fig. 2 CMOS realization of an op-amp adopted from.8

Operational transconductance amplifier (OTA) is a differential voltage controlled current source. Fig. 3 shows the symbol of OTA. For an ideal MO-OTA with transconductance gm, the output current Io is given by Io = ± gm (V+ - V-). The transconductance (gm) can be varied by changing the bias current Ibias. The transconductance (gm) of CMOS OTA is equal to (β Ibias /4)1/2 (whereβ=μCox(WL)) and of bipolar OTAs is Ibias/2VT, respectively.79

Fig. 3 Symbolic notation of OTA.

A representative CMOS realization of the MO-OTA is shown in Fig. 4.35, 76

Fig. 4 CMOS realization of the MO-OTA proposed by Zeki and Kuntman.35

Out of a large number of elements proposed so far, current conveyors are the most popular active building blocks because of their superior and versatile terminal characteristics as well as their widespread applications which include amplifiers, oscillators, filters, wave shaping circuits, precision rectifiers and numerous others.

The first generation current conveyor CCI has been introduced in 1968 as the first CC. The CCI is a three terminal active building block with terminals labeled X, Y and Z as shown in Fig. 5.

Fig. 5 Symbolic notation of CCI

The CCI± is characterized by the hybrid matrix:

IYVXIZ=0101000±10VYIXVZ         (1)

where plus and minus signs denote a positive and negative CCI.

An exemplary CMOS implementation of CCI is shown in Fig. 6.

Fig. 6 An exemplary CMOS implementation of CCI proposed by Bruun.22

Second generation current conveyor CCII is the most versatile conveyor structure among all CCs and has shown to be useful in almost all analog circuit operations. The symbolic notation is shown in Fig. 7.

Fig. 7 Symbolic notation of CCII

For an ideal CCII, the relation between the voltages and currents is given by the hybrid matrix:

IYVXIZ=0001000±10VYIXVZ        (2)

The CCII is defined in a positive and negative version, depending on the direction of the current in the Z terminal. Fig. 8 shows a typical CMOS implementation of the CCII+ [20].

Fig. 8 A typical CMOS implementation of CCII+ proposed by Liu, Tsao and Wu20

That a four-terminal floating nullor (FTFN) is a very flexible and versatile building block in active network synthesis was demonstrated in.10- 11, 18 The acronym FTFN has been introduced in 1987 by Senani for the first time in 10 and 11. The symbolic notation of the FTFN is shown in Fig. 9.

Fig. 9 Symbolic notation of FTFN

The port characteristics of an FTFN can be described as: VX = VY, IX = IY = 0, IZ = IW. Fig. 10 shows the CMOS realization of the FTFN.

Fig. 10 CMOS realization of the FTFN proposed by Cam, Toker and Kuntman43

The Differential difference amplifier (DDA) is an extension to the concept of the op-amp. The main difference is that instead of two single-ended inputs as the case in op-amps, it has two differential input ports (Vpp –Vpn) and (Vnp –Vnn) as shown in the Fig. 11. Therefore, the output of the DDA can be written as Vo = A[(Vpp –Vpn) - (Vnp –Vnn)], where A is the gain of the DDA.

Fig. 11 Symbolic notation of the DDA

The CMOS implementation of the DDA is shown in Fig. 12.

Fig. 12 CMOS realization of the DDA proposed by Säckinger and Guggenbuhl9

Trans-impedance operational amplifier or more popularly known as current feedback operational amplifier (CFOA) is receiving growing attention as an alternative building blocks for analog circuit design, because it offers several advantages over the traditional voltage–mode op-amp (VOA). Fig. 13 shows the symbolic notation of the CFOA.

Fig. 13 Symbolic notation of the CFOA

The port relations of an ideal CFOA are described by the following hybrid matrix:

IYVXIZVW=0000100001000010VYIXVZIW       (3)

A CMOS implementation of the CFOA is shown in Fig. 14.

Fig. 14 A CMOS realization of the CFOA proposed by Mahmoud, Madian and Soliman86

Third generation current conveyors (CCIIIs) can be considered as current controlled current sources with a unity gain. This type of current conveyor is useful to take out the current flowing through a floating branch of a circuit and can be utilized in various multifunction filters, inductance simulation and active filters.30, 31, 36, 45, 49 The terminal block diagram of an ideal dual-output CCIII is shown in Fig. 15.

Fig. 15 Symbolic notation of CCIII

The defining hybrid matrix equation of a dual-output CCIII is given by:

IYVXIZ±=0−101000±10VYIXVZ       (4)

A sample CMOS realization of the dual-output third generation current conveyor is shown in Fig. 16.

Fig. 16 A sample CMOS implementation of CCIII proposed by Minaei59

Differential difference current conveyor (DDCC) possesses the advantages of high input impedance at port Y1, Y2 and Y3 and high output impedance at port Z which suits for voltage signal inputs and current signal output.93, 97 Fig. 17 indicates the symbol of a DDCC.

Fig. 17 Symbolic notation of DDCC

The port relations of an ideal DDCC can be characterized by:

VXIY1IY2IY3IZ=01−110000000000000000±10000IXVY1VY2VY3VZ   (5)

A typical CMOS implementation of the DDCC is shown in Fig. 18.

Fig. 18 A CMOS realization of the DDCC proposed by Hou and Lin 60

Dual output current conveyor (DOCCII) is a four terminal building block. The two output terminals have opposite directions. The symbolic notation is shown in Fig. 19.

Fig. 19 Symbolic notation of DOCCII.

The ideal DOCCII can be characterized by:

IYVXIZ±=0001000±10VYIXVZ    (6)

A sample CMOS DOCCII implementation of the DOCCII is shown in Fig. 20.

Fig. 20 CMOS realization of the DOCCII modified by Singh, V.K., Singh, A.K. and Senani 67 from the implementation of Liu, Tsao and Wu 20)

Multiple output current conveyor (MOCCII), which has multiple outputs, has its symbolic notation shown in Fig. 21.

Fig. 21 Symbolic notation of MOCCII

The relation between the voltage terminals and current for an ideal MOCCII is given by the hybrid matrix:

IYVXIZ±=0001000±10VYIXVZ      (7)

For CMOS implementation of MOCC, one can use DOCC by adding multiple number of positive or negative outputs.51

Differential voltage current conveyor DVCC is a four terminal device, which has two high input impedance at terminals Y1 and Y2 and low input impedance at terminal X.74 The symbolic notation of the DVCC is shown in Fig. 22.

Fig. 22 Symbolic notation of DVCC

The ideal DVCC can be characterized by:

IY1IY2VXIZ=000000001−1000010VY1VY2IXVZ           (8)

We can get the CMOS implementation of a DVCC, if we ground the terminal Y3 of the CMOS DDCC 66, which is shown in Fig. 18.

The inverting second generation current conveyor (ICCII) is considered to be a special case from the DVCC or the DDCC. The symbolic representation of the balanced output ICCII is shown in Fig. 23.

Fig. 23 Symbolic notation of ICCII

For an ideal ICCII the terminal characteristics in terms of current and voltage is given by the hybrid matrix:

IYVXIZ=000−100010VYIXVZ            (9)

For implementing ICCII+, the DDCC or the DVCC are employed 42, 46, 94, by grounding the terminals Y1 and Y3 of the circuit (DDCC) which is shown in Fig. 18.

Fully differential second generation current conveyor (FDCCII) is an eight-terminal analog building block shown symbolically in Fig. 24.

Fig. 24 Symbolic notation of FDCCII

The port relations of an ideal FDCCII are described by the following matrix equation:

VX+VX−IZ+IZ−=001−11000−1101100000010000IX+IX−VY1VY2VY3VY4        (10)

A typical CMOS realization of the FDCCII is shown in Fig. 25.

Fig. 25 CMOS realization of the FDCCII proposed by El-Adawy, Soliman and Elwan 77 and modified by Chen98

Some other types of active building blocks have also been frequently used in analog signal processing such as OTRA, CDBA, CDTA, etc.

Operational Transresistance Amplifier (OTRA) (commercially available, e.g. LM3900 and referred to as Norton amplifier) has attracted attention due to its advantages in the current-mode circuit design.5 The symbolic notation is shown in Fig. 26.

Fig. 26 Symbolic notation of OTRA

The ideal OTRA is characterized by:

VpVnVZ=000000Rm−Rm0IpInIz       (11)

A typical CMOS implementation of the OTRA is shown in Fig. 27.21

Fig. 27 CMOS realization of the OTRA proposed by Chen, J.J, Tsao and Chen, C.-C.21

The current differencing buffered amplifier (CDBA) has been found to be a versatile active building block for voltage- and current-mode signal processing applications. Fig. 28 shows the symbolic notation of the CDBA.

Fig. 28 Symbolic notation of the CDBA

The defining equations of the CDBA are: Vp = Vn = 0, Iz=Ip - In, and VW = VZ. A typical CMOS implementation of the CDBA is shown in Fig. 29.

Fig. 29 CMOS realization of the CDBA proposed by Acar and Ozoguz34

Current Differencing Transconductance Amplifier (CDTA) is a five terminal active element and has recently been shown to be a versatile component in the realization of a class of analog signal processing circuits. It is actually a current-mode element whose input and output signals are currents. The symbolic notation of CDTA is shown in the Fig. 30. CDTA can contain an arbitrary number of X terminals, providing currents Ix of both directions. The port relations characterizing CDTA are given by Vp = Vn = 0, Iz = Ip - In, and IX = ± g VZ, where g is the transconductance.

Fig. 30 Symbolic notation of CDTA

A representative CMOS implementation of the CDTA is shown in Fig. 31.

Fig. 31 CMOS realization of CDTA proposed by Keskin and Biolek73

A comprehensive treatment of a large variety of analog circuit building blocks, alongwith introduction of several new ones, has been dealt with in a recent paper by Biolek, Senani, Biolkova and Kolka.95

  - Concluding remarks:

This paper presented a brief introduction and review of some prominent current mode building blocks, which have currently been finding prominent attention in the area of analog signal processing. Some exemplary hardware implementations of a selected number of building blocks such as op-amp, OTA, CCI, CCII, FTFN, DDA, CFOA, CCIII, DDCC, DOCC, MOCC, DVCC, ICCII, FDCCII, OTRA, CDBA and CDTA have been reviewed. It is believed that this exposition would be useful to the analog circuit designers as well as beginners of research to acquaint them with some of the prominent building blocks and key ideas in the exciting area of analog circuit design.

  - References:

- K. C. Smith and A. Sedra, The current conveyor- A new circuit building block, Proc. IEEE 56 (1968)1368-1369.

- C. F. Wheatley and H. A. Wittlinger, OTA Obsoletes Op-Amp, P. Nat. Econ. Conf. (1969) 152-157.

- A. Sedra and K. C. Smith, A second generation current conveyor and its applications, IEEE Trans. Circuit Theory CT-17(1970) 132-134.

- M. Bialko and R.W. Newcomb, Generation of All Finite Linear Circuits Using the Integrated DVCCS, IEEE Trans.  Circuit Theory CT-18 (1971) 733 – 736.

- T.M. Frederiksen, W.F. Davis and D.W. Zobel, A new current-differencing, single supply operational amplifier, IEEE Journal of Solid-State Circuits SC-6 (1971) 340-347.

- J.H. Huijsing and J. De Krote, Monolithic nullor– a universal active network element, IEEE Journals of Solid-State Circuits SC-12 (1977) 59-64.

- E.H. Nordholt, Extending op amp capabilities by using a current-source power supply, IEEE Trans. Circ. Syst. CAS- 29 (1982) 411-414.

- P.R. Gray and R.G. Meyer, MOS operational amplifier design- a tutorial overview, IEEE Journal of Solid-State Circuits 17 (1982) 969-981.

- E. Säckinger and W. Guggenbuhl, A versatile building block: The CMOS differential difference amplifier, IEEE Journal of Solid-State Circuits SC-22 (1987) 287-294.

- R. Senani, Generation of new two-amplifier synthetic floating inductors, Electron. Lett. 23(1987) 1202–1203.

- R. Senani, A novel application of four-terminal floating nullors, Proc. IEEE, 75(1987) 1544–1546.

- C. Toumazou, F.J. Lidgey and D.G. Haigh, Analogue IC design: the current- mode approach, IEE Circuits and Systems Series 2, Peter Peregrinus Ltd. (1990).

- A. S. Sedra, G. W. Roberts and F. Gohh, The current conveyor: history, progress and new results, IEE Proc. Pt. G 137(1990) 78-87.

- J.H. Huijsing, Operational floating amplifier, IEE Proc. Pt. G 137(1990) 131-136.

- M. Higashimura, Realization of current-mode transfer function using four terminal floating nullor, Electron. Lett. 27 (1991) 170-171.

- C. Toumazou and A. Payne, operational floating conveyor, Electron. Lett. 27 (1991) 651-652.

- K. Ikeda and Y. Tomita, Realization of current-mode biquadratic filter using CCIIs with current followers, Electron. Commun. Jpn. Pt. 2, Electron. 77 (1994) 99-107.

- M. Higashimura, Current-mode all-pass filter using FTFN with grounded capacitor, Electron. Lett. 27 (1991) 1182-1183.

- S.-S. Lee, R.H. Zele, D.J. Allstot and G. Liang, A continuous-time current-mode integrator, IEEE Trans. Circ. Syst. 38 ( 1991) 1236-1238.

- S.-I. Liu, H.-W. Tsao and J. Wu, CCII-based continuous-time filters with reduced gain- bandwidth sensitivity, IEE Proc. Pt. G 138 (1991) 210-216.

- J.J. Chen, H.-W. Tsao and C.-C. Chen, Operational transresistance amplifier using CMOS technology, Electron. Lett. 28 (1992) 2087-2088.

- E. Bruun, Class AB CMOS first-generation current conveyor, Electron. Lett. 31 (1995) 422-423.

- A. Fabre, Third-generation current conveyor: a new helpful active element, Electron. Lett. 31(1995) 338-339.

- A. Fabre, O. Saaid, F. Wiest and C. Boucheron, Current controlled band pass filter based on the translinear conveyors, Electron. Lett. 31 (1995) 1727-1728.

- H.O. Elwan and A.M. Soliman, CMOS differential current conveyors and applications for analog VLSI, Analog Integrated Circuits and Signal Processing 11 (1996) 35-45.

- A. Fabre, O. Saaid, F. Wiest and C. Boucheron, High frequency applications based on a new current-controlled conveyor, IEEE Trans. Circ. Syst.I 43 (1996) 82-91.

- W. Chiu, S.-I. Liu, H.-W. Tsao and J.-J. Chen, CMOS Differential difference current conveyors and their applications, IEE Proc. Pt. G 143 (1996) 91–96.

- A. Fabre, O. Saaid, F. Wiest and C. Boucheron, Low power current-mode second-order bandpass IF Filter, IEEE Trans. Circ. Syst. II 44 (1997) 436-445.

- H.O. Elwan and A.M. Soliman, Novel CMOS differential voltage current conveyor and its applications, IEE Proc. Pt. G 144 (1997) 195-200.

- J.-W. Horng, R.-M. Weng, M.-H. Lee and C.-W. Chang, Universal active current filter using two multiple current output OTAs and one CCIII, Int. J. Electron. 82 (1997) 241–247.

- M.T. Abuelma’atti and H.A. Alzaher, Multi-function active-only current-mode filter with three inputs and one output, Int. J. Electron. 85 (1998) 431–435.

- S.A. Mahmoud and A.M. Soliman, The differential difference operational floating amplifier: A new block for analog signal processing in MOS technology, IEEE Trans. Circ. Syst. II 45 (1998) 148-158.

- I.A. Awad and A.M. Soliman, Inverting second generation current conveyors: The missing building blocks, CMOS realizations and applications, Int. J. Electronics 86 (1999) 413–432.

- C. Acar and S. Ozoguz, A new versatile building block: current differencing buffered amplifier, Microelectronics Journal 30 (1999) 157-160.

- A. Zeki and H. Kuntman, High-output-impedance CMOS dual-output OTA suitable for wide-range continuous-time filtering applications, Electron. Lett. 35 (1999) 1295-1296.

- H.-Y. Wang and C.-T. Lee, Systematic synthesis of R-L and C-D immittances using single CCIII, Int. J. Electronics 87 (2000) 293–301.

- H.A. Alzaher, H.O. Elwan and M. Ismail, CMOS fully differential second-generation current conveyor, Electron. Lett. 36 (2000) 1095-1096.

- A.M. Ismail and A.M. Soliman, Novel CMOS current feedback op-amp realization suitable for high frequency applications, IEEE Trans. Circ. Syst. I 47 (2000) 918-921.

- H. Elwan, W. Gao, R. Sadkowski and M. Ismail, CMOS low-voltage class-AB operational transconductance amplifier, Electron. Lett. 36 (2000) 1439-1440.

- D. Becvar, K. Vrba, V. Zeman and V. Musil, Novel universal active block: a universal current conveyor. In Proc. of the IEEE Int. Symp. on CAS. Geneva (Switzerland) 3 (2000) 471-473.

- C.-M. Chang and S.-K. Pai, Universal current-mode OTA-C biquad with the minimum components, IEEE Trans. Circ. Syst. 47 (2000) 1235-1238.

- S. Ozoguz, A. Toker and O. Cicekoglu, First-order all-pass sections-based current –mode universal filter using ICCII, Electron. Lett. 36 (2000) 1443-1444.

- U. Cam, A. Toker and H. Kuntman, CMOS FTFN realization based on translinear cells, Electron. Lett., IEE (UK) 36 (2000) 1255-1256.

- A. Toker, S. Ozoguz, O. Cicekoglu and C. Acar, Current-mode all-pass filters using current differencing buffered amplifier and a new high-Q bandpass filter configuration, IEEE Trans. Circ. Syst. II 47 (2000) 949-954.

- H.-C. Chow and W.-S. Feng, New symmetrical buffer design for VLSI application, Int. J. Electronics. 88 (2001) 779–797.

- A. Toker, A. Zeki and O. Cicekoglu, Generalized gyrator implementation techniques using unity gain cells, Int. J. Electronics 88 (2001) 517- 528.

- K.N. Salama, H.O. Elwan and A.M. Soliman, Parasitic-capacitance-insensitive voltage-mode MOSFET-C filters using differential current voltage conveyor, Circuits, Systems and Signal Processing 20 (2001) 11-26.

- S.S. Gupta and R. Senani, Comments on CMOS differential difference current conveyors and their applications, IEE Proc. Pt. G 148 (2001) 335–336.

- H. Kuntman, O. Cicekoglu and S. Ozoguz, A modified third generation current conveyor, its characteristic and applications, Frequenz: J. Telcommun. 56 (2002) 47–54.

- A. Zeki and A. Toker, The dual-X current conveyor (DXCCII): A new active device for tunable continuous-time filters, Int. J. Electronics 89 (2002) 913-923.

- R. Senani and A.K. Singh, A new universal current-mode Biquad filter, Frequenz: J. Telcommun. 56 (2002) 55-59.

- H. Alzaher and M. Ismail, A CMOS fully balanced four- terminal floating nullor, IEEE Trans. Circuits Syst. I 49 (2002) 413-424.

- E.O. Gunes and A. Toker, On the realization of oscillators using state equations, Int. J. Electron. Commun. (AEÜ) 56 (2002) 317-326.

- J.W. Horng, Current differencing buffered amplifiers based single resistance controlled quadrature oscillator employing grounded capacitors, IEICE Trans. Fundamental E85-A (2002) 1416- 1419.

- D. Biolek and V. Biolková, SFG simulation of general ladder filters using CDBAs, In Proceedings of the ECCTD03. Krakow (Poland) I (2003) 385-388.

- D. Biolek, M. Olšák and V. Biolková, Optimization of elliptic leap-frog CDBA-based filters, Contribution to the book Computational Methods in Circuits and Systems Applications, WSEAS Press, Electrical and Computer Engineering Series (2003) 221-225.

- D. Biolek, CDTA – building block for current-mode analog signal processing, In Proceedings of the ECCTD03. Krakow (Poland) III (2003) 397-400.

- G. Ferri and N.C. Guerrini, Low-voltage low-power CMOS current conveyors, Kluwer Academic Publishers, (2003).

- S. Minaei, A new high performance CMOS third generation current conveyor (CCIII) and its application, Electrical Engineering 85 (2003) 147–153.

- C.-L. Hou and C.-C. Lin, A filter with three voltage-inputs and one voltage-output and one current-output using current conveyors, Tamkang Journal of Science and Engineering 7 (2004) 145-148.

- M.T. Abuelma'atti and A. Bentrcia, New universal current-mode multiple-input multiple-output OTA-C filter, In Proc. of the 2004 IEEE Asia-Pacific Conf. on CAS. (2004) 1037-1040.

- K. Klahan, W. Tangsrirat and W. Surakampontorn, Realization of multiphase sinusoidal oscillator using CDBAs Circuits and Systems, In Proceedings of the 2004 IEEE Asia-Pacific Conference. Taiwan 2 (2004) 725 – 728.

- S. Maheshwari and I.A. Khan, Current-controlled current differencing buffered amplifier: implementation and applications, Active and Passive Electronic Components 27 (2004) 219-227.

- A.U. Keskin, A four quadrant analog multiplier employing single CDBA, Analog Integrated Circuits and Signal Processing 40 (2004) 99-101.

- S.S. Gupta and R. Senani, Grounded-capacitor SRCOs using a single differential difference complementary current feedback amplifier, IEE Proc. G 152 (2005) 38-48.

- A. Uygur, H. Kuntman and A. Zeki, Multi-input multi-output CDTA-based KHN filter, In Proc. of the 4th Int. Conf. on Electrical and Electronics ELECO 2005. Bursa (Turkey) (2005) 46-50.

- V.K. Singh, A.K. Singh, and R. Senani, Dual function capability of recently proposed four-current conveyor-based VM biquad, J. Circuits, Syst., Comput. 14 (2005) 51-56.

- A.U. Keskin, Design of a PID controller circuit employing CDBAs, International Journal of Electrical Engineering Education (Manchester, England) 43 (2006) 48-56.

- A.U. Keskin, C. Aydin, E. Hancioglu and C. Acar, Quadrature oscillator using Current Differencing Buffered Amplifiers (CDBA), Frequenz: J. Telcommun. 60 (2006) 57-59.

- W. Tangsrirat and W. Surakampontorn, Systematic realization of cascadable current-mode filters using current differencing transconductance amplifiers, Frequenz: J. Telcommun. 60 (2006) 241-245.

- A. Uygur and H. Kuntman, Low-voltage current differencing transconductance amplifier in a novel allpass configuration, In Proc. of the 13th IEEE Mediterranean Electrotechnical Conference MELECON'06, Spain (2006) 23-26.

- W. Tanjaroen, T. Dumawipata, S. Unhavanich, W. Tangsrirat and W. Surakampontorn, Design of current differencing transconductance amplifier and its application to current-mode KHN biquad filter, In Proc. of the ECTI-CON 2006, Thailand (2006).

- A.U. Keskin and D. Biolek, Current mode quadrature oscillator using current differencing transconductance amplifiers (CDTA), IEE Proc. Pt. G 153 (2006) 214 - 218.

- J.-W. Horng, C.-L. Hou, C.-M. Chang, H.-P. Chou and C.-T. Lin, High input impedance voltage-mode universal biquadratic filter with one input and five outputs using current conveyors, Circuits Systems Signal Processing 25 (2006) 767–777.

- A.Ü. Keskin, D. Biolek, E. Hancioglu and V. Biolková, Current-mode KHN filter employing current differencing transconductance amplifiers, Int. J. Electron. Commun. (AEÜ) 60 (2006) 443-446.

- T. Tsukutani, Y. Sumi and Y. Fukui, Electronically controlled current-mode oscillators using MO-OTAs and grounded capacitors, Frequenz: J. Telcommun 60 (2006) 220-223.

- A.A. El-Adawy, A.M. Soliman and H.O. Elwan, A novel fully differential current conveyor and applications for analog VLSI, IEEE Trans. Circuits Syst. II 47 (2000) 306-313.

- W. Tangsrirat, Current differencing transconductance amplifier-based current-mode four-phase quadrature oscillator, Indian Journal of Engineering & Materials Sciences 14 (2007) 289-294.

- P. Prommee, K. Angkeaw, J. Chanwutitum, and K. Dejhan, Dual input all-pass networks using MO-OTA and its application, ECTI-CON (2007).

- N.A. Shah, M. Quadri and S.Z. Iqbal, CDTA based universal transadmittance filter, Analog Integrated Circuit and Signal Processing 52 (2007) 65-69.

- N.A. Shah, M. Quadri and S.Z. Iqbal, Current-mode multifunction filter using current differencing transconductance amplifier Indian Journal of Pure & Applied Physics 45 (2007) 767-769.

- N.A. Shah, M. Quadri and S.Z. Iqbal, Electronically tunable transadmittance BP filter, Electronics World (2007) 51-52.

- A. Uygur and H. Kuntman, ‘Seventh-order elliptic video filter with 0.1 dB pass band ripple employing CMOS CDTAs. Int. J. Electron. Commun. (AEÜ) 61 (2007) 320-328.

- W. Tangsrirat, T. Dumawipata and W. Surakampontorn, Multiple-input single-output current-mode multifunction filter using current differencing transconductance amplifiers, Int. J. Electron. Commun. (AEÜ) 61 (2007) 209-214.

- M. Siripruchyanun and W. Jaikla, Realization of current controlled current differencing transconductance amplifier (CCCDTA) and its applications, Transactions on Electrical Eng., Electronics, and Communications ECTI-EEC 5 (2007) 41-50.

- S.A. Mahmoud, A.H. Madian and A.M. Soliman, Low-voltage CMOS current feedback operational amplifier and its application, ETRI J. 29 (2007) 212-218.

- S. Dukic, The analysis of full-wave wide-band precision rectifier with modified second type current conveyor, Facta Universitatis (NIŠ), Ser.: Elec. Energ. 20 (2007) 215-221.

- H.A. Alzaher, CMOS digitally programmable quadrature oscillators, Int. J. Circ. Theor. Appl. 36 (2008) 953-966.

- M. Siripruchyanun, C. Chanapromma, P. Silapan and W. Jaikla, BiCMOS current-controlled current feedback amplifier (CC-CFA) and its applications, WSEAS Trans. on Electronics, Special Issue Modern Circuit Components for Analogue Signal Processing and Their Applications 5 (2008) 2003-2019.

- W. Tangsrirat and W. Tanjaroen, Current-mode multiphase sinusoidal oscillator using current differencing transconductance amplifiers, Circuits Syst Signal Process. 27 (2008) 81-93.

- D. Biolek, E. Hancioglu and A.U. Keskin, High-performance current differencing transconductance amplifier and its application in precision current-mode rectification, Int. J. Electron. Commun. (AEÜ) 62 (2008) 92-96.

- M. Siripruchyanun and W. Jaikla, CMOS current-controlled current differencing transconductance amplifier and applications to analog signal processing, Int. J. Electron. Commun. (AEÜ) 62 (2008) 277-287.

- J.-W. Horng, High input impedance voltage-mode universal biquadratic filter with three inputs using DDCCs, Circuits Syst. Signal Process 27 (2008) 553–562.

- A.M. Soliman, New grounded capacitor current mode band-pass low-pass filters using two balanced output ICCII, J. of Active and Passive Electronic Devices 3 (2008) 175–184.

- D. Biolek, R. Senani, V. Biolkova and Z. Kolka, Active elements for analog signal processing: Classification, review, and new proposals, Radioengineering 17 (2008) 15-32.

- M. Siripruchyanun and W. Jaikla, A current-mode analog multiplier/divider based on CCCDTA, Int. J. Electron. Commun. (AEÜ) 62 (2008) 223-227.

- H.P. Chen, Versatile universal voltage-mode filter employing DDCCs, Int. J. of Electron. Commun. (AEÜ) 63 (2009) 78-82.

- H.P. Chen, Single FDCCII-based universal voltage-mode filter, Int. J. of Electron. Commun. (AEU), vol. 63 (2009) 713-719.

- W. Tangsrirat, D. Prasertsom and W. Surakampontorn, Low-voltage digitally controlled current differencing buffered amplifier and its application, Int. J. Electron. Commun. (AEÜ) 63 (2009) 249-358.

5/28/2012