Forest Carbon Sinks and Its Economy Evaluation of State-owned Forest Region in Heilongjiang Province

LI Changsheng

School of Humanities and Social Sciences, Harbin Institute of Technology, Harbin, Heilongjiang, 150001, China. Email: lichangsheng100@yahoo.com.cn

Abstract: Forest carbon sinks has been paying much more attention to by worsening of greenhouse effect and climate warming. After studying the characteristics of forest carbon sinks, direct fixing carbon of forest was analyzed. The operational method of measurement of forest carbon sinks was suggested. Finely, forest carbon sinks of state-owned forest region in Heilongjiang Province was calculated. The forest carbon sinks of state-owned forest region in Heilongjiang Province is 0.895 billion ton. The value is 106.349 billion Chinese dollars.

[LI Changsheng. Forest Carbon Sinks and Its Economy Evaluation of State-owned Forest Region in Heilongjiang Province. World Rural Observations 2012;4(1):34-38]. ISSN: 1944-6543 (Print); ISSN: 1944-6551 (Online). http://www.sciencepub.net/rural. 6

Key words: forest carbon sinks; economy evaluation; tree biomass carbon reserves

In terrestrial ecosystem, the forest is the largest carbon pool; the trees play an important and unique role in reducing greenhouse gas concentrations in the atmosphere and global warming by absorbing lots of carbon dioxide from the atmosphere during photosynthesis. In recent years, with increasing of the greenhouse effect and global warming, more and more attention is paid to the forest carbon sinks. Forest carbon sinks means the forest plants absorbing carbon dioxide from the atmosphere and fixing it in vegetation or soil, to reduce the concentration of the gases in the atmosphere. Although the area of forest takes up only 1 / 3 of the total area of land, but carbon storage in forest vegetation zone accounts for nearly half the total carbon pool of terrestrial. Currently, emissions of greenhouse gases of China accounted for 50% in developing countries, 15% in the global. China is rapidly shift from a state of low energy consumption to one state of high energy consumption, a large number of carbon dioxide emissions will be emitted. There is a very important theoretical and practical significance to establish a complete and science forest carbon sinks measurement system for measurement and dynamics of forest carbon sinks, and gradually integrate forest carbon sinks into the national forest ecological value statistics. This is to analyze direct fixing carbon of forest, identify the operational method of measurement of forest carbon sinks, and calculate and economic evaluation of forest carbon sinks of state-owned forest region in Heilongjiang Province.

Measurement methods of forest carbon sink

Method of calculating forest carbon sinks include carbon density, carbon balance of F-CARBON model method, CO2FIX modeling, biomass conversion method, etc. , but these methods are basically belong to the pure category of natural, calculation is complicated. From social science, the purpose of this study is to carry out economic evaluation and implementation of carbon sinks trade, the current calculation is too complex to operate. Therefore, a calculating method of forest carbon sinks should be based on the calculation methods of the natural science and the research results, considering the calculating method of the practicality and operability, this paper proposes a set of economic measurement methods - forest volume conversion factor method.

1. The formula for all fixed carbon in forest

CF = the amount of fixed carbon of tree biomass + the amount of fixed carbon of Undergrowth + the amount of fixed carbon of forest

Where：

In the formula：CF—All the amount of fixed carbon of forest；

Sij—The area of type-j forest in type-I areas；

Cij—The biomass carbon density of type-j forest in type-I areas；

- Vij—the stock volume per unit area of type-j forest in type-I areas；

δ— biomass enlarge coefficient；

α—carbon transfer coefficient of undergrowth vegetation；

β—carbon transfer coefficient of woodland；

ρ—volumetric coefficient；

γ—carbon content rate.

2.The extension effect of the rate of wood fixed carbon. Solid wood carbon savings conversion based on considering the Utilization rate of harvesting and timber quality.     Let λ be wood fixed carbon transfer coefficient, then the amount of wood fixed carbon.

Where：

CW—amount of wood fixed carbon；

Q—timber yield；

—utilization rate；

—outturn percentage。

3.The change of the formula

Allowing for alternate of woodland and current annual increment of understory vegetation playing a role in fixed carbon, when we investigate the release quantity of carbon dioxide with the expenditure of forest resources and gets, take tree biomass into account. In view of this, you can calculate the emissions of carbon dioxide from timber production.

Timber production discharge carbon into the atmosphere

=

Timber production discharge carbon dioxide into the atmosphere

=

However, in the actual production of timber, it is easier to get produced, but for reducing the number of forest biomass in the production process it is not concerned. Assuming we have mastered the wood production data, consumption of forest reserves can be derived based on the above formula, the emissions amount of carbon and carbon dioxide can be calculated in the logging and processing of timber.

When timber yield is Q, Timber production discharge carbon dioxide

These coefficients should be determined in order to accurately calculate the amount of forest carbon sinks, are different with areas and forest types. Reasonable regionalization for the national according to the theory of regionalization and forestry is a precondition for determined these coefficients, based on the regionalization to determine various conversion coefficients of forest types.

4.Determination of the average of various conversion coefficients

(1)The expansion coefficient of forest resources accumulation δ

Accumulation of trees can be converted into bio-volume with a tree as Main part by using the coefficient. The average biomass of the leaves of China conifer and broadleaf trees accounted for about 6.75% of the total tree biomass, branches accounted for 15.75%, roots accounted for 25.0%, tree trunk accounted for 52.5% according to the measured tree, hence it is not difficult to calculate that the expansion coefficient of tree bio-volume is 1.9. The Measurement coefficient of conifers and broadleaf is 1.7 and 1.8 from Japan. The research Result from France show that tree trunks and branches accounted for about 78% of the total biomass, leaves accounted for 6%, root biomass accounted for 16%, we can see that The French coefficient is much lower. The coefficient is 1.90 during this study (The default value Intergovernmental panel on climate change (IPCC) is 1.90).

(2)Volume density ρ

Accumulation of the forest biomass can be converted into dry weight conversion factor by using the coefficient. The average volume density of dominant tree species in Japan was about 0.45 (coniferous was about 0.38 t / m3, hardwood was about 0.49 t / m3). We have taken universal IPCC’s default value to 0.5.

(3)Carbon content rate γ

Biomass dry weight can be converted into the amount of fixed carbon by using the coefficient. In our country carbon content rate of hardwood is less than 0.5, while the average carbon content rate of conifers generally is equal to or above 0.5, the result of calculated the tree layer reserve in forest is objective and impartial as the average carbon content rate with 0.5. IPCC’s default value is 0.5.

(4)The conversion coefficient of fixed carbon of Undergrowth α

α is 0.195, by which fixed carbon of undergrowth (including litter) would be calculated based on forest biomass.

(5)The conversion coefficient of fixed carbon of forest β

β is 1.244, by which fixed carbon of forest would be calculated based on fixed carbon of forest biomass.

(6)The conversion coefficient of wood fixed carbon

It is the product of forest harvesting percentage of outturnmultiplying by wood processing utilization rate. After a certain amount of forest having been harvested, the amount of wood fixed carbon would be calculated by this coefficient.

The coefficient value would reach the maximum when maximize the forest harvesting percentage of outturn and wood processing utilization rate. Assume that percentage of outturn is 70%, utilization rate is 80%, and then conversion coefficient of wood fixed carbon is 0.14.

The coefficient value would reach the minimum when minimize the forest harvesting percentage of outturn and wood processing utilization rate. Assume that percentage of outturn is 70%, utilization rate is 80%, and then conversion coefficient of wood fixed carbon is 0.10.

This formula could calculate existing forest fixed carbon as well as fixed carbon caused by forest stock volume increasing. Relevant standard coefficient value of forest carbon sinks in different regions, different forest species and forest types could be set by state to guide the calculation and evaluation of forest carbon sinks.

Dynamic changes of state-owned forest in Heilongjiang Province

Dynamic changes of the area, stock in past 24 years of state-owned forest resources in Heilongjiang Province are shown in Figure 1 to Figure 2 respectively.

Fig 1  Dynamic changes of the area of state-owned forests in Heilongjiang Province

It is shown that the forest area declined fast in 1988, 1989 and 1990. From 1991 to 2004, it declined slowly, and since 2005, forest area has begun to rise in Figure 1.

Figure 2 shows that stock declined fast in 1988, 1989 and 1990, and from 1991 to 2000 it showed some fluctuations, while declining was the general trend. Until 2002, it had reached the bottom point, and it began to rise since 2003. During recent 4 years, forest stock rises rapidly, and develops very well.

Fig 2. Dynamic changes of the stock of state-owned forests in Heilongjiang Province

Table 1   Forest carbon sinks of state-owned forest in Heilongjiang Province

| Age | Forest area /million hectare | Forest reserves/million m 3 | Tree biomass carbon /million ton | forest carbon reserves /million ton |
| --- | --- | --- | --- | --- |
| 1987 | 10.79346 | 705.213 | 334.9762 | 817.0069 |
| 1988 | 10.70520 | 663.457 | 315.1421 | 768.6315 |
| 1989 | 10.60633 | 633.119 | 300.7315 | 733.4842 |
| 1990 | 10.15850 | 632.228 | 300.3083 | 732.4519 |
| 1991 | 10.14421 | 635.830 | 302.0193 | 736.6250 |
| 1992 | 10.12395 | 636.615 | 302.3921 | 737.5344 |
| 1993 | 10.12300 | 645.269 | 306.5028 | 747.5603 |
| 1994 | 10.12496 | 645.928 | 306.8158 | 748.3237 |
| 1995 | 10.08555 | 635.246 | 301.7419 | 735.9484 |
| 1996 | 10.05999 | 636.300 | 302.2425 | 737.1695 |
| 1997 | 10.06004 | 638.505 | 303.2899 | 739.7240 |
| 1998 | 10.05295 | 631.632 | 300.0252 | 731.7615 |
| 1999 | 10.05402 | 615.591 | 292.4057 | 713.1776 |
| 2000 | 10.05405 | 610.141 | 289.8170 | 706.8636 |
| 2001 | 10.05403 | 622.812 | 295.8357 | 721.5433 |
| 2002 | 10.05456 | 615.970 | 292.5858 | 713.6166 |
| 2003 | 10.05437 | 622.930 | 295.8918 | 721.6800 |
| 2004 | 10.05437 | 643.646 | 305.7319 | 745.6800 |
| 2005 | 10.05653 | 667.532 | 317.0777 | 773.3525 |
| 2006 | 10.05582 | 684.959 | 325.3555 | 793.5421 |
| 2007 | 10.09777 | 711.303 | 337.8689 | 824.0623 |
| 2008 | 10.09777 | 731.865 | 347.6359 | 847.8839 |
| 2009 | 10.09777 | 752.149 | 357.2708 | 871.3834 |
| 2010 | 10.09777 | 772.295 | 366.8401 | 894.7231 |

Forest carbon sinks of state-owned forest in Heilongjiang Province and evaluation

(1) Tree biomass fixed carbon

Tree biomass carbon reserves=forest stock × enlarge coefficient×volumetric coefficient × carbon content rate= V×1.9×0.5×0.5

(2) Total forest carbon sinks

Total forest carbon reserves=tree biomass fixed carbon+ fixed carbon of Undergrowth+ fixed carbon of forest

=V×1.9×0.5×0.5+0.195(V×1.9×0.5×0.5) +1.244(V×1.9×0.5×0.5)

=2.439(V×1.9×0.5×0.5)

According to the above tow formulas, Table 1 shows tree biomass fixed carbon and total forest carbon sinks of state-owned forest in Heilongjiang Province based on forest stock.

It is shown that forest carbon sinks of state-owned forest in Heilongjiang Province was 895 million tons.

The value of forest carbon sinks equals to the product of the amount of forest carbon sinks multiply by unit price of forest carbon sinks.

The value of forest carbon sinks

In the formula: Vt—the value of forest carbon sinks；

CF—the amount of forest carbon sinks；

P—price of forest carbon sinks，Yuan/ton

We adopt the international carbon price 5 dollars/ton, the price of carbon is 3 × (44/12) = 18.33 dollars/ton. The value of forest carbon sinks of state-owned forest in Heilongjiang Province is 16.400 billion dollars. We calculate at 1 dollar = 6.4846 Yuan, the value of forest carbon sinks of state-owned forest in Heilongjiang Province is 106.349 billion Yuan.

References

- Jingyun Fang, Anping Chen. Estimation of forest biomass in China: Explains for Fang’s paper in Science[J]. Journal of Plant Ecology, 2002, 26(2):243-249

- Homann P S, Remillard S M, Harmon M E, at al. Carbon storage in coarse and fine fractions of Pacific northwest old-growth forest soils [J]. Soil Science Society of America Journal, 2004,68(6):2023-2030

- Sampson, Waring D A, Maier R H, et al. Fertilization effects on forest carbon storage and exchange, and net primary production: a new hybrid process model for stand management [J]. Forest Ecology Management.2006,221(1-3):91-109

- Sang Weiguo, Ma Keping, Chen Lingzhi. Estimation of carbon cycle for broad-leaved forest in temperate zone[J].  Journal of Plant ECology, 2002, 26(5): 543-548

- [5]Jingyun Fang, Anping Chen. Changes in forest biomass carbon storage in China between 1949 and 1998 [J]. Science, 2001,262:2320-2323

- Zhao Min, Zhou Guangsheng. Plants carbon reserves and carbon balance of forest ecosystem in China.[J]. Scientia Geographica Sonica, 2004, 24（1）：50-54

- Mao Zijun. Estimation methods and study progress of carbon balance for forest ecosystem[J]. Journal of Plant Ecology, 2002,26（6）:731-738

- Li Haikui, Lei Yuancai. Estimation of forest biomass and carbon reserves in China[M]. Publishing House of China Forestry, 2010

- Li Shunlong. Studies of Forest carbon sinks [M]. Publishing House of Northeast Forestry University, 2006.

1/25/2012